| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0171-v1 |
| --- | --- |

| Title: | CE1-related: Combination of LIC and affine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Xiang Ma Haitao Yang Jianle Chen | Email: | chenhuanbang@huawei.com maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes unification methods for affine and LIC. Unidirectional illumination compensation in JVET-M0500 is extended to affine coded CU. It is asserted that the BD-rate can be improved by enabling affine with LIC. In LIC, the linear model parameters are estimated based on the reconstructed neighbor samples of the current CU and their corresponding reference samples. In this contribution, 3 methods are proposed for fetching the reference samples of affine coded CU.

In method one, the top-left sub-block motion vector (MV) of the affine coded CU is used for fetching the reference samples of the whole CU. In method two, the central sub-block MV is used. In method three, the reference samples in the top template are fetched by each sub-block MVs in top row and the reference samples in the left template are fetched by each sub-block MVs in left column.

For method one, the simulation results reportedly show on average 0.67%/0.XX% luma BD-rate saving with 118%/1XX% encoding time in RA/LDB configurations over VTM4.0.

For method two, the simulation results reportedly show on average 0.63%/0.XX% luma BD-rate saving with 118%/1XX% encoding time in RA/LDB configurations over VTM4.0.

For method three, the simulation results reportedly show on average 0.72%/0.XX% luma BD-rate saving with 118%/1XX% encoding time in RA/LDB configurations over VTM4.0.

Introduction

Unidirectional illumination compensation was proposed in JVET-M0500 [1], where LIC is not applied to affine and the linear model parameters are estimated based on the reconstructed neighbor samples of the current and reference CUs.

Proposed methods

In this proposal, the LIC in JVET-M0500 is extended to affine coded CU. The derivation of linear model parameters is kept unchanged and three methods are proposed for fetching the reference samples of affine coded CU.

In method one, as shown in Figure 1, the top-left sub-block motion vector (MV) of the affine coded CU is used for fetching the reference samples of the whole CU.

Figure 1 Reference samples in proposed method 1

In method two, as shown in Figure 2, the central sub-block MV of the affine coded CU is used for fetching the reference samples of the whole CU.

Figure 2 Reference samples in proposed method 2

In method three, as shown in Figure 3, the reference samples in the top template are fetched by each sub-block MVs in top row and the reference samples in the left template are fetched by each sub-block MVs in left column.

Figure 3 Reference samples in proposed method 3

Results

The contribution has been implemented on top of VTM4.0 based on CE1.5.1.a [2] in CE1. The results of each test are shown below against VTM4.0 [3] under CTC [4].

Table 1 Coding performance of method 1 over VTM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.99% | 0.32% | 0.28% | 123% | 100% |
| Class A2 | -0.57% | -0.43% | -0.44% | 114% | 101% |
| Class B | -0.73% | -0.40% | -0.30% | 116% | 100% |
| Class C | -0.43% | -0.31% | -0.34% | 120% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | -0.24% | -0.22% | 118% | 100% |
| Class D | -0.21% | 0.05% | -0.11% | 117% | 100% |
| Class F (optional) | -1.00% | -0.63% | -0.66% | 112% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.51% | -0.25% | -0.87% | 122% | 99% |
| Class E | 0.07% | 0.40% | -0.15% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | -0.17% | -0.61% | 118% | 101% |
| Class F (optional) | -2.88% | -2.15% | -2.04% | 116% | 101% |

Table 2 Coding performance of method 2 over VTM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.96% | 0.30% | 0.33% | 123% | 100% |
| Class A2 | -0.52% | -0.41% | -0.46% | 114% | 101% |
| Class B | -0.66% | -0.29% | -0.23% | 116% | 100% |
| Class C | -0.41% | -0.32% | -0.41% | 120% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -0.20% | -0.21% | 118% | 100% |
| Class D | -0.22% | -0.03% | -0.23% | 118% | 100% |
| Class F (optional) | -0.99% | -0.62% | -0.63% | 113% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.50% | 0.02% | -0.38% | 123% | 101% |
| Class E | -0.06% | 0.15% | -0.14% | 103% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.12% | -0.14% | -0.56% | 119% | 101% |
| Class F (optional) | -2.81% | -2.09% | -2.31% | 116% | 101% |

Table 3 Coding performance of method 3 over VTM-4.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.07% | 0.15% | 0.25% | 124% | 100% |
| Class A2 | -0.63% | -0.34% | -0.44% | 114% | 101% |
| Class B | -0.78% | -0.38% | -0.27% | 116% | 100% |
| Class C | -0.44% | -0.36% | -0.29% | 120% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.26% | -0.21% | 118% | 100% |
| Class D | -0.20% | -0.05% | -0.18% | 117% | 100% |
| Class F (optional) | -1.03% | -0.61% | -0.71% | 113% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.51% | -0.04% | -0.63% | 123% | 102% |
| Class E | 0.06% | 0.32% | -0.39% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.16% | -0.08% | -0.25% | 118% | 101% |
| Class F (optional) | -2.79% | -1.62% | -2.33% | 116% | 102% |

Additionally, the performance of this contribution is compared with CE1.5.1.a.

Table 4 Coding performance of method 1 over CE1.5.1.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | 0.01% | -0.04% | 106% | 100% |
| Class A2 | -0.25% | -0.03% | -0.04% | 104% | 100% |
| Class B | -0.28% | -0.15% | -0.15% | 105% | 101% |
| Class C | -0.10% | -0.04% | 0.01% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.06% | -0.06% | 105% | 100% |
| Class D | -0.06% | 0.10% | 0.11% | 105% | 101% |
| Class F (optional) | -0.13% | -0.14% | -0.11% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.01% | -0.41% | -0.43% | 105% | 99% |
| Class E | 0.08% | 0.37% | -0.07% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.19% | 0.14% | 103% | 100% |
| Class F (optional) | -0.20% | -0.43% | -0.20% | 105% | 100% |

Table 5 Coding performance of method 2 over CE1.5.1.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.01% | 0.01% | 106% | 100% |
| Class A2 | -0.20% | -0.01% | -0.06% | 104% | 100% |
| Class B | -0.21% | -0.04% | -0.08% | 105% | 101% |
| Class C | -0.08% | -0.05% | -0.06% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.03% | -0.05% | 105% | 101% |
| Class D | -0.08% | 0.03% | -0.01% | 105% | 101% |
| Class F (optional) | -0.11% | -0.13% | -0.08% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.02% | -0.14% | 0.06% | 105% | 100% |
| Class E | -0.05% | 0.12% | -0.06% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.04% | 0.22% | 0.19% | 104% | 100% |
| Class F (optional) | -0.12% | -0.35% | -0.46% | 105% | 100% |

Table 6 Coding performance of method 3 over CE1.5.1.a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.15% | -0.07% | 106% | 100% |
| Class A2 | -0.32% | 0.06% | -0.04% | 104% | 100% |
| Class B | -0.33% | -0.13% | -0.13% | 105% | 100% |
| Class C | -0.11% | -0.09% | 0.06% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.09% | -0.05% | 105% | 100% |
| Class D | -0.05% | 0.01% | 0.04% | 105% | 100% |
| Class F (optional) | -0.15% | -0.12% | -0.15% | 105% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0+CE1.5.1.a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.01% | -0.19% | -0.20% | 105% | 101% |
| Class E | 0.07% | 0.29% | -0.31% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.01% | 0.28% | 0.51% | 103% | 100% |
| Class F (optional) | -0.10% | 0.13% | -0.44% | 104% | 101% |

Conclusion

In this contribution, it’s proposed to apply LIC to affine coded CU. It is asserted that the BD-rate can be improved by enabling affine with LIC.

Reference

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE10-related: Unidirectional illumination compensation”, JVET-M0500, the 13th JVET meeting, January 2019, Marrakech, MA.

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz, “CE1: Unidirectional illumination compensation (CE1-5.1)”, JVET-N0343, the 14th JVET meeting, March 2019, Geneva, CH.

- VTM-4.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, January 2019, Marrakech, MA.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).