| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0174 |
| --- | --- |

| Title: | Non-CE8: Exclusive Regular Merge And IBC Merge in One Shared Merge List Area |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Biao Wang Anand Meher Kotra Jianle Chen | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

In VVC Specification Draft (Draft 4) [1], shared merge list is applied both on regular merge and IBC merge mode blocks with a fixed threshold of 32 luma samples. However the worst case of merge list construction still not changed. The proposed method is addressing this problem, by disallowing simultaneous selection of IBC merge and regular merge modes inside a same shared merge region. The proposed method has 0.00%/xx% in RA/LDB overall luma BD-rate increase for CTC sequences, 0.03%/0.01% RA/LDB luma BD-rate increase for ClassF sequences and 0.20%/0.27% RA/LDB luma BD-rate increase for TGM[3] sequences.

# Introduction

Shared merge list is applied for both IBC merge and regular merge candidate list construction. The threshold of shared merge list region is fixed as 32 luma samples. If a parent block split into multiple childblocks, and any of the childblock is smaller than 32 luma samples, then the parent block area is defined as shared merge list region, and all child blocks construct one shared merge list based on this area.

Since the threshold is fixed as 32 luma samples, the following three cases result in application of shared merge list area.

- If the parent block is split by BT (horizontal or vertical), and the child blocks are 4x4 (parent 4x8 or 8x4). as shown in Figure 1 (a).

- If the parent block is split by TT (horizontal or vertical), and the child blocks are 4x4, 4x8/8x4 and 4x4 (parent 4x16 or 16x4). as shown in Figure 1 (b).

- If the parent block is split by QT, and the child blocks are 4x4 luma samples in size (parent block 8x8 luma samples in size). This is exemplified in Figure 1 (c).

Figure 1, shared merge list region in VVC Specification Draft (Draft 4)

# Problem Statement

In VVC Specification Draft (Draft 4), the IBC mode is defined as a separate prediction mode from inter and intra prediction modes. Therefore, the merge list construction worst case becomes that there is one block using IBC merge mode and another block using regular merge mode in one shared merge list region. For example in Figure 1(a), if block 1 is using IBC mode and block 2 is using regular merge mode, then a IBC merge list need to be constructed for the block 1 and a regular merge list need to be constructed for the block 2.

# Proposed Method

In the proposed method, a regular merge mode and an IBC merge mode are exclusive in a same shared merge list area. If any previous blocks of the current bock is using IBC merge, the current block is not allowed to use regular merge mode; on the other hand, if any previous blocks of the current block is using regular merge mode, the current block is not allowed to use IBC merge mode.

For example, in Figure 1(a), if block 1 is selected IBC merge mode, the block 2 is not allowed using regular merge mode; in Figure 1(b), if any of block 1-2 is using IBC merge mode, the block 2 is now allowed to use regular merge mode.

# Simulation results

Table 1 shows the simulation results using VTM 4.0, as CE8 [3] requested both in the anchor and tests with IBC on for nature sequences, for Class F and TGM both IBC and HashME are on. Since intra slice is not using regular merge mode, the proposed method only affects the RA and LDB tests.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0.01% | -0.19% | -0.08% | 100% | 99% |
| Class C | 0.00% | 0.23% | 0.11% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.01% | -0.06% | -0.04% | 100% | 99% |
| Class F | -0.02% | 0.00% | -0.02% | 100% | 100% |
| TGM | 0.13% | 0.20% | 0.14% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.05% | -0.01% | 0.12% | 100% | 99% |
| Class E | 0.06% | -0.80% | 0.59% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.04% | 1.19% | 0.33% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 1, Exclusive regular merge and IBC merge in the same shared merge list region.

# Draft specification changes

The draft text changes are based on [1], the proposed method is highlighted as follow:

merge_flag[ x0 ][ y0 ] specifies whether the inter prediction parameters for the current coding unit are inferred from a neighbouring inter-predicted partition. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If cu_skip_flag[ x0 ][ y0 ] is equal to 1, merge_flag[ x0 ][ y0 ] is inferred to be equal to 1.

- Otherwise, merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of merge_flag [ x0 ] [ y0 ] shall obey be equal to 0 when IsInSmr[ x0 ][ y0 ] is equal to TRUE and merge_flag [ x ] [ y ] is equal to 1 and CuPredMode[ x ][ y ] is not equal to CuPredMode[ x0 ][ y0 ] for x = SmrX[ x0 ][ y0 ].. SmrW[ x0 ][ y0 ] − 1 and y = SmrY[ x0 ][ y0 ].. SmrH[ x0 ][ y0 ] – 1

# Conclusion

The proposed method are addressing merge list construction worst case problem. By excluding the regular merge and IBC merge in a same shared merge list, the proposed method has 0.00%/xx% in RA/LDB coding loss for CTC overall, 0.03%/0.01% RA/LDB coding loss for ClassF and 0.20%/0.27% RA/LDB coding loss for TGM sequences. The method is proposed to be included in the next version of VVC draft.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations for SDR video ”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

- “Description of Core Experiment 8 (CE8): Screen Content Coding Tools ”, JVET-M1028, X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

- “CE4.3.1: Shared merging candidate list”, JVET-M0170, C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).