| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0177 |
| --- | --- |

| Title: | CE9: Disabling SAD based early termination of BDOF (CE9-2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Biao Wang, Anand Meher Kotra, Han Gao, Jianle Chen Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki | Tel: Email: | semih.esenlik@huawei.com, kenji.kondo@sony.com, |
| Source: | Huawei Technologies Co., Ltd., Sony Corporation |  |  |

_____________________________

# Abstract

The current contribution document reports the result of the CE test CE9-2.1. In CE9-2.1 disabling of SAD based early termination of BDOF is tested. 0.02% luma coding gain with 4% increase in decoding time is observed.

# Introduction

During the 13th JVET meeting in Marrakech, it was requested to test the disabling of SAD based early termination of BDOF as baseline for tests in CE9. The current contribution document reports the result of the CE test CE9-2.1 where the impact of SAD based early termination is tested.

# Problem Definition

Assume a CTU is split horizontally into 2 leaf CUs, and the top CU applies BDOF. Since BDOF is applied to coding blocks that are larger than 64 luma samples in one dimension, two problems exist:

- The CU-based early termination decision requires that SAD matching cost is computed for the whole CU. Accordingly in order the finish the processing of the top-left VPDU, top-right VPDU also needs to be reconstructed. Hence the intended goal of sequential processing of VPDUs cannot be achieved.

- The CU-based early termination decision introduces buffering latency and increased memory requirement for hardware implementations. Actual BDOF operation cannot start before the early termination decision is computed, therefore the additional on-chip memory is required to store reference pixel samples which need to be accessed twice.

# Proposed Solution

According to the proposed solution the SAD based early termination is disabled completely, both CU-based early termination decision and 4x4 block based early termination decision.

It is noted that the SAD early termination can help reduce the decoding time for software implementations, whereas the complexity is increased for hardware implementations.

# Experimental Results

The simulations are performed according to CTC described in [1].

## Simulation result of CE9-2.1

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.02% | 101% | 105% |
| Class A2 | -0.04% | 0.02% | 0.05% | 101% | 105% |
| Class B | -0.02% | -0.03% | 0.01% | 101% | 105% |
| Class C | -0.01% | -0.04% | -0.05% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | 0.00% | 101% | 104% |
| Class D | -0.02% | -0.01% | -0.07% | 100% | 102% |
| Class F | -0.01% | 0.00% | 0.04% | 101% | 113% |

# Conclusion

The current contribution document reports the result of the CE test CE9-2.1. In CE9-2.1 disabling of SAD based early termination of BDOF is additionally tested. 0.02% luma coding gain with 4% increase in decoding time is observed.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

# Draft Specification Changes

### 8.5.7 Decoding process for inter blocks

### 8.5.7.1 General

…

…

…

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.7.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - When one or more of the following conditions is true, mvOffset[ 0 ] is set equal to 0:

    - xSb is not equal to xCb and mvOffset[ 0 ] is less than 0

    - ( xSb + sbWidth ) is not equal to ( xCb + cbWidth) and mvOffset[ 0 ] is greater than 0

  - When one or more of the following conditions is true, mvOffset[ 1 ] is set equal to 0:

    - ySb is not equal to yCb and mvOffset[ 1 ] is less than 0

    - ( ySb + sbHeight ) is not equal to ( yCb + cbHeight ) and mvOffset[ 1 ] is greater than 0

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXL, bdofFlag, and cIdx as inputs.

  - Otherwise if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCb, bdofFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCr, bdofFlag, and cIdx as inputs.

- If bdofFlag is equal to TRUE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variables cuDiffThres, bdofBlkDiffThres, and cuSumDiff are derived as follows:

cuDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) ) * cbWidth*cbHeight 	(8-796)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )	(8-797)

cuSumDiff =  0		(8-798)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff = i=03j=03Abs( predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j ] − 	(8-799)
		predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j] )

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-800)

cuSumDiff += bdofBlkSumDiff		(8-801)

- When cuSumDiff is less than cuDiffThres, bdofFlag is set equal to FALSE.

- The array predSamples of prediction samples is derived as follows:

  - If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived as follows:

    - If bdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.7.4 is invoked with nCbW set equal to the luma coding subblock width sbWidth, nCbH set equal to the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1 and bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) − 1as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

#### 8.5.7.4 Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 2, 14 − bitDepth ).

- The variable shift2 is set to equal to Max( 8, bitDepth − 4 ).

- The variable shift3 is set to equal to Max( 5, bitDepth − 7 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to Max( 2, 1  <<  ( 13 − bitDepth ) ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset2 +	(8-852)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift2 )

- Otherwise (bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).