| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0178 |
| --- | --- |

| Title: | CE9: Implicit splitting of BDOF application region (CE9-2.4 and CE9-2.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haunbang Chen Xiang Ma Semih Esenlik Haitao Yang Jianle Chen | Tel: Email: | 0049 162 210 6260 semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

The current contribution document reports the result of the CE tests CE9-2.4 and CE9-2.6. In CE9-2.4 BDOF application region is implicitly split into maximum 16x16 sub-blocks. In CE9-2.6 SAD based early termination is additionally disabled. Almost no change in coding performance is observed by both modifications, whereas 4% increase in decoding time is observed for CE9-2.6.

# Introduction

The implicit splitting of BDOF region was first proposed in [1]. The current contribution document reports the result of the CE tests CE9-2.4 and CE9-2.6.

# Problem Definition

Assume a CTU is split horizontally into 2 leaf CUs, and the top CU applies BDOF. Since BDOF is applied to coding blocks that are larger than 64 luma samples in one dimension, two problems exist:

- The CU-based early termination decision requires that SAD matching cost is computed for the whole CU. Accordingly in order the finish the processing of the top-left VPDU, top-right VPDU also needs to be reconstructed. Hence the intended goal of sequential processing of VPDUs cannot be achieved.

- BDOF is applied on a 4x4 sub-block basis where gradients are computed for the 6x6 extended window surrounding the sub-block. Similarly to the first problem, the top-right VPDU needs to be reconstructed in order to enable the computation of gradients along the VPDU boundary, before the processing of top-left VPDU is finished. The problem is illustrated in the figure below:

Figure 1: Illustration of delayed processing of BDOF at VPDU boundaries

# Proposed Solution

According to the second proposed solution, if BDOF application region is split into sub-blocks with a size of maximum 16x16 samples, where each sub-block is considered to be an independent BDOF application region. In other words, The BDOF operation considers the CU to be composed of multiple sub-blocks of size 16x16, and the gradients are calculated using padded samples along the 16x16 boundaries that divide the CU into sub-blocks.

It is noted that implicit splitting of BDOF application region is already performed when DMVR and BDOF are applied together, BDOF application region is implicitly split along DMVR sub-blocks. According to the proposal implicit splitting of BDOF is extended to coding blocks that do not apply DMVR. It is further noted that the proposed BDOF sub-block splitting follows the same logic as the DMVR sub-block splitting.

Results for two CE tests are presented in the document:

- CE9-2.4: BDOF application region is split into maximum 16x16 sub-blocks (even if DMVR is not applied)

- CE9-2.6: CE9-2.4 + SAD based early termination is disabled.

It is asserted that both of the proposed solutions solve the 2 problems stated in the Problems section. In CE9-2.6 both CU based and 4x4 SAD based early termination are disabled.

# Experimental Results

The simulations are performed according to CTC described in [2].

## Simulation result of CE9-2.4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.01% | 102% | 101% |
| Class A2 | 0.00% | 0.01% | 0.00% | 102% | 101% |
| Class B | 0.03% | -0.02% | -0.03% | 101% | 101% |
| Class C | 0.02% | -0.01% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.01% | 101% | 100% |
| Class D | 0.08% | -0.05% | 0.01% | 100% | 99% |
| Class F (optional) | 0.01% | 0.01% | 0.01% | 101% | 100% |

## Simulation result of CE9-2.6

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 102% | 105% |
| Class A2 | -0.03% | 0.03% | 0.06% | 102% | 105% |
| Class B | 0.01% | 0.01% | -0.04% | 102% | 105% |
| Class C | 0.01% | -0.04% | -0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 102% | 104% |
| Class D | 0.07% | -0.04% | -0.01% | 101% | 102% |
| Class F (optional) | -0.01% | 0.02% | 0.00% | 102% | 109% |

# Conclusion

The current contribution document reports the result of the CE tests CE9-2.4 and CE9-2.6. In CE9-2.4 BDOF application region is implicitly split into maximum 16x16 sub-blocks. In CE9-2.6 SAD based early termination is additionally disabled. Almost no change in coding performance is observed by both modifications, whereas 4% increase in decoding time is observed for CE9-2.6. It is proposed to include one of the tests in the next VVC draft specification.

# References

- “CE9-related: BDOF Modifications to Enable 64x64 VPDU”, JVET-M1029, H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

# Draft Specification Changes for CE9-2.6

### 8.5.7 Decoding process for inter blocks

### 8.5.7.1 General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index gbiIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

Let predSamplesL0L, predSamplesL1L and predSamplesIntraL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesL0Cb, predSamplesL1Cb, predSamplesL0Cr and predSamplesL1Cr, predSamplesIntraCb, and predSamplesIntraCr be (cbWidth / 2)x(cbHeight / 2) arrays of predicted chroma sample values.

- If numSbY is equal to 1 and numSbX is equal to 1 the following ordered steps are applied

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - GbiIdx[ xCb ][ yCb ] is equal to 0.

      - Idx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

  - If bdofFlag is equal to one, the variables numSbY, numSbX are modified as follows:

numSbX = ( cbWidth > 16 )  ?  ( cbWidth >> 4 ) : 1

numSbY = ( cbHeight > 16 )  ?  ( cbHeight >> 4 ) : 1

  - For X=(0,1), xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

    - predFlagLX[ xSbIdx ][ ySbIdx ] is set equal to predFlagLX[ 0 ][ 0 ]

    - refMvLX[ xSbIdx ][ ySbIdx ] is set equal to refMvLX[ 0 ][ 0 ]

    - mvLX[ xSbIdx ][ ySbIdx ] is set equal to mvLX[ 0 ][ 0 ]

The width and the height of the current coding sublock subCbWidth, subCbHeight in luma samples are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-793)

sbHeight  =  cbHeight / numSbY		(8-794)

…

…

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.7.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - When one or more of the following conditions is true, mvOffset[ 0 ] is set equal to 0:

    - xSb is not equal to xCb and mvOffset[ 0 ] is less than 0

    - ( xSb + sbWidth ) is not equal to ( xCb + cbWidth) and mvOffset[ 0 ] is greater than 0

  - When one or more of the following conditions is true, mvOffset[ 1 ] is set equal to 0:

    - ySb is not equal to yCb and mvOffset[ 1 ] is less than 0

    - ( ySb + sbHeight ) is not equal to ( yCb + cbHeight ) and mvOffset[ 1 ] is greater than 0

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXL, bdofFlag, and cIdx as inputs.

  - Otherwise if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCb, bdofFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCr, bdofFlag, and cIdx as inputs.

- If bdofFlag is equal to TRUE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variables cuDiffThres, bdofBlkDiffThres, and cuSumDiff are derived as follows:

cuDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) ) * cbWidth*cbHeight 	(8-796)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )	(8-797)

cuSumDiff =  0		(8-798)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff = i=03j=03Abs( predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j ] − 	(8-799)
		predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j] )

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-800)

cuSumDiff += bdofBlkSumDiff		(8-801)

- When cuSumDiff is less than cuDiffThres, bdofFlag is set equal to FALSE.

- The array predSamples of prediction samples is derived as follows:

  - If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived as follows:

    - If bdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.7.4 is invoked with nCbW set equal to the luma coding subblock width sbWidth, nCbH set equal to the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1 and bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) − 1as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

#### 8.5.7.4 Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 2, 14 − bitDepth ).

- The variable shift2 is set to equal to Max( 8, bitDepth − 4 ).

- The variable shift3 is set to equal to Max( 5, bitDepth − 7 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to Max( 2, 1  <<  ( 13 − bitDepth ) ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset2 +	(8-852)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift2 )

- Otherwise (bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).