| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0180-v1 |
| --- | --- |

| Title: | CE5-2: Loop filter line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Jianle Chen, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution reports the results for CE5-2.1, CE5-2.2 and CE5-2.3 core experiments. CE5-2 provides a mechanism of reducing the line buffer requirement of ALF (adaptive loop filter) by using the concept of virtual boundaries (VBs) which are upward shifted horizontal CTU boundaries by “N” samples. Modified ALF block classification and modified ALF filtering are applied for the samples which are near the virtual boundary to reduce the number of line buffers required. Modified ALF block classification only uses the samples which are above the VB to classify the given 4 x 4 block which is above VB. Similarly for the classification of the 4 x 4 block below VB, samples belonging to the lines below the VB are used.   Modified ALF filtering uses a combination of conditional disabling and truncated versions of the original ALF filter.

CE 5-2.1 uses modified block classification and truncated versions of the filter. The objective results are as follows: Over VTM 4.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of 0.06%, 0.10%, 0.09%, 0.11% with no increase in EncT and DecT

CE 5-2.2 additionally uses conditional disabling on top of CE 5-2.1. The objective results are as follows: Over VTM 4.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of 0.06%, 0.10%, 0.06%, 0.12% with no increase in EncT and DecT

CE 5-2.3 is a combination test with CE 5-1.2 where block based classification method of CE 5-2 is used on top of CE 5-1.2. The objective results are as follows: Over VTM 4.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of 0.07%, 0.09%, 0.08%, 0.14% with no increase in EncT and DecT

All CE methods ensure that ALF line buffer requirement is zero lines.

# Introduction

VTM 4.0 [2] consists of three cascaded in-loop filter which are namely Deblocking filter (DBF), Sample adaptive offset (SAO), and Adaptive loop filter (ALF).  The total amount of line buffer requirement for all three loop filters is deduced as: 11.25 lines for Luma component and 6.25 lines for Chroma component.

It is emphasized that 11.25 lines of Luma line buffer and 6.25 lines of Chroma line buffer is considered expensive for hardware implementations of the VVC standard and therefore reduction of line buffer requirement of loop filter is desired.

The current contribution provides the results for CE5-2 which ensure that the line buffer requirement of ALF filter is zero lines.

# Proposed Solution

CE 5-2.1 uses modified ALF block classification and the truncated filtering. CE5-2.2 additionally uses conditional disabling. CE 5-2.3 is a combination test with CE 5-1.2 [1][4] where block based classification method of CE 5-2 is used on top of CE 5-1.2.

Modified ALF block classification, truncated filtering and conditional disabling are further explained below.

## Modified ALF block classification

Fig. 3a and 3b depict modified block classification for the case when the virtual boundary is 4 lines (N==4) above the CTU boundary. As depicted in Fig. 3a, for the 4 x 4 block starting at line G, the block classification only uses the lines E till J.  However Laplacian gradient calculation for the samples belonging to line J requires one more line below (line K). Therefore line K is padded with line J.

Similarly as depicted in Fig. 3b, for the 4 x 4 block starting at line K, the block classification only uses the lines K till P.  However Laplacian gradient calculation for the samples belonging to line K require one more line above (line J). Therefore line J is padded with line K.

Figure 3. Modified ALF block classification at virtual boundary

## Modified ALF block filtering

### Truncated ALF filtering at virtual boundaries

As depicted in Fig.4, truncated version of the filters are used for filtering of the Luma samples belonging to the lines close to the virtual boundaries.

Figure 4. Modified Luma ALF filtering at virtual boundary

Similarly as depicted in Fig. 5, truncated version of the filters are also used for chroma ALF filtering

Figure 5. Modified Chroma ALF filtering at virtual boundary

### Conditional Disabling of ALF filtering at virtual boundaries

In addition to the truncated version of the ALF filtering, conditional disabling of ALF filtering is also considered.

In Figure 4, if absolute sum of the padded coefficients is greater than a threshold (T) times the absolute sum of the non-padded coefficients, then the filter is disabled for those respective lines.

For e.g. with lines M and H, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines M and H.

Similarly for lines L and I, if (2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5+absc6+absc7+absc8+absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines L and I.

Similarly for lines J and K, if (2*(abs c0+ abs c1 + abs c2+ abs c3+absc4+absc5+absc6+absc7+absc8)>(T* 2*absc9+absc10+absc11+absc12), then the ALF filtering is disabled for lines J and K.

Where T is generally a threshold which can take a fractional values ranging between 0 and 1. For example, T value can be 0.5, 0.6 or 0.7

Similarly for Chroma, as depicted in Fig. 5, for lines N and K, if (2*abs  c0>(T* 2*abs c1+abs c2+absc3+absc4+absc5)+absc6), then the ALF filtering is disabled for lines N and K.

for lines M and L, if ((2*(abs c0+ abs c1 + abs c2+ abs c3)>(T* 2*absc4+absc5)+absc6), then the ALF filtering is disabled for lines M and L.

the function abs(x) is defined as follows;Abs( x ) =  x;x  >=  0-x;x < 0

# Simulation Results

The proposed methods are implemented on top of VTM4.0 [2] and the simulation is performed under common test conditions as described in JVET-M1010 [6] with ALF SIMD turned off in both the Anchor and Test.

## Results for CE5-2.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.13% | 0.13% | 100% | 102% |
| Class A2 | 0.08% | 0.08% | 0.07% | 100% | 102% |
| Class B | 0.05% | 0.05% | 0.05% | 100% | 102% |
| Class C | 0.06% | 0.05% | 0.07% | 100% | 100% |
| Class E | 0.08% | 0.07% | 0.03% | 100% | 101% |
| Overall | 0.06% | 0.07% | 0.07% | 100% | 101% |
| Class D | 0.03% | 0.04% | 0.04% | 100% | 100% |
| Class F | 0.07% | 0.04% | 0.03% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.10% | 0.12% | 100% | 103% |
| Class A2 | 0.13% | 0.07% | 0.07% | 100% | 102% |
| Class B | 0.09% | 0.08% | -0.04% | 100% | 102% |
| Class C | 0.09% | 0.00% | 0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.06% | 0.03% | 100% | 102% |
| Class D | 0.01% | -0.04% | 0.00% | 100% | 101% |
| Class F | 0.06% | 0.04% | 0.00% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.09% | 0.16% | 100% | 102% |
| Class C | 0.07% | 0.29% | 0.18% | 101% | 102% |
| Class E | 0.13% | 0.47% | -0.13% | 101% | 101% |
| Overall | 0.09% | 0.18% | 0.09% | 100% | 102% |
| Class D | -0.02% | -0.29% | -0.28% | 101% | 102% |
| Class F | 0.02% | 0.07% | -0.45% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | 0.01% | 0.30% | 100% | 102% |
| Class C | 0.09% | -0.14% | -0.27% | 100% | 101% |
| Class E | 0.14% | 0.26% | -0.07% | 100% | 101% |
| Overall | 0.11% | 0.02% | 0.02% | 100% | 102% |
| Class D | 0.07% | 0.52% | -0.32% | 101% | 102% |
| Class F | 0.16% | 0.16% | -0.34% | 101% | 102% |

## Results for CE5-2.2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.19% | 0.18% | 100% | 101% |
| Class A2 | 0.09% | 0.09% | 0.08% | 100% | 101% |
| Class B | 0.05% | 0.06% | 0.06% | 100% | 101% |
| Class C | 0.04% | 0.05% | 0.08% | 100% | 100% |
| Class E | 0.08% | 0.12% | 0.10% | 100% | 102% |
| Overall | 0.06% | 0.09% | 0.09% | 100% | 101% |
| Class D | 0.03% | 0.04% | 0.04% | 100% | 103% |
| Class F | 0.05% | 0.04% | 0.03% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.07% | 0.15% | 100% | 101% |
| Class A2 | 0.15% | 0.00% | 0.11% | 100% | 101% |
| Class B | 0.11% | 0.04% | 0.07% | 100% | 101% |
| Class C | 0.04% | 0.04% | 0.01% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.04% | 0.08% | 100% | 101% |
| Class D | 0.00% | -0.15% | -0.17% | 100% | 102% |
| Class F | 0.05% | 0.06% | 0.02% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.14% | -0.07% | 100% | 101% |
| Class C | 0.04% | 0.32% | 0.17% | 100% | 101% |
| Class E | 0.08% | 0.21% | 0.28% | 100% | 101% |
| Overall | 0.06% | 0.10% | 0.10% | 100% | 101% |
| Class D | -0.01% | 0.19% | -0.48% | 101% | 102% |
| Class F | -0.01% | 0.08% | 0.10% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.02% | 0.12% | 100% | 101% |
| Class C | 0.08% | -0.12% | 0.03% | 100% | 102% |
| Class E | 0.15% | 0.10% | 0.18% | 100% | 102% |
| Overall | 0.12% | -0.02% | 0.11% | 100% | 102% |
| Class D | 0.06% | -0.05% | 0.07% | 100% | 102% |
| Class F | 0.11% | -0.47% | -0.37% | 100% | 101% |

## Results for the CE5-2.3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.21% | 0.16% | 100% | 101% |
| Class A2 | 0.07% | 0.13% | 0.08% | 100% | 101% |
| Class B | 0.06% | 0.07% | 0.07% | 100% | 101% |
| Class C | 0.05% | 0.07% | 0.09% | 100% | 102% |
| Class E | 0.11% | 0.11% | 0.19% | 101% | 103% |
| Overall | 0.07% | 0.11% | 0.11% | 100% | 101% |
| Class D | 0.03% | 0.06% | 0.06% | 100% | 102% |
| Class F | 0.06% | 0.06% | 0.05% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.07% | 0.00% | 100% | 101% |
| Class A2 | 0.09% | -0.10% | 0.04% | 100% | 100% |
| Class B | 0.09% | 0.18% | 0.06% | 100% | 101% |
| Class C | 0.08% | -0.01% | 0.00% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.02% | 0.03% | 100% | 101% |
| Class D | 0.09% | -0.05% | -0.11% | 100% | 102% |
| Class F | 0.04% | 0.04% | 0.08% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.10% | 0.12% | 100% | 102% |
| Class C | 0.06% | 0.46% | -0.10% | 101% | 103% |
| Class E | 0.15% | 1.02% | -0.26% | 101% | 102% |
| Overall | 0.08% | 0.45% | -0.05% | 101% | 102% |
| Class D | 0.05% | 0.05% | -0.09% | 101% | 104% |
| Class F | 0.01% | -0.17% | 0.15% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.20% | 0.19% | 100% | 101% |
| Class C | 0.10% | 0.05% | -0.28% | 101% | 103% |
| Class E | 0.14% | 0.71% | 0.21% | 101% | 102% |
| Overall | 0.14% | 0.27% | 0.04% | 101% | 102% |
| Class D | 0.06% | 0.04% | -0.31% | 100% | 103% |
| Class F | 0.14% | 1.02% | 0.08% | 101% | 102% |

# Conclusion

This contribution provides results for CE5-2.1, CE5-2.2 and CE5-2.3. All three methods reduce the line buffer requirement of ALF filter to zero lines. Objective results shows that the Luma BD-Rate loss is less than 0.1% without any increase in EncT and DecT.  Based on the subjective evaluation, it is suggested to adopt one of the proposed CE tests to the VVC draft specification and VTM reference software. Specification text changes are attached along with the contribution.

# References

- Description of Core Experiment 9 (CE5): Adaptive loop filter, Document JVET-M1025, 	C.-Y. Chen, V. Seregin

- Versatile Video Coding (Draft 4), Document JVET-M1001_v5, B. Bross, J. Chen, S. Liu

- Non-CE: Loop filter line buffer reduction, Document  JVET-M0301, A. M. Kotra et al.

- Adaptive loop filter with virtual boundary processing, Document JVET-M0164, C.-Y.Chen et al.

- Non-CE8.c.7: Single-source SAO and ALF virtual boundary processing with cross9x9, JCTVC-G212, C.-Y.Chen et al.

- “JVET common test conditions and software reference configurations”, JVET-M1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).