| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0181-v1 |
| --- | --- |

| Title: | CE11-2.3: Disabling of sub-pu deblociing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Jianle Chen, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution reports the results for CE11-2.3. This CE test evaluates the impact of sub-pu boundaries deblocking.

Objective results of disabling sub-PU boundary deblocking are as follows

Over VTM-4.0 Anchor with ALF ON (AI, RA, LDB, LDP): 0.00%, 0.04%, 0.10%, 0.06% with no change in EncT and 2% decrease in DecT.

Over VTM-4.0 Anchor with ALF OFF (AI, RA, LDB, LDP): ??%, ??%, ??%, ??% with no change in EncT and 2% decrease in DecT.

# Introduction

VTM-4.0 [1] [2] performs deblocking on 8 x 8 sample grid. Beside the CU and TU boundaries, the sub-PU boundaries which result due to tools like Affine, ATMVP, CIIP tools are also deblocked. CE11-2.3 test checks the coding performance of sub-PU boundaries deblocking in VTM-4.0.

# Experimental Results

The simulation is performed under common test conditions as described in JVET-M1010 [3].

The results are presented for both ALF ON and ALF OFF conditions as suggested to be reported by the CE11 description document [4].

The results are as follows:

Table 1. Objective results of the proposed deblocking method on top of VTM-4.0 for CTC configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.03% | 0.00% | 101% | 99% |
| Class A2 | 0.01% | -0.08% | 0.03% | 101% | 98% |
| Class B | 0.03% | -0.06% | -0.04% | 100% | 98% |
| Class C | 0.03% | -0.03% | 0.03% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.05% | 0.00% | 100% | 98% |
| Class D | -0.02% | -0.06% | -0.03% | 100% | 98% |
| Class F | 0.04% | 0.06% | 0.04% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.10% | -0.12% | -0.31% | 101% | 99% |
| Class C | 0.03% | 0.25% | -0.02% | 100% | 98% |
| Class E | 0.18% | 0.17% | -0.27% | 100% | 94% |
| Overall | 0.10% | 0.08% | -0.21% | 100% | 97% |
| Class D | 0.00% | -0.23% | -0.24% | 100% | 99% |
| Class F | -0.02% | 0.10% | -0.48% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.04% | 0.32% | 100% | 98% |
| Class C | 0.04% | -0.26% | -0.12% | 100% | 99% |
| Class E | 0.05% | 0.09% | -0.54% | 100% | 94% |
| Overall | 0.06% | -0.05% | -0.04% | 100% | 98% |
| Class D | 0.07% | 0.22% | -0.54% | 100% | 98% |
| Class F | 0.09% | -0.23% | -0.04% | 100% | 98% |

Table 2.   Objective results of the proposed deblocking method on top of VTM-4.0 with ALF OFF

(to be updated)

# Conclusion

This contribution reports the results of CE11-2.3 which disables the deblocking of sub-PU boundaries.  For CTC test conditions, the test results in overall Luma BD-rate loss of 0.05% when ALF is on and more loss is found for affine sequences in low delay case.

# References

- B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 4)," Document of Joint Video Experts Team, JVET-M1001.

- VTM-4.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-M1010.

- A. Norkin, A. M. Kotra, "Description of Core Experiment 11 (CE11): Deblocking," Document of Joint Video Experts Team, JVET-M1031.