| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0182-v1 |
| --- | --- |

| Title: | CE11-related: Simplification of sub-pu deblocking and longer tap filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Semih Esenlik, Jianle Chen, Biao Wang, Han Gao, | Tel: Email: | anand.meher.kotra@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes two simplifications to the existing deblocking filter design in VTM-4.0. The first simplification suggests to not apply longer tap filter for any of the implicit transform unit edges. The second simplification proposes to disable deblocking of the first and last sub-pu edges aligning on the 8 x 8 sample grid for a given coding unit which use sub-pu tools like Affine, ATMVP or CIIP tools.  These two simplifications reportedly result in reduced number of conditional checks in the deblocking filter design.

Objective results are as follows

Over VTM-4.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.00%, 0.03%, 0.05%, 0.01% is reported with no change in EncT and DecT

Internal subjective evaluation reportedly shows no visual degradation when compared to VTM-4.0

# Introduction

VTM-4.0 [1] allows a longer tap filer to be applied for coding unit edges and implicit TU edges whenever “large block condition” is satisfied. However to enable parallel friendly deblocking design using both long filters and sub-block deblocking the long filters is restricted to modify at most 5 samples on a side that uses sub-block deblocking (AFFINE, ATMVP or CIIP). Additionally the sub-block deblocking is adjusted such that that sub-block boundaries on an 8x8 grid that are close to a CU or an implicit TU boundary is restricted to modify at most two samples on each side.

Following is applied to sub-block boundaries that not are aligned with the CU boundary.

If(mode block Q == SUBBLOCKMODE && edge!=0){

if (!(implicitTU && (edge == (64 / 4))))

if (edge == 2 || edge == (orthogonalLength - 2) || edge == (56 / 4) || edge == (72 / 4))

Sp = Sq = 2;

else

Sp = Sq = 3;

else

Sp = Sq = bSideQisLargeBlk ? 5:3

}

Where edge equal to 0 corresponds to CU boundary, edge equal to 2 or equal to orthogonalLength-2 corresponds to sub-block boundary 8 samples from a CU boundary etc. Where implicit TU is true if implicit split of TU is used.

Therefore the above design results in additional conditional checks and filters with different filter tap lengths like (5/7, 7/5, 5/5) and also the additional restriction of only modifying at most 2 samples when the sub-pu edge is close to the CU boundary or implicit TU boundary

The current proposal targets to reduce the number of conditional checks, reduce filters with different filter tap lengths, and remove additional restrictions of the sub-pu edge deblocking.

# Proposal

As shown in Fig.1 below, for the coding units which use sub-pu tools like Affine, ATMVP or CIIP, the first and last sub-pu edges belonging to the coding unit are not deblocked. This allows direct application of the longer tap filter for the coding unit edges without any additional conditional checks. Furthermore for the implicit TU edges, only normal VVC strong or weak filter is allowed to be applied.

Figure 1. Sub-pu deblocking is applied to all sub-pu edges except for the first and last sub-pu edges aligning with 8 x 8 sample grid

# Experimental results

The proposed method is implemented on top of VTM-4.0 [2] and the simulation is performed under common test conditions as described in JVET-M1010 [3].

Table 1.   Objective results of the proposed deblocking method on top of VTM-4.0 for CTC configurations

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.03% | 0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.04% | 101% | 100% |
| Class B | 0.03% | -0.01% | 0.02% | 100% | 100% |
| Class C | 0.03% | -0.02% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.05% | 0.02% | 100% | 100% |
| Class F | 0.02% | 0.01% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.07% | -0.36% | 101% | 101% |
| Class C | 0.00% | 0.19% | -0.02% | 100% | 99% |
| Class E | 0.12% | 0.57% | -0.47% | 100% | 100% |
| Overall | 0.05% | 0.18% | -0.28% | 100% | 100% |
| Class D | -0.01% | 0.07% | -0.53% | 99% | 98% |
| Class F | -0.09% | -0.09% | -0.35% | 100% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.16% | -0.06% | 100% | 101% |
| Class C | 0.03% | -0.37% | -0.54% | 100% | 101% |
| Class E | -0.04% | -0.27% | 0.06% | 100% | 100% |
| Overall | 0.01% | -0.26% | -0.19% | 100% | 101% |
| Class D | 0.05% | 0.07% | -0.57% | 100% | 101% |
| Class F | 0.12% | -0.02% | -0.02% | 100% | 101% |

# Conclusion

This contribution proposes two simplifications in the deblocking design of VTM-4.0. First simplification restricts longer tap filter not to be applied for implicit TU edges and the second simplification proposes to disable the first and last sub-pu edges aligning on the 8 x 8 sample grid to directly allow application of the longer tap filter without further conditional checks. It is reported that these two simplifications results in a simplified design and specification text and at the same time not degrading the subjective quality. Objective numbers with CTC configuration reportedly show negligible BD-Rate loss. It is suggested to further study these simplifications in a deblocking core experiment.

# References

- B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 4)," Document of Joint Video Experts Team, JVET-M1001.

- VTM-4.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-M1010.

- A. Norkin, A. M. Kotra, "Description of Core Experiment 11 (CE11): Deblocking," Document of Joint Video Experts Team, JVET-M1031.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd  may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).