| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0185-r2 |
| --- | --- |

| Title: | CE3-related: A unified MPM list for intra mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Wang, A. M. Kotra, S. Esenlik H. Gao, J. Chen (Huawei) | Tel: Email: | biao.wang@huawei.com, |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the result of a method using a unified 6-MPM list in the intra mode coding, regardless of whether multiple reference line (MRL) or intra sub-partition (ISP) coding mode is applied or not. Compared to VTM4.0, this method achieve 0.06% coding gain in AI, with 101% encoding and 100% decoding time observed.

# Introduction

During the 13th JVET meeting in Marrakech, it was requested to harmonize the MPM list generation for different coding tools, such as Multiple Reference Line (MRL), Intra sub-partition (ISP) mode coding. In VTM4.0, when MRL is applied planar and DC modes are excluded in MPM list and when ISP is applied DC mode is excluded from the MPM list. The current contribution proposes a unified MPM list for different coding tools including MRL and ISP. The unified MPM list always includes planar and DC modes and the restriction regarding planar and DC mode in MPM list for MRL and ISP coding tools are removed.

# Problem Definition

In VTM4.0, three different 6-MPM list generations are used for different coding tools, including:

- A 6-MPM list for intra blocks with MRL mode enabled, where planar and DC mode are excluded

- A 6-MPM list for intra blocks with ISP mode enabled, where DC mode is excluded

- A 6-MPM list for normal intra block, where planar and DC are always included.

For each of above 6-MPM list, different 6-MPM construction is used and this makes the MPM list construction complicated. A complex MPM list construction might compromise the throughput of the decoder, in particular for small blocks such as 4x4 samples.

# Proposed Solutions

A unified 6-MPM list is proposed for intra blocks irrespective of whether MRL and ISP coding tools are applied or not. The MPM list is constructed based on intra modes of the left and above neighboring block as in VTM4.0. Suppose the mode of the left is denoted as Left and the mode of the above block is denoted as Above, the unified MPM list is constructed as follows:

- When a neighboring block is not available, its intra mode is set to Planar by default.

- If both modes Left and Above are non-angular modes:

  - MPM list  {Planar, DC, V, H, V-4, V+4}

- If one of modes Left and Above is angular mode, and the other is non-angular:

  - Set a mode Max as the larger mode in Left and Above

  - MPM list  {Planar, Max, DC, Max -1, Max +1, Max -2}

- If Left and Above are both angular and they are different:

  - Set a mode Max as the larger mode in Left and Above

  - if the difference of mode Left and Above is in the range of 2 to 62, inclusive

    - MPM list  {Planar, Left, Above, DC, Max -1, Max +1}

  - Otherwise

    - MPM list  {Planar, Left, Above, DC, Max -2, Max +2}

- If Left and Above are both angular and they are the same:

  - MPM list  {Planar, Left, Left -1, Left +1, DC, Left -2}

Besides, the first bin of the mpm index codeword is CABAC context coded. In total three contexts are used, corresponding to whether the current intra block is MRL enabled, ISP enabled, or a normal intra block.

# Experimental Results

The simulations are performed according to CTC described in [1]. AI and RA results are reported.

## Simulation result

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.26% | -0.20% | 100% | 101% |
| Class A2 | -0.05% | 0.04% | -0.05% | 101% | 101% |
| Class B | -0.06% | 0.06% | 0.09% | 101% | 100% |
| Class C | 0.08% | 0.05% | 0.05% | 102% | 99% |
| Class E | 0.04% | 0.19% | 0.12% | 101% | 99% |
| Overall | -0.06% | 0.02% | 0.02% | 101% | 100% |
| Class D | -0.08% | 0.06% | -0.17% | 101% | 100% |
| Class F | 0.22% | 0.19% | 0.17% | 102% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.20% | -0.08% | 101% | 101% |
| Class A2 | 0.04% | 0.05% | 0.10% | 102% | 101% |
| Class B | -0.01% | 0.10% | -0.18% | 101% | 100% |
| Class C | 0.04% | -0.06% | -0.14% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.09% | 101% | 100% |
| Class D | -0.02% | -0.12% | -0.10% | 100% | 99% |
| Class F | 0.18% | 0.05% | -0.01% | 101% | 100% |

# Conclusion

The current contribution document reports the results of using a unified MPM list in intra mode coding. The presented MPM list is shared between intra blocks applied with MRL, ISP or normal intra blocks. It harmonized the different MPM list construction in VTM4.0, meanwhile it provides 0.06% coding gain in AI, with no encoding and decoding time increase observed.

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).