| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0187 |
| --- | --- |

| Title: | CE9-2.2: On early termination of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, we introduce solutions to address the increase in hardware complexity due to the computation of the sum of absolute difference (SAD) for early termination in bi-directional optical flow (BDOF). Although the SAD calculation is performed at CU level, it could potentially increase the memory size up to CTU level. We propose to divide the SAD calculation to avoid this potential increase in memory size. If we divide the SAD calculation from 128x128 to 16x16 regions to reduce memory size, our simulation results show almost no drop in BD performance i.e. 0.03%, -0.01% and -0.03% BD-rate for Y, U and V component in Random Access (RA) configuration.

Introduction

In the contribution JVET-M0073 [1], it was claimed that current design of BDOF increase memory size because it needs SAD calculation for CU level as shown in Figure 1.

Figure 1: Early stage of BDOF process

This issue was discussed in core experiment (CE) 9. And it’s planned to test CE9-2.2 [2].

Solutions of CE9-2.2

To avoid having large memory, an early termination region is divided to 16x16 as show in Figure 2. Current design of BDOF checks an early termination for CU level. As introduced in [1], it increases memory size. In this solution, the region for early termination was divided for each 16x16 region. This solution allows calculating SAD for each 16x16 region. Consequently, this solution doesn’t need large memory corresponding to CU size.

Figure 2: CU boundary and early termination region

In some cases, CU of BDOF is split as smaller than 16. If the CU size is smaller than 16, the size of early termination region is aligned to CU boundary as shown in Figure 3.

Figure 3: Early termination region in small CU case

A threshold to decide early termination is calculated based on the threshold of 4x4. For example, in case of 16x16 the threshold of early termination is sixteen times of threshold of 4x4. As mentioned above, early termination size may be smaller than 16x16. In those cases, the threshold is proportional to the area of early termination region.

The flowchart of implementation is shown in Figure 4.

Figure 4: An implementation for early termination

In prediction block, it’s predicted using the optical flow algorithm. The prediction runs each early termination region 16x16. If the SAD value is smaller than the threshold, the region is skipped and replaced usual bi-prediction.

Test cases

As described in [2], this contribution reports test results for three cases. In VTM-3.0, BDOF has 2-stages early termination for CU level and 4x4 block level. As mentioned in JVET-M0073, the CU level early termination requires large buffer size to store inter prediction blocks. To consider the trade-off for hardware complexity and decoding time, it’s tested that the SAD region for early termination is divided for each 16x16 size sub-block to reduce buffer size.

1) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination.

2) The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled.

3) The early termination for 1st stage (CU level) is disabled.

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9-2.2a | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. | K. Kondo | T. Chujoh |
| CE9-2.2b | The SAD region is divided for each 16x16 sub-blocks for 1st stage of early termination. And an early termination for 2nd stage (4x4 block level) is disabled. | K. Kondo | T. Chujoh |
| CE9-2.2c | The early termination for 1st stage (CU level) is disabled. | K. Kondo | T. Chujoh |

Experiment

As introduced above, three tests carried out. JVET common test condition [3] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: SSE42.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: SSE42

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

The summary of test results

Test results are shown in Table 1, Table 2 and Table 3 for CE9-2.2a, CE9-2.2b and CE9-2.2c.

Table 1 Test results for CE9-2.2a

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.06% | 101% | 102% |
| Class A2 | 0.05% | -0.02% | -0.03% | 101% | 102% |
| Class B | 0.04% | 0.03% | 0.00% | 100% | 102% |
| Class C | 0.02% | -0.03% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.01% | -0.03% | 100% | 102% |
| Class D | 0.02% | 0.01% | -0.03% | 100% | 101% |
| Class F | 0.01% | -0.02% | -0.04% | 100% | 107% |

Table 2 Test results for CE9-2.2b

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.02% | 101% | 102% |
| Class A2 | 0.05% | 0.01% | 0.00% | 101% | 103% |
| Class B | 0.03% | 0.00% | -0.01% | 101% | 103% |
| Class C | 0.01% | -0.02% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | -0.02% | 101% | 102% |
| Class D | 0.01% | -0.07% | -0.12% | 100% | 101% |
| Class F | 0.01% | -0.01% | -0.05% | 101% | 107% |

Table 3 Test results for CE9-2.2c

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 101% | 103% |
| Class A2 | -0.02% | 0.02% | 0.03% | 101% | 103% |
| Class B | 0.01% | 0.00% | 0.01% | 101% | 103% |
| Class C | 0.00% | -0.05% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 101% | 103% |
| Class D | 0.00% | 0.00% | -0.03% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.01% | 100% | 109% |

As shown in test results, the impact of coding efficiency is negligible. For the decoder runtime, CE9-2.2a and CE9-2.2b is slightly faster than CE9-2.2c. The reason is expected as that CE9-2.2a and CE9-2.2b can skip to process BDOF for early termination region 16x16, but CE9-2.2c doesn’t early exit.

Recommendation

Based on the experimental results, it is recommended to adopt the CE9-2.2a or CE9-2.2b.

References

[1] Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki, “Non-CE9: On early termination for BDOF”, JVET-M0073, the 13th JVET meeting, Marrakech, MA, Jan. 2019.

[2] Semih Esenlik, Xiaoyu Xiu, “Description of Core Experiment 9 (CE9): Decoder Motion Vector Derivation”, JVET-M1029, the 13th JVET meeting, Marrakech, MA, Jan. 2019.

[3]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR”, Document: JVET-M0449

Draft Specification Changes

- 8.5.7 Decoding process for inter blocks

- 8.5.7.1 General

- ...

- For X being each of 0 and 1, when predFlagLX[ xSbIdx ][ ySbIdx ] is equal to 1, the following applies:

  - The reference picture consisting of an ordered two-dimensional array refPicLXL of luma samples and two ordered two-dimensional arrays refPicLXCb and refPicLXCr of chroma samples is derived by invoking the process specified in clause 8.5.7.2 with X and refIdxLX as inputs.

  - The motion vector offset mvOffset is set equal to refMvLX[ xSbIdx ][ xSbIdx ] − mvLX[ xSbIdx ][ ySbIdx ].

  - When one or more of the following conditions is true, mvOffset[ 0 ] is set equal to 0:

    - xSb is not equal to xCb and mvOffset[ 0 ] is less than 0

    - ( xSb + sbWidth ) is not equal to ( xCb + cbWidth) and mvOffset[ 0 ] is greater than 0

  - When one or more of the following conditions is true, mvOffset[ 1 ] is set equal to 0:

    - ySb is not equal to yCb and mvOffset[ 1 ] is less than 0

    - ( ySb + sbHeight ) is not equal to ( yCb + cbHeight ) and mvOffset[ 1 ] is greater than 0

  - If cIdx is equal to 0, the following applies:

    - The array predSamplesLXL  is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth, the coding subblock height sbHeight in luma samples, the luma motion vector offset mvOffset, the refined luma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXL, bdofFlag, and cIdx as inputs.

  - Otherwise if cIdx is equal to 1, the following applies:

    - The array predSamplesLXCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCb, bdofFlag, and cIdx as inputs.

  - Otherwise (cIdx is equal to 2), the following applies:

    - The array predSamplesLXCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.7.3 with the luma location ( xCb, yCb ), the coding subblock width sbWidth / 2, the coding subblock height sbHeight / 2, the chroma motion vector offset mvOffset, the refined chroma motion vector refMvLX[ xSb ][ xSb ], the reference array refPicLXCr, bdofFlag, and cIdx as inputs.

- If bdofFlag is equal to TRUE, the following applies:

- The variable shift is set equal to Max( 2, 14 − BitDepthY ).

- The variables cuDiffThres, bdofBlkDiffThres, and cuSumDiff etSumDiff[ etxIdx ][ etyIdx ], numEtxIdx and numEtyIdx are derived as follows:

cuDiffThres  =  ( 1 << ( BitDepthY − 8 + shift ) ) * cbWidth*cbHeight etDiffThres = ( 1 << ( BitDepthY − 3 + shift )) *  (cbWidth>>Max(0,  Log2( cbWidth ) ))*(cbHeight>> Max(0,  Log2( cbHeight ) ) 	(8-796)

bdofBlkDiffThres  =  1 << ( BitDepthY − 3 + shift )	(8-797)

numEtxIdx = Max( 1, cbWidth >> 4), numEtyIdx = Max( 1, cbHeight >> 4)

cuSumDiff = 0 etSumDiff[etxIdx ][ etyIdx ] =  0	 for etxIdx = 0 .. numEtxIdx-1 and etyIdx = 0 .. numEtyIdx-1	(8-798)

- For xIdx = 0..(sbWidth >> 2 ) − 1 and yIdx = 0..( sbHeight >> 2 ) − 1, the variables bdofBlkSumDiff and the bidirectional optical flow utilization flag bdofUtilizationFlag[ xIdx ][ yIdx ] are derived as follows:

bdofBlkSumDiff = i=03j=03Abs( predSamplesL0L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j ] − 	(8-799)
		predSamplesL1L[ ( xIdx << 2 ) + 1 + i ][ ( yIdx << 2 ) + 1 + j] )

bdofUtilizationFlag[ xIdx ][ yIdx ] = 	bdofBlkSumDiff >= bdofBlkDiffThres	 (8-800)

cuSumDiff etSumDiff[xIdx>>2 ][ yIdx>>2] += bdofBlkSumDiff		(8-801)

- When cuSumDiff is less than cuDiffThres, bdofFlag is set equal to FALSE.

- The array predSamples of prediction samples is derived as follows:

- If cIdx is equal to 0, the prediction samples inside the current luma coding subblock, predSamples[ xL + xSb ][ yL + ySb ] with xL = 0..sbWidth − 1 and yL = 0..sbHeight − 1, are derived as follows:

- If bdofFlag is equal to TRUE, the bidirectional optical flow sample prediction process as specified in clause 8.5.7.4 is invoked with nCbW set equal to the luma coding subblock width sbWidth, nCbH set equal to the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, etSumDiff[etxIdx ][ etyIdx ] for etxIdx = 0 .. numEtxIdx-1 and etyIdx = 0 .. numEtyIdx-1 , etDiffThres and bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( sbWidth >> 2 ) − 1, yIdx = 0..( sbHeight >> 2 ) − 1as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- Otherwise (bdofFlag is equal to FALSE), the weighted sample prediction process as specified in clause 8.5.7.5 is invoked with the luma coding subblock width sbWidth, the luma coding subblock height sbHeight and the sample arrays predSamplesL0L and predSamplesL1L, and the variables predFlagL0[ xSbIdx ][ ySbIdx ], predFlagL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, gbiIdx, and cIdx as inputs, and predSamples[ xL + xSb ][ yL + ySb ] as outputs.

- 8.5.7.4  Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the sum of absolute difference for early termination reagion etSumDiff[etxIdx ][ etyIdx ] with etxIdx = 0 .. numEtxIdx-1 and etyIdx = 0 .. numEtyIdx-1 ,

- the threhold for early termination region etDiffThres,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

Variables bitDepth, shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable bitDepth is set equal to BitDepthY.

- The variable shift1 is set to equal to Max( 2, 14 − bitDepth ).

- The variable shift2 is set to equal to Max( 8, bitDepth − 4 ).

- The variable shift3 is set to equal to Max( 5, bitDepth − 7 ).

- The variable shift4 is set equal to Max( 3, 15 − bitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to Max( 2, 1  <<  ( 13 − bitDepth ) ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If etSumDiff[xSbIdx>>2][xSbIdx>>2] is less than etDiffThres or bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset2 +	(8-852)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift2 )

- Otherwise (etSumDiff[xSbIdx>>2][xSbIdx>>2] >= etDiffThres is equal to TRUE and  bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-853)

vy = Clip3( 1, nCbH, y )		(8-854)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-855)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-856)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-857)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-858)

- The variables temp[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift2 ) − ( predSamplesL1[ hx ][ vy ] >> shift2 )	(8-859)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift3	(8-860)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift3	(8-861)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij ( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-862)

sGy2 = ij(tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-863)

sGxGy = ij(tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-864)

sGxdI = ij( − tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-865)

sGydI = ij( − tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-866)

  - The horizontal and vertical motion offset of the current subblock are derived as:

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).