#### Rice parameter derivation process for dec_abs_level[ ]

Inputs to this process are the base level of remainder baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the syntax elements sig_coeff_flag[ x ][ y ] and the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ] − sig_coeff_flag[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ] − sig_coeff_flag[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] − sig_coeff_flag[ xC + 1 ][ yC + 1 ]	(9-5)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ] − sig_coeff_flag[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevelPass1 [ xC ][ yC + 2 ] − sig_coeff_flag[ xC ][ yC + 2 ]
}

locSumAbs -= baseLevel * 5
if( locSumAbs < 0 )
	locSumAbs = 0
if( locSumAbs > 31 )
	locSumAbs = 31

If baseLevel is greater than 0, given the variable locSumAbs, the Rice parameter cRiceParam are derived as specified in Table 9-5.

Otherwise, he variable s is set equal to Max( 0, QState – 1 ).

Given the variables locSumAbs and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ]0 are derived as specified in Table 9-5.

Table 9-5 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs and s

| s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
| 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
| 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
| 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
| 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

#### Binarization process for abs_remainder[ ]

Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the base level of remainder baseLevel, the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture).

Output of this process is the binarization of the syntax element.

-

The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[] as specified in clause 9.5.3.2 with the base level of remainder baseLevel, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = ( cRiceParam  = =  1  ?  6  :  7 )  <<  cRiceParam	(9-13)

The binarization of the syntax element abs_remainder[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of abs_remainder[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, abs_remainder[ n ] )		(9-14)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.5.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of abs_remainder[ n ], suffixVal, is derived as follows:

suffixVal = abs_remainder[ n ] − cMax		(9-15)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.5.3.5 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.

#### Binarization process for dec_abs_level[ ]

Input to this process is a request for a binarization of the syntax element dec_abs_level[ n ], the colour component cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[] as specified in clause 9.5.3.2 with the base level of remainder baseLevel equal to 0,  the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = ( cRiceParam  = =  1  ?  6  :  7 )  <<  cRiceParam	(9-16)

The binarization of dec_abs_level[ n ] is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of dec_abs_level[ n ], prefixVal, is derived as follows:

prefixVal = Min( cMax, dec_abs_level[ n ] )		(9-17)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.5.3.3 for prefixVal with the variables cMax and cRiceParam as inputs.

When the prefix bin string is equal to the bit string of length 4 with all bits equal to 1, the suffix bin string is present and it is derived as follows:

- The suffix value of dec_abs_level[ n ], suffixVal, is derived as follows:

suffixVal = dec_abs_level[ n ] − cMax		(9-18)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.5.3.5 for the binarization of suffixVal with the Exp-Golomb order k set equal to cRiceParam + 1.