| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0189 |
| --- | --- |

| Title: | Non-CE7: Unified last position coding for 32-point transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yinji Piao Kiho Choi | Email: | [yj1003.piao@samsung.com] [kiho14.choi@samsung.com] |
| Source: | Samsung Electronics Co.,Ltd |  |  |

_____________________________

# Abstract

This contribution suggests a unified last position coding for different type of transforms. Due to 16-point zero-out approach for 32-point MTS, last positions in range of 12 to 15 are represented by different binarizations in MTS, sub-block transform, DCT2 and TransformSkip mode. To simply the logic in parsing stage, a unified binarizations for last position within block size of 32 is suggested regardless of the selected transform type. Only 0.02% and 0.01% BD-rate loss is observed with the unified solution in AI and RA configurations, respectively.

# Introduction

In last meeting, high frequency coefficients out of top-left 16x16 region were recommended[1][2] to be zero-out for 32-piont MTS(DST7/DCT8) transform to reduce computational complexity of MTS. The binarizations for last position in range of 12 to 15 were modified accordingly to meet the valid block size of 16 instead of original 32. Similar to MTS, sub-block transform [3] adopted last meeting also considers zero-out for 32-point transform. However, since high frequency coefficients of 32-point DCT2 and 32-point transformSkip mode[4] are still kept without zero-out, the binarizations of last position in range of 12-15 are considered within block size of 32.

Table I shows binarizations for last position, prefix with truncated unary code and suffix with fixed binary code. Current last positions in range of 12 to 15 in 32-point MTS and sub-block transform are represented by 7-bin prefix, while 8-bin prefix for 32-point DCT2 and 32-point TransformSkip mode.

Table I. Binarization of last_sig_coeff_x, last_sig_coeff_y

| Magnitude of last position component | Truncated unary (context model) | Fixed binary (bypass) |  |
| --- | --- | --- | --- |
| 0 | 0 | - | 0 |
| 1 | 10 | - | 0 |
| 2 | 110 | - | 0 |
| 3 | 111(0) | - | 0 |
| 4-5 | 11110 | X | 0-1 |
| 6-7 | 11111(0) | X | 0-1 |
| 8-11 | 1111110 | XX | 0-3 |
| 12-15 | 1111111(0) | XX | 0-3 |
| 16-23 | 111111110 | XXX | 0-7 |
| 24-31 | 111111111 | XXX | 0-7 |

# Proposed method

To simplify the logic in parsing process of last position, a straightforward binarization using original transform size is suggested in this contribution by removing the dependency to transform type and transform size. Following conditions in s/w and WD can be fully removed by only considering 32-point zero-out for 64-point transform.

if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 )

log2TbWidth = 4

else

log2TbWidth = Min( log2TbWidth, 5 )

if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 )

log2TbHeight = 4

else

log2TbHeight = Min( log2TbHeight, 5 )

Parse last_sig_coeff

# Results

Proposed unification was implemented on top of VTM 4.0. Tests were conducted under VTM common test condition [5] as well as under low QP conditions described in [6]. More details can be found in accompanying excel sheets.

Table I. Test results at CTC and lowQP condition

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.00% | 101% | 103% | 0.01% | 0.03% | -0.01% | 103% | 106% |
| Class A2 | 0.01% | 0.15% | 0.03% | 103% | 106% |  |  |  |  |  |
| Class B | 0.02% | 0.04% | 0.04% | 101% | 107% | 0.00% | 0.02% | 0.00% | 101% | 101% |
| Class C | 0.01% | -0.04% | 0.01% | 100% | 97% | 0.00% | 0.00% | -0.01% | 101% | 103% |
| Class E | 0.04% | 0.02% | -0.05% | 100% | 99% | 0.01% | 0.01% | 0.00% | 105% | 100% |
| Overall | 0.02% | 0.03% | 0.01% | 101% | 102% |  |  |  |  |  |
| Class D | 0.01% | -0.09% | -0.04% | 100% | 99% | 0.00% | -0.01% | 0.03% | 111% | 105% |
| Class F | 0.00% | -0.01% | 0.00% | 101% | 103% | 0.01% | 0.02% | -0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 106% | 105% |  |  |  |  |  |
| Class A2 | -0.01% | 0.09% | 0.04% | 105% | 104% |  |  |  |  |  |
| Class B | 0.02% | 0.03% | -0.06% | 101% | 103% |  |  |  |  |  |
| Class C | 0.02% | -0.11% | -0.13% | 100% | 99% | 0.00% | -0.01% | 0.02% | 100% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.05% | 103% | 102% |  |  |  |  |  |
| Class D | 0.01% | -0.08% | 0.15% | 100% | 99% | 0.00% | 0.02% | -0.05% | 99% | 100% |
| Class F | -0.01% | 0.00% | 0.08% | 100% | 99% |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  | Over VTM-4.0_lowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.14% | 0.07% | 100% | 97% |  |  |  |  |  |
| Class C | -0.02% | 0.35% | 0.03% | 102% | 114% | -0.01% | 0.04% | 0.01% | 101% | 108% |
| Class E | -0.01% | 0.42% | 0.30% | 102% | 114% |  |  |  |  |  |
| Overall | -0.01% | 0.17% | 0.11% | 101% | 106% |  |  |  |  |  |
| Class D | 0.02% | 0.05% | -0.22% | 99% | 100% | -0.02% | -0.05% | -0.02% | 99% | 102% |
| Class F | -0.05% | -0.33% | 0.86% | 88% | 100% |  |  |  |  |  |

# Conclusions

This document proposes a unified solution for last position coding in different types of 32-point transforms. Since simplified method does not introduce performance loss, we suggest inclusion of proposed unification into next version of VTM s/w.

# Reference

- JVET, “CE6-related: 32 point MTS based on skipping high frequency coefficients”, JVET-M0297, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “CE6-related: A study of primary transforms”, JVET-M0046, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “CE6: Sub-block transform for inter blocks”, JVET-M0140, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “Non-CE8: Unified Transform Type Signaling and Residual Coding for Transform Skip”, JVET-M0464, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “JVET common test conditions and software reference configurations”, JVET-M1010, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

- JVET, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding”, JVET-M1027, 13th JVET Meeting, Marrakech, MA, 9-18 Jan., 2019

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).