| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0190 |
| --- | --- |

| Title: | Non-CE6: Modifications on sub-block transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung Dongcheol Kim Geonjung Ko Ju-Hyng (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a modified sub-block transform. In the current VVC specification, sub-block transform can be applied for inter-predicted blocks with cu_cbf equal to 1. It is proposed to allow sub-block transform only when the tu_cbf_luma is not equal to 0, and therefore infer tu_cbf_luma of a part of the partitioned TUs to be equal to 1 when SBT is applied to the current CU. Experimental results show that the proposed method can achieve x.xx% and x.xx% luma BD-rates for RA and LDB configurations, with xxx% and xxx% encoding runtimes, respectively.

Introduction

Sub-block transform [1] was adopted to VVC in the JVET-M meeting. For inter-predicted blocks with cu_cbf equal to 1 and cu_sbt_flag equal to 1, a CU is tiled into 2 sub-blocks and residuals of one sub-block is coded using implicit transform type combinations and residuals of the remaining sub-block is zeroed out. Up to 8 TU partitioning modes are supported and 3 flags are signaled to indicate the TU partitioning mode used for the current block. The tu_cbf_luma is signaled only for the non zeroed-out sub-block and it is inferred to be equal to 0 for the zeroed-out sub-block.

Proposed method

In the proposed method, SBT is allowed only when tu_cbf_luma of the non zeroed-out sub-block is 1 and therefore tu_cbf_luma of each sub-block can be inferred according to sub-TU index and position of the residual TU.

Table 1 shows the proposed spec text changes for the transform_unit syntax structure related to tu_cbf_luma coding. Compared to the current spec text [2], the proposed method is much simpler regarding the condition check for tu_cbf_luma.

Table 1: Proposed specification text for the transform_unit syntax structure

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| ⋯ |  |

When SBT is applied to the current CU, tu_cbf_luma is not signaled and can be inferred as described in Table 2 according to sub-TU index and a syntax element specifying position of the residual TU.

Table 2: Derivation of tu_cbf_luma according to subTuIndex and cu_sbt_pos_flag

| subTuIndex | cu_sbt_pos_flag | tu_cbf_luma |  |
| --- | --- | --- | --- |
|  |  | VTM 4 | Proposed method |
| 0 | 0 | signaled | inferred (1) |
| 0 | 1 | inferred (0) | inferred (0) |
| 1 | 0 | inferred (0) | inferred (0) |
| 1 | 1 | signaled | inferred (1) |

Experimental results

The proposed method is implemented on top of VTM-4.0 [3] and tested under the common test condition (CTC) [4]. Table 3 shows summarized experimental results of the proposed method compared to VTM-4.0 with RA and LDB configurations. Detailed results are available in the attached excel sheets.

Table 3: Coding performance of the proposed method

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class B | 0.01% | -0.07% | -0.04% | 100% | #DIV/0! |
| Class C | -0.01% | -0.02% | 0.05% | 100% | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -0.02% | -0.15% | -0.03% | 100% | #DIV/0! |
| Class F | -0.01% | 0.08% | 0.06% | 100% | #DIV/0! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | -0.07% | 0.40% | 0.20% | 100% | #DIV/0! |
| Class E | -0.02% | 0.03% | -0.03% | 100% | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -0.04% | -0.15% | -0.67% | 100% | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |

Conclusion

In this contribution, it is proposed to allow SBT for inter-predicted blocks only when tu_cbf_luma of a part of the partitioned TUs equal to 1 and therefore infer tu_cbf_luma to 1 when SBT is applied to the current CU. It is reportedly shown that the proposed method can achieve x.xx% and x.xx% luma BD-rates with xxx% and xxx% encoding runtimes for RA and LDB, respectively. Also, the proposed specification text related to the condition check for tu_cbf_luma parsing is much simpler than that of the current specification. It is recommended to adopt the proposed method to VVC.

Reference

- Y. Zhao, H. Gao, H. Yang, J. Chen, “CE6: Sub-block transform for inter blocks (CE6.4.1),” JVET-M0140, 13th meeting, Marrakech, MA, 9-18 Jan. 2019

- B.Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET Common Test Conditions and Software Reference. Configurations for SDR Video,” JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).