| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0192 |
| --- | --- |

| Title: | [AHG17/AHG12] Bitstream merging with variable initial Qp |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Eric Nassor Gerald Kergourlay Frederic Maze | Email: | nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr gerald.kergourlay@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes new syntax elements in the PPS that optionally associate a QP offset (tile_group_qp_offset[ i]) value to each tile group in the picture.

It is reported that these new syntax elements permit to fix QP initialization issues when combining tile groups from streams using different initial QP values.

It is also asserted that the stream combination is simplified and complexity reduced by avoiding the rewriting of the headers of the combined tile groups.

# Introduction

In OMAF, viewport dependent streaming may consist for example in assembling tile groups (TG) from different video streams of different qualities to emphasize a specific region with higher quality. It consists in taking tile groups from bitstream encoded with different initial QP values for instance.

This creates an issue if we want to merge TGs from different videos with different initial QP values. For example, in the figure below, the first video has a high quality and its initial QP is a low value for example 20 (init_qp_minus26 = -6). The second video with a low quality has high value for its initial QP for example 40 (init_qp_minus26 = 14).

We want to merge the TG 3 from first video with a high quality and the TG 4 from second video with a low quality in a new video. It is not possible to keep the same tile_group_qp_delta for the two TG because the QP value used to encode the blocks inside the two TGs are different. It is thus necessary to generate a new PPS with a new initial value (for example init_qp_minus26 = -6) and also to parse all the TG headers and modify the value tile_group_qp_delta of one of the TG inside each picture of the video (for example modifying the TG and changing tile_group_qp_delta to 20).  The first operation is not too complex because the non-VCL unit PPS are relatively small and not frequent and the headers are encoded with mostly fixed length values. The second operation is very complex and time consuming because it requires reading all the video bitstreams, decoding all the TG headers which contains many variable length fields and rewriting the complete bitstream. Moreover, we can remark that the value tile_group_qp_delta for one of the TG will have a value far different from 0 and thus will use a large number of bits in its variable length encoding.

It is thus desirable to have a better encoding for the TG QP to allow an efficient merge of several videos allowing easy resolution of QP conflicts without requiring modification of the VCL units (and in particular the TG headers).

# Proposal

In order to solve the problem described above when merging videos, we propose to associate a QP offset to each tile group inside the PPS that will be added to the initial QP value and to the tile group QP delta values.

The syntax of the PPS is modified as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_qp_offset _flag | u(1) |
| if (signalled_qp_offset _flag) { |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_qp_offset [ i ] | se(v) |
| } |  |

The semantics of the new syntax element is the following:

signalled_qp_offset_flag equal to 1 specifies the presence of tile_group_qp_offset[ i ] in the PPS. signalled_qp_offset_flag equal to 0 specifies the absence of the tile_group_qp_offset[ i ].

tile_group_qp_offset[ i ] when present, specifies the offset value that applies to the initial value of TileGroupQpY for the i-th tile group. The value of tile_group_qp_offset[ i ] shall be in the range of −QpBdOffsetY to +63, inclusive. When not present, the value of tile_group_qp_offset[ i ] is inferred to be equal 0 for each value of i in range of 0 to num_tile_groups_in_pic_minus1, inclusive.

The syntax elements of the tile group header remain identical but the semantics of tile_group_qp_delta are the following:

tile_group_qp_delta specifies the initial value of QpY to be used for the coding blocks in the tile group until modified by the value of CuQpDeltaVal in the coding unit layer. The initial value of the QpY quantization parameter for the tile group, TileGroupQpY , is derived as follows:

TileGroupQpY  = 26 + init_qp_minus26 + tile_group_qp_offset [ tileGroupQpIdx ] + tile_group_qp_delta

The value of TileGroupQpY shall be in the range of −QpBdOffsetY to +63, inclusive.

The variable tileGroupQpIdx which specifies the index of the tile group is derived as follows:

tileGroupQpIdx = 0
while( tile_group_address  !=  tile_group_id[ tileGroupQpIdx ] )
	tileGroupQpIdx ++

# Example

The figure above shows how this proposal solves the issues presented in previous example. The bitstream resulting from the combination of the tile groups from the two bitstreams contains a PPS with an initial QP value equal to 20. The PPS provides also QP offsets for the tile groups that correct the initial QP of the tile groups so that it corresponds to the initial QP of the originated stream. As a result, the QpY is correct without having to rewrite the tile_group_qp_delta of the tile group headers.

The proposal allows solving the issue of tile groups QP value without modification of the tile group NAL unit. Only the PPS is rewritten which reduces the complexity of bitstream merging and also simplifies some applications like for example viewport dependent streaming.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).