| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0196-v1 |
| --- | --- |

| Title: | CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposed to use a 6-tap interpolation filter is used for luma motion compensation of 4x4 CUs and the sub-blocks of affine CUs. Compared with VTM-4.0, the worst-case memory bandwidth of 4x4 CUs is reduced to 67%, and the number of multiplications and additions of affine CUs is reduced to 65% and 62%, respectively. Experimental results report 0.08% luma BD-rate change in RA and 0.11% luma BD-rate change in LD_B with negligible encoding and decoding time change.

# Proposed method

In this contribution, a 6-tap filter is used for 4x4 sized partitions, including 4x4 CUs and the sub-blocks of affine CUs. The filter taps are shown in Table 4. Note that the taps are truncated from that of 8-tap and does not require additional hardware for storage.

The first purpose is to reduce the memory bandwidth of the 4x4 uni-pred CU to be smaller than that of the 8x8 bi-pred CU. At the same time, the memory bandwidth of the 4x4 CU is reduced to 67%.

The second purpose is to reduce the worst computational complexity of the affine CU to be smaller than that of 8x8 bi-pred CU. By using the 6-tap filter for the 4x4 sub-blocks of the affine CU, the worst-case complexity of the affine CU is reduced to 65% (multiplications) and 62% (additions). Details are given in Table 2 and Table 3. Complexity of using 8x8 sub-block size is provided for comparison.

Table 1. Memory bandwidth using different filter lengths for 4x4 uni-pred CU

| filter size | memory bandwidth | ratio to 8-tap |
| --- | --- | --- |
| 8 | 121 | - |
| 6 | 81 | 67% |

| CU size (W×H) | Number of luma samples to be fetched per luma sample (W + N) × (H + N) / (W × H) |
| --- | --- |
| 8×8 bi-prediction 8 tap | 7.03 |
| 4×4 uni-prediction 8 tap | 7.56 |
| 4×4 uni-prediction 6 tap | 5.06 |

Table 2. Number of multiplications using different filter lengths for the affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 753,664 | 983,040 | 130% | 638,976 | 85% | 65% |
| 128 | 64 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 64 | 128 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 128 | 32 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 32 | 128 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 128 | 16 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 16 | 128 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 128 | 8 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 8 | 128 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 64 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 64 | 32 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 32 | 64 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 64 | 16 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 16 | 64 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 8 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 8 | 64 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 32 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 32 | 16 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 16 | 32 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 8 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 8 | 32 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 16 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 8 | 5,888 | 7,680 | 130% | 4,992 | 85% | 65% |
| 8 | 8 | 2,944 | 3,840 | 130% | 2,496 | 85% | 65% |

Table 3. Number of additions using different filter lengths for the affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 659,456 | 860,160 | 130% | 532,480 | 81% | 62% |
| 128 | 64 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 64 | 128 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 128 | 32 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 32 | 128 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 128 | 16 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 16 | 128 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 128 | 8 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 8 | 128 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 64 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 64 | 32 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 32 | 64 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 64 | 16 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 16 | 64 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 8 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 8 | 64 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 32 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 32 | 16 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 16 | 32 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 8 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 8 | 32 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 16 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 8 | 5,152 | 6,720 | 130% | 4,160 | 81% | 62% |
| 8 | 8 | 2,576 | 3,360 | 130% | 2,080 | 81% | 62% |

Table 4. The 6-tap interpolation filter

| MV | Tap filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 0 | 0 | 0 |
| 1/16 | 1 | -3 | 63 | 4 | -2 | 1 |
| 1/8 | 1 | -5 | 62 | 8 | -3 | 1 |
| 3/16 | 2 | -8 | 60 | 13 | -4 | 1 |
| 1/4 | 3 | -10 | 58 | 17 | -5 | 1 |
| 5/16 | 3 | -11 | 52 | 26 | -8 | 2 |
| 3/8 | 2 | -9 | 47 | 31 | -10 | 3 |
| 7/16 | 3 | -11 | 45 | 34 | -10 | 3 |
| 1/2 | 3 | -11 | 40 | 40 | -11 | 3 |
| 9/16 | 3 | -10 | 34 | 45 | -11 | 3 |
| 5/8 | 3 | -10 | 31 | 47 | -9 | 2 |
| 11/16 | 2 | -8 | 26 | 52 | -11 | 3 |
| 6/8 | 1 | -5 | 17 | 58 | -10 | 3 |
| 13/16 | 1 | -4 | 13 | 60 | -8 | 2 |
| 7/8 | 1 | -3 | 8 | 62 | -5 | 1 |
| 15/16 | 1 | -2 | 4 | 63 | -3 | 1 |

# Complexity analysis

The following table shows the complexity analysis suggested by M0862 and M1022.

| Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- |
| Translation model (4x4 CU): 8.19/18.88 affine mode: 6.59 / 13.19 | Same | Multiplication: 42.5 (reduced by 25%) Addition: 37.5 (reduced by 27%) | Same |

# Simulation results

Tests were carried out by using the VTM4.0 reference software. The CTC (common test conditions) defined in [1] was followed. Simulation results are shown in table 1.

Table 1: simulation results

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.03% | 100% | 99% |
| Class A2 | 0.04% | -0.07% | 0.02% | 101% | 99% |
| Class B | 0.05% | -0.08% | 0.01% | 101% | 100% |
| Class C | 0.18% | 0.03% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.05% | -0.01% | 101% | 100% |
| Class D | 0.25% | 0.09% | 0.10% | 101% | 103% |
| Class F | 0.06% | 0.04% | 0.02% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.08% | -0.27% | 101% | 100% |
| Class C | 0.14% | 0.23% | 0.17% | 100% | 99% |
| Class E | 0.17% | 0.21% | -0.36% | 101% | 100% |
| Overall | 0.11% | 0.16% | -0.15% | 101% | 100% |
| Class D | 0.39% | 0.16% | -0.54% | 101% | 100% |
| Class F | 0.03% | -0.48% | -0.11% | 102% | 100% |

# Conclusions

In this contribution, a 6-tap filter is used for luma motion compensation of 4x4 CUs and the sub-blocks of affine CUs. The worst-case memory bandwidth of 4x4 sized CUs is reduced to 67%. The number of multiplications and additions of the affine mode is reduced to 65% and 62%, respectively. Experimental results show 0.08% luma BD-rate change in RA and 0.11% luma BD-rate change in LD_B with negligible encoding and decoding time change.

It is suggested to adopt the proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).