| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0196-v1 |
| --- | --- |

| Title: | CE2: Using the shorter-tap filter for 4x4 sized partitions (Test 2.4.6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposed to use a 6-tap interpolation filter is used for luma motion compensation of 4x4 CUs and the sub-blocks of affine CUs. Compared with VTM-4.0, the worst-case memory bandwidth of 4x4 CUs is reduced to 67%, and the number of multiplications and additions of affine CUs is reduced to 65% and 62%, respectively. Experimental results report 0.08% luma BD-rate change in RA and 0.11% luma BD-rate change in LD_B with negligible encoding and decoding time change.

# Proposed method

In this contribution, a 6-tap filter is used for 4x4 sized partitions, including 4x4 CUs and the sub-blocks of affine CUs. The filter taps are shown in Table 4.

The first purpose is to reduce the memory bandwidth of the 4x4 uni-pred CU to be smaller than that of the 8x8 bi-pred CU. At the same time, the memory bandwidth of the 4x4 CU is reduced to 67%.

The second purpose is to reduce the worst computational complexity of the affine CU to be smaller than that of 8x8 bi-pred CU. By using the 6-tap filter for the 4x4 sub-blocks of the affine CU, the worst-case complexity of the affine CU is reduced to 65% (multiplications) and 62% (additions). Details are given in Table 2 and Table 3. Complexity of using 8x8 sub-block size is provided for comparison.

Table 1. Memory bandwidth using different filter lengths for 4x4 uni-pred CU

| filter size | memory bandwidth | ratio to 8-tap |
| --- | --- | --- |
| 8 | 121 | - |
| 6 | 81 | 67% |

| CU size (W×H) | Number of luma samples to be fetched per luma sample (W + N) × (H + N) / (W × H) |
| --- | --- |
| 8×8 bi-prediction 8 tap | 7.03 |
| 4×4 uni-prediction 8 tap | 7.56 |
| 4×4 uni-prediction 6 tap | 5.06 |

Table 2. Number of multiplications using different filter lengths for the affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 753,664 | 983,040 | 130% | 638,976 | 85% | 65% |
| 128 | 64 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 64 | 128 | 376,832 | 491,520 | 130% | 319,488 | 85% | 65% |
| 128 | 32 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 32 | 128 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 128 | 16 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 16 | 128 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 128 | 8 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 8 | 128 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 64 | 188,416 | 245,760 | 130% | 159,744 | 85% | 65% |
| 64 | 32 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 32 | 64 | 94,208 | 122,880 | 130% | 79,872 | 85% | 65% |
| 64 | 16 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 16 | 64 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 64 | 8 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 8 | 64 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 32 | 47,104 | 61,440 | 130% | 39,936 | 85% | 65% |
| 32 | 16 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 16 | 32 | 23,552 | 30,720 | 130% | 19,968 | 85% | 65% |
| 32 | 8 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 8 | 32 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 16 | 11,776 | 15,360 | 130% | 9,984 | 85% | 65% |
| 16 | 8 | 5,888 | 7,680 | 130% | 4,992 | 85% | 65% |
| 8 | 8 | 2,944 | 3,840 | 130% | 2,496 | 85% | 65% |

Table 3. Number of additions using different filter lengths for the affine CU

| W | H | (1) use 8x8 sub-blocks (8 tap) | (2) use 4x4 sub-blocks (8 tap) | (2)/(1) | (3) use 4x4 sub-blocks (6 tap) | (3)/(1) | (3)/(2) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 128 | 659,456 | 860,160 | 130% | 532,480 | 81% | 62% |
| 128 | 64 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 64 | 128 | 329,728 | 430,080 | 130% | 266,240 | 81% | 62% |
| 128 | 32 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 32 | 128 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 128 | 16 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 16 | 128 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 128 | 8 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 8 | 128 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 64 | 164,864 | 215,040 | 130% | 133,120 | 81% | 62% |
| 64 | 32 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 32 | 64 | 82,432 | 107,520 | 130% | 66,560 | 81% | 62% |
| 64 | 16 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 16 | 64 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 64 | 8 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 8 | 64 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 32 | 41,216 | 53,760 | 130% | 33,280 | 81% | 62% |
| 32 | 16 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 16 | 32 | 20,608 | 26,880 | 130% | 16,640 | 81% | 62% |
| 32 | 8 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 8 | 32 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 16 | 10,304 | 13,440 | 130% | 8,320 | 81% | 62% |
| 16 | 8 | 5,152 | 6,720 | 130% | 4,160 | 81% | 62% |
| 8 | 8 | 2,576 | 3,360 | 130% | 2,080 | 81% | 62% |

Table 4. The 6-tap interpolation filter

| MV | Tap filter |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 64 | 0 | 0 | 0 |
| 1/16 | 1 | -3 | 63 | 4 | -2 | 1 |
| 1/8 | 1 | -5 | 62 | 8 | -3 | 1 |
| 3/16 | 2 | -8 | 60 | 13 | -4 | 1 |
| 1/4 | 3 | -10 | 58 | 17 | -5 | 1 |
| 5/16 | 3 | -11 | 52 | 26 | -8 | 2 |
| 3/8 | 2 | -9 | 47 | 31 | -10 | 3 |
| 7/16 | 3 | -11 | 45 | 34 | -10 | 3 |
| 1/2 | 3 | -11 | 40 | 40 | -11 | 3 |
| 9/16 | 3 | -10 | 34 | 45 | -11 | 3 |
| 5/8 | 3 | -10 | 31 | 47 | -9 | 2 |
| 11/16 | 2 | -8 | 26 | 52 | -11 | 3 |
| 6/8 | 1 | -5 | 17 | 58 | -10 | 3 |
| 13/16 | 1 | -4 | 13 | 60 | -8 | 2 |
| 7/8 | 1 | -3 | 8 | 62 | -5 | 1 |
| 15/16 | 1 | -2 | 4 | 63 | -3 | 1 |

# Complexity analysis

The following table shows the complexity analysis suggested by M0862 and M1022.

| Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- |
| Translation model (4x4 CU): 8.19/18.88 affine mode: 6.59 / 13.19 | Same | Multiplication: 42.5 (reduced by 25%) Addition: 37.5 (reduced by 27%) | Additional one filter tap table (96 bytes) |

# Simulation results

Tests were carried out by using the VTM4.0 reference software. The CTC (common test conditions) defined in [1] was followed. Simulation results are shown in table 1.

Table 1: simulation results

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.03% | 100% | 99% |
| Class A2 | 0.04% | -0.07% | 0.02% | 101% | 99% |
| Class B | 0.05% | -0.08% | 0.01% | 101% | 100% |
| Class C | 0.18% | 0.03% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.05% | -0.01% | 101% | 100% |
| Class D | 0.25% | 0.09% | 0.10% | 101% | 103% |
| Class F | 0.06% | 0.04% | 0.02% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.08% | -0.27% | 101% | 100% |
| Class C | 0.14% | 0.23% | 0.17% | 100% | 99% |
| Class E | 0.17% | 0.21% | -0.36% | 101% | 100% |
| Overall | 0.11% | 0.16% | -0.15% | 101% | 100% |
| Class D | 0.39% | 0.16% | -0.54% | 101% | 100% |
| Class F | 0.03% | -0.48% | -0.11% | 102% | 100% |

# Conclusions

In this contribution, a 6-tap filter is used for luma motion compensation of 4x4 CUs and the sub-blocks of affine CUs. The worst-case memory bandwidth of 4x4 sized CUs is reduced to 67%. The number of multiplications and additions of the affine mode is reduced to 65% and 62%, respectively. Experimental results show 0.08% luma BD-rate change in RA and 0.11% luma BD-rate change in LD_B with negligible encoding and decoding time change.

It is suggested to adopt the proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text

##### 8.5.7.3.1General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-811)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-812)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-813)

xFracL = refMvLX[ 0 ] & 15		(8-814)

yFracL = refMvLX[ 1 ] & 15		(8-815)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.7.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - bdofFlag is equal to TRUE.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + .

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.7.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, and padVal, and ( xSb, ySb ) as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-816)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-817)

xFracC = mvLX[ 0 ] & 31		(8-818)

yFracC = mvLX[ 1 ] & 31		(8-819)

  - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

  - The list padVal[ dir ] is derived as follows for dir = 0..1:

    - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xC : yC)	(8-820)

    - If disp is less than 0, padVal[ dir ] is set equal to disp.

    - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1).

    - Otherwise, padVal[ dir ] is set equal to 0.

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.7.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), refPicLX, and padVal as inputs.

##### 8.5.7.3.2Luma sample 8-tap interpolation filtering process

Inputs to this process are:

-  a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

–	a list padVal[ dir ] with dir = 0,1 specifying reference sample padding direction and amount.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9 and Table 8-10. If inter_affine_flag[ xSb ][ ySb ] is false, Table 8-9 is used, otherwise, Table 8-10 is used.

The luma interpolation filter coefficients fPadL[ 0 ] are set equal to fL[ xFracL ] and fPadL[ 1 ] are set equal to fL[ yFracL ] and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ xFracL ][ 2 ] + fPadL[ xFracL ][ 1 ] + fPadL[ xFracL ][ 0 ]	(8-821)

fPadL[ dir ][ 0 ] = 0		(8-822)

fPadL[ dir ][ 1 ] = 0		(8-823)

–	Otherwise, if padVal[ dir ] is equal to − 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-824)

fPadL[ dir ][ 0 ] = 0		(8-825)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 6 ] = fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ]		(8-826)

fPadL[ dir ][ 7 ] = 0		(8-827)

–	Otherwise, if padVal[ dir ] is equal to 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-828)

fPadL[ dir ][ 6 ] = 0		(8-829)

fPadL[ dir ][ 7 ] = 0		(8-830)

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

xInti = sps_ref_wraparound_enabled_flag ?
	ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(8-831) 
	Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-832)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-833)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-834)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-835)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-836)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * temp[ i ]   >>  shift2	(8-837)

Table 8-9 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

Table 8-10 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | 0 | 1 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | 0 | 2 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | 0 | 3 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | 0 | 3 | −11 | 52 | 26 | −8 | 2 | 0 |
| 6 | 0 | 2 | −9 | 47 | 31 | −10 | 3 | 0 |
| 7 | 0 | 3 | −11 | 45 | 34 | −10 | 3 | 0 |
| 8 | 0 | 3 | −11 | 40 | 40 | −11 | 3 | 0 |
| 9 | 0 | 3 | −10 | 34 | 45 | −11 | 3 | 0 |
| 10 | 0 | 3 | −10 | 31 | 47 | −9 | 2 | 0 |
| 11 | 0 | 2 | −8 | 26 | 52 | −11 | 3 | 0 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 3 | 0 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 2 | 0 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 1 | 0 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |