| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0198-v3 |
| --- | --- |

| Title: | CE9: Simplification of BDOF’s optical flow parameter derivation (Test 9.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a further study of M0316. It is proposed to simplify the parameter derivation of BDOF by removing all multiplication operations. Experimental results show 0.10% luma BD-rate change in RA.

# Problem statement

BDOF is a high complexity tool. Effort in JVET-L0256 has reduced the worst-case complexity of BDOF by reducing number of multiplication and addition operations. However, in VTM-4.0, BDOF still takes overall up to 13% number of multiplications. Additionally, from pipeline point of view, BDOF increases complexity of reconstruction stage by 253%. And multiplication used for BDOF is of 32-bit instead of 16-bit which is used for interpolation process. Table 1 gives an illustration of current BDOF’s complexity regarding multiplication operations.

Table 1. Number of multiplications of BDOF in VTM-4.0

| W | H | prediction stage | reconstruction stage |  |  | increase of mul in recon stage due to BDOF | overall increase of mul due to BDOF |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | additional complexity by BDOF |  |  |  |  |
|  |  | 8-tap interpolation (16 bit mul) | correlation (32bit mul) | motion refinement | weighted prediction |  |  |
| 128 | 128 | 538,624 | 81,920 | 1,024 | 32,768 | 253% | 15% |
| 128 | 64 | 276,480 | 40,960 | 512 | 16,384 | 253% | 14% |
| 64 | 128 | 269,312 | 40,960 | 512 | 16,384 | 253% | 15% |
| 128 | 32 | 145,408 | 20,480 | 256 | 8,192 | 253% | 14% |
| 32 | 128 | 134,656 | 20,480 | 256 | 8,192 | 253% | 15% |
| 128 | 16 | 79,872 | 10,240 | 128 | 4,096 | 253% | 12% |
| 16 | 128 | 67,328 | 10,240 | 128 | 4,096 | 253% | 15% |
| 128 | 8 | 47,104 | 5,120 | 64 | 2,048 | 253% | 11% |
| 8 | 128 | 33,664 | 5,120 | 64 | 2,048 | 253% | 15% |
| 64 | 64 | 138,240 | 20,480 | 256 | 8,192 | 253% | 14% |
| 64 | 32 | 72,704 | 10,240 | 128 | 4,096 | 253% | 14% |
| 32 | 64 | 69,120 | 10,240 | 128 | 4,096 | 253% | 14% |
| 64 | 16 | 39,936 | 5,120 | 64 | 2,048 | 253% | 12% |
| 16 | 64 | 34,560 | 5,120 | 64 | 2,048 | 253% | 14% |
| 64 | 8 | 23,552 | 2,560 | 32 | 1,024 | 253% | 11% |
| 8 | 64 | 17,280 | 2,560 | 32 | 1,024 | 253% | 14% |
| 32 | 32 | 36,352 | 5,120 | 64 | 2,048 | 253% | 14% |
| 32 | 16 | 19,968 | 2,560 | 32 | 1,024 | 253% | 12% |
| 16 | 32 | 18,176 | 2,560 | 32 | 1,024 | 253% | 14% |
| 32 | 8 | 11,776 | 1,280 | 16 | 512 | 253% | 11% |
| 8 | 32 | 9,088 | 1,280 | 16 | 512 | 253% | 14% |
| 16 | 16 | 9,984 | 1,280 | 16 | 512 | 253% | 12% |
| 16 | 8 | 5,888 | 640 | 8 | 256 | 253% | 11% |
| 8 | 8 | 2,944 | 320 | 4 | 128 | 253% | 11% |

# Proposed complexity reduction

This proposal simplifies the parameter derivation of BDOF by assuming there is no vertical optical flow when deriving Vx, and there is no horizontal optical flow when deriving Vy. The algorithm is described as below:

s1=[i,j]∊Ω absIx1+Ix0.

s2=[i,j]∊Ω absIy1+Iy0.

s3=[i,j]∊Ω signIx1+Ix0*(I1-I0).

s4=[i,j]∊Ω signIy1+Iy0*(I1-I0).

vx =-s3s1.

vy =-s4s2.

If S1 > th & s2 < th

Horizontal flow = Vx, Vertical flow = 0

If S2 > th & s2 < th

Horizontal flow = 0, Veritcal flow = Vy,

Else

Horizontal flow = Vx, Veritcal flow = Vy,

Value of th is set to 112.

By comparing the proposed BDOF with the current BDOF in VTM-4.0, all multiplication operations are removed. Additionally, 13% addition operations are also removed. Table 2 gives an illustration of complexity reduction of the proposed method.

Besides, the 1st stage early termination (CU level) is disabled to provide a fair comparison as described in M1029: “The maximum size where early termination is used should be blocks of size 256 luma samples.”

Table 2. Number of multiplications of the proposed BDOF in VTM-4.0

| W | H | prediction stage | reconstruction stage |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | additional complexity by BDOF |  |  |
|  |  | 8-tap interpolation (16 bit mul) | correlation (32bit mul) | motion refinement | weighted prediction |
| 128 | 128 | 538,624 | 0 | 0 | 32,768 |
| 128 | 64 | 276,480 | 0 | 0 | 16,384 |
| 64 | 128 | 269,312 | 0 | 0 | 16,384 |
| 128 | 32 | 145,408 | 0 | 0 | 8,192 |
| 32 | 128 | 134,656 | 0 | 0 | 8,192 |
| 128 | 16 | 79,872 | 0 | 0 | 4,096 |
| 16 | 128 | 67,328 | 0 | 0 | 4,096 |
| 128 | 8 | 47,104 | 0 | 0 | 2,048 |
| 8 | 128 | 33,664 | 0 | 0 | 2,048 |
| 64 | 64 | 138,240 | 0 | 0 | 8,192 |
| 64 | 32 | 72,704 | 0 | 0 | 4,096 |
| 32 | 64 | 69,120 | 0 | 0 | 4,096 |
| 64 | 16 | 39,936 | 0 | 0 | 2,048 |
| 16 | 64 | 34,560 | 0 | 0 | 2,048 |
| 64 | 8 | 23,552 | 0 | 0 | 1,024 |
| 8 | 64 | 17,280 | 0 | 0 | 1,024 |
| 32 | 32 | 36,352 | 0 | 0 | 2,048 |
| 32 | 16 | 19,968 | 0 | 0 | 1,024 |
| 16 | 32 | 18,176 | 0 | 0 | 1,024 |
| 32 | 8 | 11,776 | 0 | 0 | 512 |
| 8 | 32 | 9,088 | 0 | 0 | 512 |
| 16 | 16 | 9,984 | 0 | 0 | 512 |
| 16 | 8 | 5,888 | 0 | 0 | 256 |
| 8 | 8 | 2,944 | 0 | 0 | 128 |

# Experimental results

Tests were carried out by using the VTM4.0 reference software. The CTC (common test conditions) defined in [1] was followed. Simulation results are shown in table 1.

Table 1: simulation results

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.03% | 0.07% | 99% | 101% |
| Class A2 | 0.11% | 0.06% | 0.02% | 99% | 101% |
| Class B | 0.12% | 0.05% | 0.04% | 100% | 100% |
| Class C | 0.10% | 0.07% | 0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.04% | 0.06% | 99% | 100% |
| Class D | 0.21% | 0.18% | 0.15% | 100% | 99% |
| Class F | 0.02% | 0.06% | 0.02% | 99% | 103% |

# Conclusions

In this contribution, a simplified BDOF is proposed to reduce the number of multiplications and additions. All multiplication operations are removed and 13% addition operations are reduced. 0.10% BD rate change is observed for RA configurations. Encoding and decoding time impact is negligible. It is suggested to adopt the proposed method to the next VVC working draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Syntax Structure

#### Bidirectional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

- the bidirectional optical flow utilization flags bdofUtilizationFlag[ xIdx ][ yIdx ] with xIdx = 0..( nCbW >> 2 ) − 1, yIdx = 0..( nCbH >> 2 ) − 1.

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

The variable bitDepth is set equal to BitDepthY.

The variable shift2 is set equal to Max( 3, 15 − bitDepth ) and the variable offset2 is set equal to 1  <<  ( shift2 − 1 ).

The variable mvRefineThres is set equal to 1  <<  ( 13 − bitDepth ).

For xIdx = 0..( nCbW >> 2 ) − 1 and yIdx = 0..( nCbH >> 2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx << 2) + 1 and ySb is set equal to ( yIdx << 2 ) + 1.

- If bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to FALSE, for x = xSb − 1..xSb + 2, y = ySb − 1.. ySb + 2, the prediction sample values of the current subblock are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset2 +	(8-694)
	predSamplesL1[ x + 1 ][ y + 1 ] )  >>  shift2 )

- Otherwise (bdofUtilizationFlag[ xSbIdx ][ yIdx ] is equal to TRUE), the prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(8-695)

vy = Clip3( 1, nCbH, y )		(8-696)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> 4	(8-697)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> 4	(8-698)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> 4	(8-699)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> 4	(8-700)

- The variables temp[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> 6 ) − ( predSamplesL1[ hx ][ vy ] >> 6 )	(8-701)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> 3	(8-702)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> 3	(8-703)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij ( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-704)

sGy2 = ij(tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-705)

sGxGy = ij(tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j  −1..4	(8-706)

sGxdI = ij( − tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-707)

sGydI = ij( − tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(8-708)

s1 = ijAbs(tempX[xSb+x][ySb+y]) with x, y = −1..4

s2 = ijAbs(tempY[xSb+x][ySb+y]) with x, y = −1..4

s3 = ij(Sign(tempX[xSb+x][ySb+y]) *temp[xSb+x][ySb+y]) with x, y = −1..4

s4 = ijSign(tempY[xSb+x][ySb+y]) *temp[xSb+x][ySb+y]) with x, y = −1..4

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	(8-709)
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	(8-710)
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGx2 ) ) )  :  0

- if s1 is greater than 112 and s2 is smaller than 112, the value of horizontal and vertical motion refinement of current subblock are derived as:

vx = s1 > 0 ? Clip3(-mvRefineThres, mvRefineThres, -s3>>Floor(Log2(s1))) : 0

vy = 0

- Else if  s1 is smaller than 112 and s2 is greater than 112, the value of horizontal and vertical motion refinement of current subblock are derived as:

- ,

vx =  0

vy = s1 > 0 ? Clip3(-mvRefineThres, mvRefineThres, -s4>>Floor(Log2(s2))) : 0

- Else the value of horizontal and vertical motion refinement of current subblock are derived as:

- ,

vx = s1 > 0 ? Clip3(-mvRefineThres, mvRefineThres, -s3>>Floor(Log2(s1))) : 0

vy = s1 > 0 ? Clip3(-mvRefineThres, mvRefineThres, -s4>>Floor(Log2(s2))) : 0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = Round( ( vx * ( gradientHL1[ x + 1 ][ y + 1 ] − gradientHL0[ x + 1 ][ y + 1 ] ) ) >> 1 )	(8-711)
	+ Round( ( vy * (gradientVL1[ x + 1 ][ y + 1 ] − gradientVL0[ x + 1 ][ y + 1 ] ) ) >> 1 )

[Ed. (JC): Round() operation is defined for float input. The Round() operation seems redundant here since the input is an integer value. To be confirmed by the proponent]

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset2 +	(8-712)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift2 )