| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0202-v1 |
| --- | --- |

| Title: | AHG16/Non-CE8: Report on conformance check failures of IBC block vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jingya Li Che-Wei Kuo Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

It is observed that current VTM4.0’s conformance check fails for intra block copy (IBC) chroma block vectors, wherein illegal chroma block vector could be judged to be legal by current conformance check conditions.

# Introduction

## Chroma conformance checks failure:

Current VTM4 conformance check for chroma block vector is as below:

- It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, and the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, and the output shall be equal to TRUE.

- One or both of the following conditions shall be true:

- ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0.

- ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

This conformance check allows invalid referencing due to separate tree. Example is given in Figure 1, wherein Luma CTB size = 128x128 and chroma CTB size = 64x64.

Figure 1: example of chroma conformance check failure

In this example, the luma IBC block size is 64x64, the chroma IBC block size is 32x16, and the top half of the 32x32 chroma block may be coded in a non-IBC mode. According to the current IBC specification, the luma IBC vector is valid, because it belongs to the first CU being decoded for the current 64x64 VDPU and the luma reference area (though exceeding the preceding three 64x64 VDPUs) is still treated as available. For chroma, it is already the second chroma CU of the current 64x64 VDPU being decoded, the chroma vector, which is inherited from the luma IBC block, should be treated as invalid because the chroma reference block is located in a VPDU which exceeds more than 3 preceding VPDUs. However, in the current IBC design, the chroma vector in this case is still treated as valid.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).