| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0203-v1 |
| --- | --- |

| Title: | Non-CE4: Modification of the temporal merging candidate for the inter merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geon-Won Lee Geun-Bae Kim Sang-Guk Cha Da-Yoon Nam Jong-Ki Han | Tel: Email: | +82-2-3408-3739 geonwonlee@sju.ac.kr gbkim@sju.ac.kr sgsept29@sju.ac.kr ndy9709@sju.ac.kr hjk@sejong.edu |
| Source: | Sejong University |  |  |

_____________________________

- Abstract

In this contribution, when the merge mode is used, if the reference picture of the collocated block is in the reference picture list of the current picture, then it is considered as target picture. If the reference picture list of the current picture does not include the reference picture of the collocated block, then the picture related to the index 0 in the reference picture list is set to the target picture as in VTM 4.0. The target picture is used to make the temporal merge candidate, where the collocated MV is scaled based on the difference between POCs of the target picture and the reference picture of the collocated block.

When the proposed method is applied, simulation results reportedly show that the average 0.02%, -0.18% and -0.22% BD-rate changes are achieved on top of VTM 4.0 for RA, LDB and LDP configurations, respectively, under the common test conditions. There is no change in the encoding and decoding running times.

Introduction

In VTM 4.0, when the merge mode is used, the motion vector of the collocated block is scaled based on the difference between POCs of target picture and the reference picture of the collocated block, where the target picture is indicated by index 0 of the reference picture list. The scaled motion vector is used as the temporal merge candidate for current coding block.

In this contribution, when the reference picture of the collocated block is in the reference picture list of the current picture, it is considered as target picture, instead of the picture indicated by the index 0. If the reference picture list of the current picture does not include the reference picture of the collocated block, then the picture related to the index 0 in the list is set to the target picture.

Proposed method

In this contribution, a modified method to make the temporal merge candidate is proposed. It is used to construct the temporal merge candidates in the merge mode and the merge triangle mode, except for the subblock merge mode. In VTM 4.0, the temporal merge candidate is derived by scaling the collocated MV for the target picture which is indexed by index 0 in the reference picture list. In the proposed method, when the difference between the POCs of the current picture and the collocated picture is 1, the target picture is set to the reference picture of collocated MV if it is in the reference picture list.

Figure 1 shows an example of setting the target picture to make the temporal merge candidate in the proposed method, where the collocated block refers to a picture whose POC is 15. In VTM 4.0, the collocated MV is scaled by considering the picture (POC 9) as a target picture, because it is indexed by index 0 in the reference picture list. In the proposed method, the target picture is set to a picture (POC 15) which is indexed by index 2, because it is equal to the reference picture of the collocated block. If the reference picture of the collocated block is not in the reference picture list, then the picture indicated by index 0 is set to the target picture as in VTM 4.0.

Figure 1. An example of selecting the target picture and scaling the collocated MV to make the temporal merge candidate.

Experimental results

The proposed method is implemented on top of VTM-4.0[1] software. The simulations are performed on Ubuntu using RA, LDB and LDP configurations, under the common test condition described in JVET-M1010[2].

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 100% | 99% |
| Class A2 | 0.02% | 0.01% | 0.03% | 100% | 100% |
| Class B | 0.02% | 0.02% | 0.00% | 100% | 100% |
| Class C | 0.02% | 0.01% | 0.01% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.01% | 100% | 100% |
| Class D | 0.04% | 0.03% | 0.04% | 99% | 99% |
| Class F | 0.01% | 0.02% | 0.00% | 99% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.37% | -0.57% | 100% | 100% |
| Class C | -0.30% | -0.45% | -0.62% | 100% | 100% |
| Class E | -0.09% | -0.66% | -0.72% | 100% | 100% |
| Overall | -0.18% | -0.47% | -0.62% | 100% | 100% |
| Class D | -0.41% | -0.58% | -1.19% | 102% | 99% |
| Class F | -0.17% | -0.27% | -0.64% | 100% | 100% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.44% | -0.28% | 100% | 99% |
| Class C | -0.35% | -0.55% | -0.89% | 100% | 100% |
| Class E | -0.13% | -0.33% | -0.70% | 100% | 99% |
| Overall | -0.22% | -0.45% | -0.59% | 100% | 99% |
| Class D | -0.50% | -0.79% | -0.40% | 99% | 98% |
| Class F | 0.00% | -0.43% | -0.26% | 100% | 100% |

Conclusion

In this contribution, the method to set the target picture in making the temporal merge candidate is proposed. As we observe from the simulation results, the average 0.02%, -0.18% and -0.22% BD-rate changes are achieved on top of VTM 4.0 for RA, LDB and LDP configurations, respectively, under the common test conditions. There is no change in the encoding and decoding running times.

References

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-M1010, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).