| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0212 |
| --- | --- |

| Title: | CE4-1.5: Remove TMVP merge candidate for the specified blocksizes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Core Experiment (CE) 4.1.5 Test (a)~(c) for removing unnecessary TMVP merge candidates.

- Test (a): Remove TMVP merge candidate for the block size with samples <= 32:

-0.01% and -0.01% for RA and LDB, respectively

- Test (b): Remove TMVP merge candidate for the block size with samples <= 64:

-0.03% and 0.01% for RA and LDB, respectively

- Test (c): Remove TMVP merge candidate for the block size with samples of 4xN / Nx4 and 8x8:

-0.03% and 0.04% for RA and LDB, respectively

Introduction

At the 11th JVET meeting, the granularity of temporal MV compression is set to 8x8 and at the 12th JVET meeting, sub-block size of ATMVP have been fixed to 8x8. In specific cases, there are some redundancy operations between TMVP and ATMVP. Besides, TMVP with smaller block size becomes unnecessary in some cases due to the storage granularity.

In merge mode, TMVP derives one temporal candidate MVP from two temporal co-located blocks, which are the blocks locate at the bottom right and at the center of the current block, following a specified order. With the granularity of temporal MV storage has been set to 8x8 and the adoption of 8x8 ATMVP, it exists some unnecessary operations in TMVP deriving process:

- In affine merge mode, ATMVP derives multiple motion for 8x8 sub-blocks of one CU based on the motion information of the collocated blocks from temporal reference picture. Both ATMVP and TMVP derive motion information from temporal domain. When the size of current CU is 8x8, both of these two tools will derive one MVP for current CU. In principal, the motion vectors derivation process for these two tools have redundancy at 8x8 coding block.

- Since the granularity of temporal MV storage has been set to 8x8, TMVP with smaller block size might become unnecessary in some cases, e.g. the width or height of CU is 4, when some adjacent CUs derive MVP from the same MV storage units, which will make these kind of partition meaning less.

Proposed Method

In order to remove the unnecessary process of deriving TMVP in merge mode, three cases have been tested over the construction of current merge candidate list.

Remove TMVP merge candidate for block size with samples ≤ 32 (Test a)

In this test, 4x4/4x8/8x4 TMVP is removed from merge list. At the 13th JVET meeting, Share merge list has been adopted to the current VVC design which makes some small blocks share the same merge list. In the current design, the threshold of Share merge list is set to 32, which means part of blocks with number of samples smaller than 32 will be set as sharing blocks for parallel processing. In this test, it remove TMVP merge candidate for all the blocks with samples <=32, which are beneficial for hardware design since the timing for processing smaller blocks can be reduced and make the construction process of merge list even simpler.

Remove TMVP merge candidate for block size with samples≤64 (Test b)

In this test, 4x4/4x8/8x4/4x16/16x4/8x8 TMVP is removed from merge list. Since the granularity of temporal MV storage has been set to 8x8, TMVP with smaller block size might become unnecessary in some cases, e.g. the width or height of CU is 4, when some adjacent CUs derive MVP from the same MV storage units, which will make these kind of partition meaning less.

Figure. 1 Illustration of unnecessary cases for TMVP

Remove TMVP merge candidate for block size with samples of 4xN / Nx4 and 8x8 (Test c)

In this test, 4x4/4x8/8x4/4x16/16x4/4x32/32x4/8x8 TMVP is removed from merge list. This test remove all of the 4xN and Nx4 TMVP process in merge list which are beneficial for hardware design by removing TMVP process of more types of blocks. For example, for blocks which are slender, such as 4x32 and 32x4, it may introduce difficulty in parallel processing.

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the tests on top of the VTM-4.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets. Due to the unstable performance of the computer resource, the coding time is not reliable.

Coding performance and complexity of Test (a)

Table 1. Test results of CE 4-1.5.a

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | 0.02% | 101% | 98% |
| Class A2 | -0.03% | -0.04% | -0.04% | 101% | 99% |
| Class B | -0.02% | -0.04% | -0.05% | 109% | 98% |
| Class C | 0.01% | -0.08% | -0.15% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | -0.06% | 104% | 98% |
| Class D | 0.03% | 0.02% | -0.01% | 102% | 97% |
| Class F | -0.02% | 0.02% | -0.04% | 85% | 97% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.02% | 0.03% | 80% | 98% |
| Class C | 0.00% | 0.26% | 0.09% | 82% | 101% |
| Class E | 0.00% | 0.10% | -0.05% | 83% | 101% |
| Overall | -0.01% | 0.11% | 0.03% | 81% | 100% |
| Class D | 0.05% | 0.08% | -0.40% | 83% | 101% |
| Class F | -0.07% | 0.10% | 0.38% | 59% | 97% |

Coding performance and complexity of Test (b)

Table 2. Test results of CE 4-1.5.b

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.04% | 101% | 100% |
| Class A2 | -0.07% | -0.07% | -0.05% | 100% | 99% |
| Class B | -0.04% | -0.12% | -0.09% | 99% | 100% |
| Class C | -0.01% | 0.00% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.07% | -0.06% | 100% | 100% |
| Class D | 0.03% | -0.08% | -0.13% | 100% | 98% |
| Class F | -0.02% | -0.01% | -0.06% | 84% | 98% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.16% | -0.12% | 100% | 67% |
| Class C | -0.01% | 0.21% | -0.02% | 101% | 64% |
| Class E | 0.04% | 0.38% | -0.08% | 100% | 67% |
| Overall | 0.01% | 0.10% | -0.08% | 100% | 66% |
| Class D | 0.14% | 0.28% | -0.09% | 102% | 68% |
| Class F | -0.08% | 0.04% | 0.48% | 70% | 64% |

Coding performance and complexity of Test (c)

Table 3. Test results of CE 4-1.5.c

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.04% | -0.03% | 100% | 101% |
| Class A2 | -0.07% | -0.07% | -0.03% | 100% | 102% |
| Class B | -0.04% | -0.06% | -0.02% | 99% | 102% |
| Class C | 0.01% | -0.06% | -0.08% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.06% | -0.04% | 100% | 102% |
| Class D | 0.05% | -0.12% | -0.15% | 101% | 101% |
| Class F | -0.03% | 0.00% | -0.05% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.07% | -0.36% | 80% | 97% |
| Class C | 0.07% | 0.17% | 0.03% | 80% | 97% |
| Class E | 0.12% | 0.05% | -0.48% | 82% | 97% |
| Overall | 0.04% | 0.04% | -0.26% | 81% | 97% |
| Class D | 0.17% | 0.52% | -0.62% | 81% | 95% |
| Class F | -0.24% | -0.28% | -0.06% | 58% | 96% |

Conclusion

This proposal reports the experimental results of Core Experiment (CE) 4.1.5. Under the CTC test conditions, the average luma BD-rate change of these tests are: Test a: -0.01% and -0.01% for RA and LDB, respectively, Test b: -0.03% and 0.01% for RA and LDB, respectively and Test c: -0.03% and 0.04% for RA and LDB, respectively.

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).