| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0213_v3 |
| --- | --- |

| Title: | CE4-related: Remove TMVP candidate for the specified blocksizes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd., Peking University |  |  |

_____________________________

- Abstract

This contribution proposes to remove unnecessary Temporal Motion Vector Prediction (TMVP) derivation process in inter prediction process. In this contribution, the complexity and procedure of TMVP has been analyzed and 4 tests have been conducted.

- Test (a): Remove TMVP merge candidate for the block size with samples <= 32, and remove TMVP for AMVP mode with 4x4 blocksize:

-0.02% and 0.04% for RA and LDB, respectively

- Test (b): Remove TMVP merge candidate for the block size with samples <= 64, and remove TMVP for AMVP mode with 4x4 blocksize:

-0.03% and 0.02% for RA and LDB, respectively

- Test (c): Remove 4x4 TMVP for merge and AMVP modes:

-0.01% and 0.01% for RA and LDB, respectively

- Test (d): Remove both 8x4/4x8 TMVP candidate from merge and AMVP for the block size with samples <= 32:

0.00% and 0.03% for RA and LDB, respectively

Introduction

In current design, TMVP is involved in many coding tools, including translation merge, AMVP mode, affine merge, affine mvp mode, ATMVP mode and triangle merge mode. Different processing block size is set for these modes, which is summarized in Table.1.

Table 1. Processing block size for TMVP related modes

| Coding tools | Processing block size | Procedure in TMVP | Position/PU derive | Boundary check | scale |
| --- | --- | --- | --- | --- | --- |
| Translation merge | all block | check C0C1 | up to 2 | 1 | 1 |
| AMVP | all block | check C0C1 | up to 2 | 1 | 1 |
| Affine merge | width ≥ 8 & height ≥ 8 | check C0 | 1 | 1 | 1 |
| ATMVP | width ≥ 8 & height ≥ 8 | check center position | 1 | 0/clip | N |
| Affine mvp | width > 8 & height > 8 | check C0C1 | up to 2 | 1 | 1 |
| Triangle | width * height ≥ 64 | check C0 and check C1 | 2 | 1 | 2 |

From Table 1, it can be seen that except for affine mvp mode, small blocks which samples <=64 will be processed by all of the above coding tools. With the granularity of temporal MV storage has been set to 8x8 and the adoption of 8x8 ATMVP, it exists some unnecessary operations in TMVP deriving process:

- ATMVP derives multiple motion for 8x8 sub-blocks of one CU based on the motion information of the collocated blocks from temporal reference picture. When the size of current CU is 8x8, only one MVP will be derived for current CU in ATMVP, resulting in the redundant process in ATMVP process since the main idea of this tool is to derive multiple motion for one CU.

- Since the granularity of temporal MV storage has been set to 8x8, TMVP with smaller block size might become unnecessary in some cases, e.g. the width or height of CU is 4, when some adjacent CUs derive MVP from the same MV storage units, which will make these kind of partition meaning less.

- With the adoption of HMVP, more information of nearing coded blocks have been utilized, including both spatial and temporal information. Thus, TMVP might not be necessary for some blocks. Since the construction of MVP list becomes more complex, removing unnecessary process is benefit for hardware design.

Proposed Method

From Table 1, is can be seen that for a block which samples <=64, up to 10 position/PU derivation, 5 boundary checks and 7 scale operations needed be conducted, which makes the timing of small blocks even tight. In order to remove the unnecessary process of deriving TMVP, it is proposed to remove TMVP from merge list and AMVP list at the specified block sizes.

Three cases have been tested over the construction of current mvp list in current design.

Remove TMVP merge candidate for block size with samples ≤ 32, TMVP for AMVP mode for 4x4 block size (Test a)

In this test, 4x4/4x8/8x4 TMVP is removed from merge list and 4x4 TMVP is also removed from AMVP list.

At the 13th JVET meeting, Share merge list has been adopted to the current VVC design which makes some small blocks share the same merge list. In the current design, the threshold of Share merge list is set to 32, which means part of blocks with samples smaller than 32 will set as sharing blocks for parallel processing. In this test, it remove TMVP candidate from merge list for all the blocks with samples <=32 and remove TMVP from AMVP list for 4x4 blocks, which are benefit for hardware design since the timing for processing smaller blocks can be reduced and make the construction process of merge/AMVP list even simpler.

Remove TMVP merge candidate for block size with samples ≤ 64, TMVP for AMVP mode for 4x4 block size (Test b)

In this test, 4x4/4x8/8x4/4x16/16x4/8x8 TMVP is removed from merge list and 4x4 TMVP is also removed from AMVP list.

Since the granularity of temporal MV storage has been set to 8x8, TMVP with smaller block size might become unnecessary in some cases, e.g. the width or height of CU is 4, when some adjacent CUs derive MVP from the same MV storage units, which will make these kind of partition meaning less.

Besides, ATMVP derives multiple motion for 8x8 sub-blocks of one CU based on the motion information of the collocated blocks from temporal reference picture. When the size of current CU is 8x8, only one MVP will be derived for current CU in ATMVP, resulting in the redundant process between ATMVP and TMVP.

Remove 4x4 TMVP for merge and AMVP modes (Test c)

In this test, 4x4 TMVP is removed in all modes. From Table 1, it can be seen that for a 4x4 block, up to 4 position/PU derivation, 2 boundary checks and 2 scale operations needed be conducted. With removal of TMVP for 4x4 blocks, these operations can be saved and it is more friendly for hardware design.

Experimental Results

The experiments were performed under JVET common test conditions.

BD-Rate results of the tests on top of the VTM-4.0 compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets. Due to the unstable performance of the computer resource, the coding time is not reliable.

Coding performance and complexity of Test (a)

Table 2. Test results of Test(a)

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.02% | 102% | 99% |
| Class A2 | -0.04% | -0.05% | -0.05% | 101% | 99% |
| Class B | -0.02% | -0.02% | -0.11% | 100% | 99% |
| Class C | 0.00% | -0.13% | -0.07% | 106% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.06% | 102% | 99% |
| Class D | 0.02% | -0.03% | 0.02% | 107% | 97% |
| Class F | -0.02% | 0.02% | -0.01% | 88% | 98% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.01% | -0.18% | 81% | 98% |
| Class C | 0.05% | 0.34% | 0.04% | 83% | 98% |
| Class E | 0.10% | 0.41% | -0.48% | 83% | 97% |
| Overall | 0.04% | 0.22% | -0.18% | 82% | 98% |
| Class D | 0.09% | 0.03% | -0.31% | 81% | 97% |
| Class F | -0.14% | -0.43% | -0.37% | 58% | 95% |

Coding performance and complexity of Test (b)

Table 3. Test results of Test(b)

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.00% | 107% | 100% |
| Class A2 | -0.07% | -0.07% | -0.07% | 109% | 99% |
| Class B | -0.04% | -0.11% | -0.03% | 100% | 98% |
| Class C | 0.00% | -0.05% | -0.02% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.08% | -0.03% | 104% | 99% |
| Class D | 0.05% | 0.01% | -0.05% | 106% | 97% |
| Class F | -0.03% | 0.00% | -0.02% | 85% | 96% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.30% | 0.00% | 82% | 95% |
| Class C | 0.03% | 0.16% | 0.04% | 83% | 98% |
| Class E | 0.07% | 0.30% | -0.60% | 83% | 97% |
| Overall | 0.02% | 0.01% | -0.14% | 83% | 96% |
| Class D | 0.17% | 0.10% | -0.15% | 81% | 97% |
| Class F | -0.26% | -0.26% | -0.21% | 58% | 93% |

Coding performance and complexity of Test (c)

Table 4. Test results of Test(c)

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | 0.00% | 101% | 99% |
| Class A2 | -0.07% | -0.07% | -0.07% | 104% | 100% |
| Class B | 0.00% | 0.05% | -0.01% | 106% | 100% |
| Class C | 0.01% | 0.02% | 0.00% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.02% | 103% | 99% |
| Class D | 0.02% | 0.06% | -0.04% | 101% | 97% |
| Class F | -0.01% | -0.01% | 0.00% | 85% | 97% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.07% | -0.15% | 81% | 96% |
| Class C | 0.00% | 0.21% | -0.04% | 81% | 98% |
| Class E | 0.02% | 0.07% | -0.54% | 82% | 98% |
| Overall | 0.01% | 0.06% | -0.21% | 81% | 97% |
| Class D | 0.03% | 0.43% | -0.26% | 82% | 97% |
| Class F | -0.15% | -0.16% | 0.74% | 58% | 95% |

Coding performance and complexity of Test (d)

Table 5. Test results of Test(d)

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.11% | -0.01% | 106% | 99% |
| Class A2 | -0.05% | -0.03% | -0.06% | 103% | 99% |
| Class B | 0.00% | 0.00% | 0.01% | 106% | 99% |
| Class C | 0.02% | -0.08% | -0.04% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 104% | 99% |
| Class D | 0.02% | 0.05% | -0.07% | 101% | 97% |
| Class F | -0.02% | 0.02% | 0.00% | 89% | 98% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.20% | -0.38% |  |  |
| Class C | 0.07% | 0.25% | 0.22% |  |  |
| Class E | 0.06% | 0.45% | -0.48% |  |  |
| Overall | 0.03% | 0.11% | -0.21% |  |  |
| Class D | 0.14% | -0.05% | -0.06% |  |  |
| Class F | -0.17% | -0.72% | -0.10% | 59% | 102% |

Conclusion

This proposal suggests to remove TMVP process from merge list and AMVP list at specified block sizes. Under the CTC test conditions, the average luma BD-rate change of four tests are: Test a: -0.02% and 0.04% for RA and LDB, respectively, Test b: -0.03 % and 0.02 % for RA and LDB, respectively, and Test c: % and % for RA and LDB, respectively .

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).