| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0214 |
| --- | --- |

| Title: | CE8: BDPCM with harmonized residual coding and CCB limitation (CE8-3.1a, CE8-3.1b, CE8-5.1a, CE8-5.1b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | G. Clare (b<>com) F. Henry (Orange) B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI) M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent) Orange Labs 4 rue Clos Courtel 35510 Cesson Sevigne France b<>com 1219 Avenue des Champs Blancs, 35510 Cesson-Sévigné France Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +33 2 99 12 45 58 gordon.clare@b-com.com felix.henry@orange.com firstname.lastname@hhi.fraunhofer.de {mengxxu, xlxiangli, xiaozhongxu, maxwellgao, shanl}@tencent.com |
| Source: | b<>com, HHI, Orange, Tencent |  |  |

_____________________________

# Abstract

This contribution reports the study and results of CE8-3.1a, CE8-3.1b, CE8-5.1a and CE8-5.1b.

CE8-3.1 uses BDPCM as described in JVET-M0057 but, instead of using a BDPCM-specific residual coding, it re-uses the residual coding proposed for transform skip in JVET-M0464.

CE8-5.1 is a combination of CE8-3.1 with CE8-4.3 : the residual coding of JVET-M0464 is used both for the transform skip residual and for the BDPCM residual.

In all CEs, the “a” and “b” variant correspond respectively to a limitation of the maximum number of Context Coded Bins (CCBs) per sample to 2 and 3 respectively.

The following performances (YUV BDR change, Encoder time with no shortcut, decoder time) are reported in All Intra for class F and class SCC:

CE8-3.1a:

Class F		-2,63%	-1,72%	-1,70%	104%	100%

ClassSCC	-3,51%	-2,25%	-2,20%	103%	99%

CE8-3.1b:

Class F		-2,78%	-1,74%	-1,84%	104%	100%

ClassSCC 	-3,78%	-2,41%	-2,35%	102%	99%

CE8-5.1a:

Class F		-3,77%	-2,26%	-2,31%	103%	100%

ClassSCC 	-5,83%	-3,24%	-3,25%	102%	98%

CE8-5.1b:

Class F		-3,94%	-2,38%	-2,59%	104%	101%

ClassSCC	-6,23%	-3,63%	-3,63%	102%	97%

# Rationale

In JVET-M0057 [1] a new version of BDPCM was proposed, which consists of a Block-based DPCM in the spatial domain with either vertical or horizontal prediction (without intra prediction or transform stages). This type of prediction ensures that the throughput is sufficiently high (at least 8 pixels per cycle for any block size), while maintaining the same compression performance as earlier versions. During the Marrakesh meeting, it was commented that BDPCM uses its own method to encode its residue, and that it would be desirable to harmonize the residual coding with existing solutions.

On the other hand, JVET-M0464 [2] proposed a few modifications to be made to the regular residual coding in order to code the transform skip residue. These modifications permit the residual coding of transform skip to be very close to the regular one while significantly improving performance for Screen Content. It was thus suspected that BDPCM could also use the JVET-M0464 residual coding method.

Consequently, all tests described in this document have in common the fact that BDPCM uses JVET-M0464 residual coding.

In JVET-M0464, an individual sample may require a significant number of context coded bins (CCBs). In order to limit the number of CCBs at the block level, contribution JVET-M0449 [3] proposed to count the number of CCBs in a block, and switch to bypass any remaining syntax elements in the block, once a given limit is reached. The limit is set to k x W x H where W and H are the width and height of the block. This ensures that there are at most k CCBs per sample in the block. Values k=2 and 3 are tested in the core experiments.

Table 1 summarizes the characteristics of each test described in this document.

Table 1: configuration of each test

|  | BDPCM residual coding | Transform skip residual coding | CCB limitation |
| --- | --- | --- | --- |
| CE8-3.1a | JVET-M0464 | Regular residual coding | 2 |
| CE8-3.1b | JVET-M0464 | Regular residual coding | 3 |
| CE8-5.1a | JVET-M0464 | JVET-M0464 | 2 |
| CE8-5.1b | JVET-M0464 | JVET-M0464 | 3 |

The next sections describe each technology in more detail. As requested in CE8 [4], section 5 describes the encoder implemented (with respect to shortcut aspects).

# BDPCM

In this section, we describe BDPCM as proposed in JVET-M0057 [1]

A bdpcm_flag is transmitted at the CU level whenever it is a luma intra CU having each dimension smaller than or equal to 32. This flag indicates whether regular intra coding or DPCM is used. This flag is encoded using a single CABAC context.

If DPCM is used, a flag is transmitted to indicate whether the prediction is horizontal or vertical. The predictor is similar to the RDPCM predictor for HEVC (sample copy in the prediction direction). The reference pixels used are the unfiltered ones.

Due to the shape of the horizontal (resp. vertical) predictor, which use the left (A)  (resp. top (B)) pixel for prediction of the current pixel, the most throughput-efficient way of processing the block is to process all the pixels of one column (resp. line)  in parallel, and to process these columns (resp. lines) sequentially. In order to increase throughput, a block of height 4 is divided into two halves with a vertical frontier when the predictor chosen on this block is horizontal, and a block of width 4 is divided into two halves with a horizontal frontier when the predictor chosen on this block is vertical..

When a block is divided, samples from one area are not allowed to use pixels from another area to compute the prediction: if this situation occurs, the prediction pixel is replaced by the reference pixel in the prediction direction. This is shown on Figure 1 for different positions of current pixel X in a 4x8 block predicted vertically.

Figure 1: dividing a block of 4x8 samples into two independently decodable areas.

Thanks to this property, it then becomes possible to process any block size with a throughput of at least 8 samples per cycle.

The prediction error is quantized in the spatial domain, after rescaling, in a way identical to the Transform Skip quantizer. Each pixel is reconstructed by adding the dequantized prediction error to the prediction. Thus, the reconstructed pixels are used to predict the next pixels in raster-scan order.

In order to maintain the coherence of the regular intra mode prediction, the first mode in the MPM list is associated with a Block-DPCM CU (without being transmitted) and is available for MPM generation for subsequent blocks.

The deblocking filter is de-activated on a border between two Block-DPCM blocks, since neither of the blocks uses the transform stage usually responsible for blocking artifacts.

# Transform Skip Residual Coding

The method described here was originally proposed in JVET-M0464 [2]. While this method is called Transform Skip Residual Coding, we also use it for BDPCM residual coding in the present Core Experiment, as it is considered an efficient way of coding spatial residues that do not exhibit the properties of transformed residues (such as energy clustering on certain predefined frequencies).

In order to adapt the residual coding to the statistics and signal characteristics of the transform skip levels, which represent the quantized prediction residual (spatial domain), it was proposed to modify the following:

No last significant scanning position: Since the residual signal reflects the spatial residual after the prediction and no energy compaction by transform is performed for TS, the higher probability for trailing zeros or insignificant levels at the bottom right corner of the transform block is not given anymore. Thus, last significant scanning position signalling is omitted in this case. Instead, the first subblock to be processed is the most bottom right subblock within the transform block.

Subblock CBFs: The absence of the last significant scanning position signalling requires the subblock CBF signalling with coded_sub_block_flag for TS to be modified as follows:

- Due to quantization, the aforementioned sequence of insignificance may still occur locally inside a transform block. Thus, the last significant scanning position is removed as described before and coded_sub_block_flag is coded for all sub-blocks.

- The coded_sub_block_flag for the subblock covering the DC frequency position (top-left subblock) presents a special case. In VVC Draft 3, the coded_sub_block_flag for this subblock is never signaled and always inferred to be equal to 1. When the last significant scanning position is located in another subblock, it means that there is at least one significant level outside the DC subblock. Consequently, the DC subblock may contain only zero/non-significant levels although the coded_sub_block_flag for this subblock is inferred to be equal to 1. With the absence of the last scanning position information in TS, the coded_sub_block_flag for each subblock is signaled. This also includes the coded_sub_block_flag for the DC subblock except when all other coded_sub_block_flag syntax elements are already equal to 0. In this case, the DC coded_sub_block_flag is inferred to be equal to 1 (inferDcSbCbf=1). Since there has to be at least one significant level in this DC subblock, the sig_coeff_flag syntax element for the first position at (0,0) is not signaled and derived to be equal to 1 (inferSbDcSigCoeffFlag=1) instead if all other sig_coeff_flag syntax elements in this DC subblock are equal to 0.

- The context modeling for coded_sub_block_flag is changed. The context model index is calculated as the sum of the coded_sub_block_flag to the right and the coded_sub_block_flag below the current subblock instead of and a logical disjunction of both.

sig_coeff_flag context modelling: The local template in sig_coeff_flag  context modeling is modified to only include the neighbor to the right (NB0) and the neighbor below (NB1) the current scanning position. The context model offset is just the number of significant neighboring positions sig_coeff_flag[NB0] + sig_coeff_flag[NB1]. Hence, the selection of different context sets depending on the diagonal d within the current transform block is removed. This results in three context models and a single context model set for coding the sig_coeff_flag flag.

abs_level_gt1_flag and par_level_flag context modelling: a single context model is employed for abs_level_gt1_flag and par_level_flag.

abs_remainder coding: Although the empirical distribution of the transform skip residual absolute levels typically still fits a Laplacian or a Geometrical distribution, there exist larger instationarities than for transform coefficient absolute levels. Particularly, the variance within a window of consecutive realization is higher for the residual absolute levels. This motivates the following modifications of the abs_remainder syntax binarization and context modelling:

- Using a higher cutoff value in the binarization, i.e., the transition point from the coding with sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag to the Rice codes for abs_remainder, and dedicated context models for each bin position yields higher compression efficiency. Increasing the cutoff will result in more "greater than X" flags, e.g. introducing abs_level_gt5_flag, abs_level_gt7_flag, and so on until a cutoff is reached. The cutoff itself is fixed to 5 (numGtFlags=5).

- The template for the rice parameter derivation is modified, i.e., only the neighbor to the left and the neighbor below the current scanning position are considered similar to the local template for sig_coeff_flag context modeling.

coeff_sign_flag context modelling: Due to the instationarities inside the sequence of signs and the fact that the prediction residual is often biased, the signs can be coded using context models, even when the global empirical distribution is almost uniformly distributed. A single dedicated context model is used for the coding of the signs and the sign is parsed after sig_coeff_flag to keep all context coded bins together.

Reduction of context coded bins: The first scanning pass, i.e., the transmission of the sig_coeff_flag, abs_level_gt1_flag, and par_level_flag syntax elements, is unchanged. However, the limit on the maximum number of context coded bins per sample (CCBs) is removed and handled differently. The reduction of CCBs can be guaranteed by specifying a mode with CCB > k as invalid with k being a positive integer number. Note that k=2 for the regular level coding mode of the current VVC development. Such a limitation corresponds to a reduction of the quantization space.

# CCBs limitation

This mechanism is described in JVET-M0449 [3]. Note that in JVET-M0449, the CCB limitation context is applied to an obsolete version  of BDPCM residual coding, but the same principle is applied to the Transform Skip residual coding in the present CE.

To improve the syntax parsing throughput, it is proposed to restrict with up to k context coded bins for each BDPCM residue. That is, for an MxN block in a single luma or chroma channel, only the first kxMxN bins of the absolute residue are coded with context, and the remaining bins are coded with equal probability. Note that this is in line with current coefficient coding where up to 32 context coded bins are allowed for each 4x4 coefficient group.

# Encoder shortcuts

For the main results (section 6), no encoder shortcuts are performed.  The two BDPCM modes (horizontal and vertical prediction) are consistently checked prior to all the intra modes subsequently being checked.

We provide a few additional results (section 7) with an encoder shortcut as described in JVET-M0057 [1] or JVET-M0822 [5]: during the full RD check of intra modes, if BDPCM has a lower RD cost than the first intra mode, then other intra modes are not evaluated and BDPCM is selected for the current block.

# Main Results

The tests were performed according to the test conditions defined in CE8 [4].

At the time of contribution, all results were available for Class F and Screen Content in All Intra and Random Access.

Due to the large number of test and references, there were still code points missing for other configurations.  The issue of runtimes in CE8 was discussed on the CE8 reflector and it was decided to keep all configurations in CE8 as initially planned, knowing that there could be late results (such as IBC on, low delay, natural content).

Results tend to be more complete for CE8-5.1 as this CE is considered to be the most relevant one for adoption.

The following sections report the results for individual CEs covered in this contribution, for All Intra and Low Delay. The complete set of results can be found in the spreadsheets joined to this contribution.

Additional results in combination with Palette are provided in section 7.

## CE8-3.1a Results

### IBC off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,05% | 0,05% | 110% | 102% |
| Class A2 | 0,01% | 0,09% | -0,01% | 109% | 100% |
| Class B | -0,02% | 0,01% | 0,07% | 110% | 101% |
| Class C | -0,19% | -0,18% | -0,04% | 108% | 100% |
| Class E | -0,08% | 0,02% | 0,04% | 110% | 100% |
| Overall | -0,06% | -0,01% | 0,03% | 109% | 101% |
| Class D | -0,24% | -0,27% | -0,29% | 108% | 98% |
| Class F | -5,90% | -4,52% | -4,51% | 109% | 97% |
| ClassTGM | -15,10% | -12,15% | -12,24% | 108% | 93% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | 0,04% | 0,05% | -0,20% | 101% | 99% |
| Class C | 0,01% | 0,11% | -0,07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | 0,05% | -0,28% | -0,09% | 101% | 101% |
| Class F | -4,04% | -3,48% | -3,29% | 102% | 100% |
| ClassTGM | -7,09% | -5,25% | -5,29% | 101% | 99% |

### IBC on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,07% | -0,04% | 107% | 102% |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | 0,00% | -0,03% | 0,01% | 106% | #NOMBRE! |
| Class C | -0,19% | -0,01% | -0,11% | 105% | 104% |
| Class E | 0,00% | -0,05% | 0,06% | 106% | 102% |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | -0,19% | -0,24% | -0,11% | 105% | 103% |
| Class F | -2,63% | -1,72% | -1,70% | 104% | 100% |
| ClassTGM | -3,51% | -2,25% | -2,20% | 103% | 99% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,19% | 0,05% | 102% | 98% |
| Class A2 | 0,00% | -0,04% | 0,11% | 101% | 99% |
| Class B | 0,05% | -0,16% | -0,23% | 101% | 100% |
| Class C | 0,00% | 0,27% | 0,05% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | -0,03% | -0,03% | 101% | #NOMBRE! |
| Class D | 0,07% | 0,00% | 0,23% | 101% | 102% |
| Class F | -1,74% | -1,26% | -1,29% | 101% | 101% |
| ClassTGM | -1,61% | -0,81% | -0,87% | 100% | 101% |

## CE8-3.1b Results

### IBC off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,04% | 0,03% | 110% | 102% |
| Class A2 | 0,01% | 0,10% | 0,00% | 109% | 102% |
| Class B | -0,02% | 0,02% | 0,09% | 110% | 101% |
| Class C | -0,19% | -0,17% | -0,17% | 108% | 102% |
| Class E | -0,08% | 0,00% | 0,03% | 110% | 104% |
| Overall | -0,06% | -0,01% | 0,00% | 109% | 102% |
| Class D | -0,22% | -0,25% | -0,33% | 108% | 100% |
| Class F | -6,26% | -4,90% | -4,82% | 108% | 98% |
| ClassTGM | -16,09% | -13,20% | -13,29% | 106% | 94% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | 0,03% | 0,05% | -0,17% | 101% | 99% |
| Class C | 0,01% | -0,03% | -0,20% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | 0,03% | -0,15% | -0,12% | 101% | 101% |
| Class F | -4,37% | -3,76% | -3,69% | 102% | 100% |
| ClassTGM | -7,72% | -5,89% | -5,86% | 100% | 99% |

### IBC on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,07% | -0,06% | 106% | 100% |
| Class A2 | 0,03% | 0,06% | 0,01% | 106% | 101% |
| Class B | 0,00% | -0,03% | -0,03% | 106% | 103% |
| Class C | -0,20% | -0,01% | -0,12% | 105% | 104% |
| Class E | 0,00% | -0,02% | 0,09% | 106% | 103% |
| Overall | -0,04% | -0,01% | -0,03% | 106% | 102% |
| Class D | -0,21% | -0,24% | -0,17% | 105% | 103% |
| Class F | -2,78% | -1,74% | -1,84% | 104% | 100% |
| ClassTGM | -3,78% | -2,41% | -2,35% | 102% | 99% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,21% | 0,04% | 101% | 99% |
| Class A2 | 0,01% | -0,04% | 0,11% | 101% | 100% |
| Class B | 0,06% | -0,22% | -0,09% | 101% | 101% |
| Class C | -0,02% | 0,22% | 0,00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | -0,07% | 0,00% | 101% | #NOMBRE! |
| Class D | 0,03% | 0,11% | 0,34% | 101% | 102% |
| Class F | -1,81% | -1,45% | -1,35% | 101% | 101% |
| ClassTGM | -1,68% | -0,92% | -0,92% | 100% | 101% |

## CE8-5.1a Results

### IBC off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,28% | 0,30% | 0,45% | 110% | 102% |
| Class A2 | 0,01% | 0,07% | 0,00% | 109% | 101% |
| Class B | -0,03% | 0,02% | 0,05% | 110% | 102% |
| Class C | -0,33% | -0,30% | -0,29% | 108% | 102% |
| Class E | -0,11% | 0,00% | -0,06% | 110% | 102% |
| Overall | -0,14% | 0,00% | 0,02% | 109% | 102% |
| Class D | -0,26% | -0,18% | -0,33% | 107% | 101% |
| Class F | -7,19% | -5,37% | -5,42% | 108% | 94% |
| ClassTGM | -17,91% | -14,15% | -14,34% | 107% | 90% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,22% | 0,05% | 102% | 101% |
| Class A2 | -0,01% | -0,15% | -0,01% | 102% | 100% |
| Class B | 0,05% | 0,01% | -0,30% | 101% | 100% |
| Class C | 0,02% | -0,04% | -0,15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | 0,01% | -0,13% | 101% | 100% |
| Class D | 0,16% | -0,41% | -0,30% | 99% | 101% |
| Class F | -4,95% | -3,92% | -3,77% | 101% | 99% |
| ClassTGM | -9,29% | -6,39% | -6,41% | 99% | 99% |

### IBC on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,27% | 0,20% | 0,37% | 107% | 101% |
| Class A2 | 0,04% | 0,03% | 0,05% | 107% | 101% |
| Class B | -0,02% | -0,05% | 0,01% | 106% | #NOMBRE! |
| Class C | -0,32% | -0,20% | -0,26% | 105% | 104% |
| Class E | -0,03% | -0,04% | 0,10% | 106% | 102% |
| Overall | -0,12% | -0,03% | 0,03% | 106% | 102% |
| Class D | -0,25% | -0,28% | -0,13% | 105% | 102% |
| Class F | -3,77% | -2,26% | -2,31% | 103% | 100% |
| ClassTGM | -5,83% | -3,24% | -3,25% | 102% | 98% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,01% | 0,24% | 102% | 99% |
| Class A2 | 0,02% | -0,14% | 0,00% | 101% | 99% |
| Class B | 0,07% | -0,12% | -0,27% | 101% | 99% |
| Class C | -0,02% | 0,26% | 0,07% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | 0,00% | -0,03% | 101% | 100% |
| Class D | 0,17% | 0,06% | 0,26% | 99% | 102% |
| Class F | -2,65% | -1,71% | -1,77% | 101% | 102% |
| ClassTGM | -3,54% | -1,23% | -1,13% | 99% | 101% |

## CE8-5.1b Results

### IBC off

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,29% | 0,32% | 0,42% | 110% | 102% |
| Class A2 | 0,02% | 0,07% | 0,02% | 109% | 103% |
| Class B | -0,03% | 0,01% | 0,03% | 110% | 101% |
| Class C | -0,32% | -0,32% | -0,24% | 108% | 102% |
| Class E | -0,11% | -0,05% | -0,03% | 109% | 102% |
| Overall | -0,14% | -0,01% | 0,02% | 109% | 102% |
| Class D | -0,29% | -0,25% | -0,27% | 107% | 100% |
| Class F | -7,80% | -5,97% | -5,94% | 108% | 96% |
| ClassTGM | -19,10% | -15,39% | -15,56% | 106% | 91% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,25% | 0,08% | 102% | 101% |
| Class A2 | 0,00% | -0,13% | -0,04% | 102% | 100% |
| Class B | 0,05% | 0,10% | -0,15% | 101% | 99% |
| Class C | -0,01% | -0,21% | -0,29% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | 0,00% | -0,12% | 101% | 100% |
| Class D | 0,14% | -0,43% | -0,61% | 99% | 101% |
| Class F | -5,50% | -4,37% | -4,26% | 101% | 99% |
| ClassTGM | -10,08% | -7,25% | -7,25% | 99% | 98% |

### IBC on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,27% | 0,21% | 0,40% | 107% | 101% |
| Class A2 | 0,03% | -0,02% | 0,06% | 106% | 101% |
| Class B | -0,02% | -0,07% | 0,02% | 106% | #NOMBRE! |
| Class C | -0,33% | -0,20% | -0,22% | 105% | 104% |
| Class E | -0,03% | -0,03% | 0,09% | 106% | 102% |
| Overall | -0,12% | -0,04% | 0,05% | 106% | 102% |
| Class D | -0,26% | -0,31% | -0,11% | 105% | 103% |
| Class F | -3,94% | -2,38% | -2,59% | 104% | 101% |
| ClassTGM | -6,23% | -3,63% | -3,63% | 102% | 97% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class B | 0,07% | -0,01% | -0,06% | 101% | 98% |
| Class C | -0,01% | 0,19% | 0,10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #NOMBRE! | #NOMBRE! |
| Class D | 0,15% | -0,05% | 0,07% | 99% | 102% |
| Class F | -2,80% | -1,82% | -1,96% | 101% | 101% |
| ClassTGM | -3,77% | -1,56% | -1,48% | 99% | 101% |

# Additional results with encoder shortcut

Additional results are provided in combination with the encoder shortcut. In order to reduce the number of simulations to test, we provide results only for CE8-5.1b, All Intra, Class F and TGM.

## IBC off

|  |  |  | All Intra FASTER |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -6,96% | -5,42% | -5,40% | 91% | 97% |
| ClassTGM | -17,68% | -14,33% | -14,50% | 72% | 92% |

## IBC on

|  |  |  | All Intra FASTER |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3,84% | -2,40% | -2,51% | 95% | 102% |
| ClassTGM | -6,04% | -3,53% | -3,52% | 86% | 98% |

# Additional results with Palette

Additional results are provided in combination with Palette. In order to reduce the number of simulations to test, we provide results only for CE8-5.1b.

## CE8-5.1b, IBC off, Palette on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCOFF plt1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class B | 0,02% | 0,04% | 0,06% | 111% | 100% |
| Class C | -0,26% | -0,26% | -0,20% | 110% | 99% |
| Class E | -0,04% | -0,02% | -0,01% | 111% | 100% |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | -0,22% | -0,11% | -0,42% | 110% | 100% |
| Class F | -1,61% | -0,90% | -0,83% | 113% | 101% |
| ClassTGM | -2,63% | -2,06% | -2,16% | 119% | 103% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCOFF plt1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class B | 0,08% | 0,12% | -0,11% | 101% | 100% |
| Class C | 0,04% | -0,06% | -0,11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | 0,21% | -0,18% | -0,34% | 101% | 100% |
| Class F | -1,31% | -0,64% | -0,81% | 102% | 101% |
| ClassTGM | -2,14% | -1,20% | -1,15% | 101% | 101% |

## CE8-5.1b, IBC on, Palette on

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 CE8 IBCON plt1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class B | 0,03% | 0,01% | 0,06% | 110% | 100% |
| Class C | -0,26% | -0,17% | -0,15% | 109% | 99% |
| Class E | 0,04% | 0,06% | 0,29% | 109% | 101% |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | -0,19% | -0,24% | -0,15% | 109% | 100% |
| Class F | -2,30% | -1,53% | -1,60% | 106% | 100% |
| ClassTGM | -3,17% | -2,00% | -2,05% | 107% | 99% |
|  |  |  | Random Access |  |  |
|  |  |  | Over VTM-4.0 CE8 IBCON plt1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class A2 | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class B | 0,10% | -0,03% | -0,04% | 103% | 100% |
| Class C | 0,03% | 0,29% | -0,06% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALEUR! | #VALEUR! | #VALEUR! | #DIV/0! | #DIV/0! |
| Class D | 0,24% | 0,03% | 0,13% | 103% | 100% |
| Class F | -1,82% | -1,11% | -1,19% | 101% | 100% |
| ClassTGM | -2,22% | -0,97% | -0,83% | 100% | 100% |

# Bibliography

[1]  JVET-M0057 CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b), 	F. Henry, M. Abdoli (Orange), P. Philippe, G. Clare (b<>com)

[2] JVET-M0464 Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)

[3] JVET-M0449 CE8-related: BDPCM entropy coding with reduced number of context coded bins, M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)

[4] JVET-M1028 Description of Core Experiment 8 (CE8): Screen Content Coding Tools, 	X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

[5] JVET-M0822 Non-CE8: Encoder optimization for palette mode, 	H. Wang, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)

# Patent rights declaration(s)

b<>com, HHI, Orange and Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).