| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0215 |
| --- | --- |

| Title: | Non-CE3 : Intra prediction mode restriction in small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dae Young Lee Tea Hyeon Kim Gwang Hoon Park 1732, Deogyeong-daero, Giheung-gu, Yongin-si, Gyeonggi-do 17104, KOREA | Tel: Email: | +82-31-201-3680 daeyung2002@khu.ac.kr taehyun@khu.ac.kr ghpark@khu.ac.kr |
| Source: | Kyung Hee University |  |  |

_____________________________

- Abstract

This contribution proposes a method of restricting the use of the odd numbered intra prediction mode in the 4x4 blocks. Odd numbered angular intra mode is restricted in the 4x4 block, hence only the even numbered angular intra mode is used. It is reported that the average luma BD-rate for AI case is XX% compared to VTM 4.0, respectively, and it is reported that the average luma BD-rate for RA case is XX% compared to VTM 4.0.1, respectively.

Introduction

In the intra prediction, many directional modes are used as shown in Fig.1. As the number of intra prediction modes increases, the prediction error becomes smaller, but the magnitude of the signal for expressing the intra prediction mode also increases. In a small block such as 4x4, the signal size for expressing the prediction mode may be burden rather than the size of the prediction error. Therefore, we propose a method of using a limited number of intra prediction modes as shown in Fig.2.

Figure 1. Intra prediction modes in VVC

Figure 2. Proposed a method of using a limited number of intra prediction modes in 4x4 blocks

Algorithm description

Encoder side

Intra prediction and cost calculation for the odd directional mode are skipped for the 4x4 block. As shown in Fig.2, the intra prediction and cost derivation processes for the even directional mode, the PLANAR mode, and the DC mode are performed first, and the intra prediction and the cost derivation process for the odd directional mode for the 4x4 block are omitted.

Encode & decoder side

Modified 6 MPM generation process in 4x4 blocks

In MPM generation of 4x4 blocks, the directional mode uses only even numbers. Details are given in Table 1. Our proposal is not applied to ISP and MRL cases.

Table 1. Modified 6MPM generation process in proposal

| VTM 4.0 | Proposal ( |  |
| --- | --- | --- |
|  | 4x4 blocks (For angular modes) | The others (no changes) |
| Left mode | (Left mode >> 1) << 1 | Left mode |
| Above mode | (Above mode >> 1) << 1 | Above mode |
| Neighbor mode + 1 | Neighbor mode + 2 | Neighbor mode + 1 |
| Neighbor mode - 1 | Neighbor mode – 2 | Neighbor mode - 1 |
| Neighbor mode + 2 | Neighbor mode + 4 | Neighbor mode + 2 |
| Neighbor mode - 2 | Neighbor mode – 4 | Neighbor mode - 2 |

Truncated Binary Code (TBC) for 29 non-MPM modes in 4x4 blocks

In the proposed method, since the 32 odd angular modes of the intra prediction are not used in the 4x4 blocks, the TBC is performed for a total of 29 modes in the 4x4 blocks which excludes the 32 odd modes and 6 MPM modes. As result, non-MPM TBC for 61 modes are changed to non-MPM TBC for 29 modes in the 4x4 blocks. Our proposal is not applied to ISP and MRL cases.

Experimental results

Experiments were performed based on VTM 4.0 software [1] and VTM 4.0.1 software [2] under JVET common test condition [3].

Table 2 shows experimental results of the proposed method compared to VTM 4.0 for AI, and experimental results of the proposed method compared to VTM 4.0.1 for RA.

Table 2. Results of the proposed method over VTM 4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.03% | 101% | 101% |
| Class A2 | -0.02% | -0.06% | -0.01% | 103% | 100% |
| Class B | -0.02% | -0.02% | 0.08% | 100% | 100% |
| Class C | -0.03% | -0.07% | -0.05% | 98% | 98% |
| Class E | -0.04% | 0.02% | 0.01% | 101% | 98% |
| Overall | -0.02% | -0.01% | 0.01% | 100% | 99% |
| Class D | -0.08% | -0.03% | -0.10% | 101% | 104% |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.16% | 0.11% | 100% | 99% |
| Class C | 0.02% | 0.19% | 0.24% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | -0.13% | -0.37% | 97% | 95% |
| Class F |  |  |  |  |  |

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, January 2019.

Patent rights declaration(s)

Kyung Hee university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).