| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0216-v1 |
| --- | --- |

| Title: | Non-CE3: Determination of wide-angle mode using the size of a coding block |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Jason Jung, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 dongcheol.Kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

In this contribution, when an intra coding block is partitioned into multiple transform blocks, it is proposed to determine wide-angle mode of the transform blocks based on the size of the coding block instead of the size of each transform block. The wide-angle modes are determined based on the block’s ratio of width and height. There are two cases where a coding block is further partitioned into multiple transform blocks. In ISP mode, the size of coding block is used at each transform block for wide-angle mode decision. However, in large coding blocks exceeding maxTbsize, the size of transform block is used for wide-angle mode decision. It is reportedly shown that the proposed method that consistently uses a coding block size provides overall BD-rate of 0.00%, 0.00%, 0.00% with 100% overall encoding time and xx% overall decoding time under Random Access (RA) configuration and 0.00%, -0.02%, -0.02% with 100% overall encoding time and xx% overall decoding time under Low Delay B (LDB) configuration.

Proposed Methods

In VVC-4.0 [1], if a coding block’s cbWidth and/or cbHeight is greater than the maximum transform block size (maxTbsize), the coding block is implicitly partitioned into multiple TUs. In that case, the coding block’s intra prediction mode is also applied to multiple TUs. Also, a luma coding block using Intra SubPartitions (ISP) can have 2 or 4 sub-TUs while the single intra prediction mode is applied to the TUs. Figure 1 shows examples of a CU implicitly partitioned into multiple TUs with or without ISP mode.

In the current VVC spec [1], a non-square intra block can use wide-angle modes depending on the ratio of its width and height. Then predModeIntra is set differently if a block uses one of wide-angle modes. There are two non-square block sizes of 128 x 64 and 64 x 128 where width and/or height is greater than maxTbsize. If the block is implicitly partitioned into TUs, it will result in two square TUs. In this case, it is determined using TU’s width and TU’s height whether a wide-angle mode can be used. Therefore, although the coding block is non-square and can use wide-angle modes, the partitioned TU block cannot use the wide-angle mode. On the contrary, for a coding block using ISP mode, the width/height of the coding block is used to determine the wide-angle modes of its sub-TU blocks. As a result, wide-angle modes can be used in ISP even if the partitioned TU is square block.

Figure 1. Large blocks with multiple TUs

For the consistency of applying the wide-angle modes for TUs between cases where a CU with height or width greater than maxTbsize and a CU using ISP, it is proposed to determine the wide-angle mode of TUs based on the CU’s width and height.

Experimental Results

The proposed method to consistently determine the wide-angle mode based on CU size is implemented on top of VTM-4.0 [2] with the modified encoder RDO option and tested under the common test condition (CTC) [3]. The encoder of VTM-4.0 does not enable the RDO check of intra prediction for large blocks with greater than 4096 samples. Therefore, the RDO logic is modified to enable intra prediction check for those large blocks and used the setting as Anchor in the simulation. Detailed experimental results are shown in Table 1.

Anchor: VTM-4.0 with the RDO check of greater than 4096 samples for intra prediction

Test: Determination of wide-angle modes using the size of a coding block

Table 1 – Comparison of the proposed method over Anchor

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with the RDO check of greater than 4096 samples |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.00% | 99% | #NUM! |
| Class A2 | 0.00% | -0.03% | -0.02% | 100% | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 100% | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | #NUM! |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | #NUM! |
| Class F | 0.00% | 0.00% | 0.00% | 99% | #NUM! |

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with the RDO check of greater than 4096 samples |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.16% | 0.22% | 100% | #DIV/0! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | #DIV/0! |
| Class E | -0.01% | 0.06% | -0.01% | 100% | #DIV/0! |
| Overall | -0.01% | -0.05% | 0.09% | 100% | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | #DIV/0! |
| Class F | -0.07% | -0.36% | -0.31% | 101% | #DIV/0! |

Conclusion

In this contribution, it is proposed to determine wide-angle mode using the size of a coding block at a transform block level instead of the size of the transform block itself. It is reportedly shown that the method provides overall BD-rate of 0.00%, 0.00%, 0.00% with 100% overall encoding time and xx% overall decoding time under Random Access (RA) configuration and 0.00%, -0.02%, -0.02% with 100% overall encoding time and xx% overall decoding time under Low Delay B (LDB) configuration. Therefore, it is recommended to adopt the proposed method to the next version of VVC specification.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET Common Test Conditions and Software Reference. Configurations for SDR Video,” JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).