| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0217 |
| --- | --- |

| Title: | CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Philipp Helle, Tobias Hinz, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports on the core experiment tests CE3-4.1 and CE3-4.2, both treating affine linear weighted intra prediction modes. These modes generate the luma intra prediction signal out of one line of reference samples left and above a current block by a matrix vector multiplication and the addition of an offset. 	
In the first test, at most four multiplications per sample are needed and the total memory requirement is 7.92 Kilobyte of memory. The reported results are -0.79% luma BD-rate-gain for the AI and -0.42% luma BD-rate-gain for the RA configuration. The encoding time overheads are 138% and 104%. The decoding time is  99%.
In the second test, at most eight multiplications per sample are needed and the total memory requirement is 7.20 Kilobyte of memory. In this test, all proposed intra prediction modes use the same set of matrices. The reported results of the second test are -0.82% luma BD-rate-gain for the AI and -0.44% luma BD-rate-gain for the RA configuration. The encoding time overheads are 141% and 105%. The decoding time is  99% and 101%.
For both tests, also encoder speedups are presented. Here, for the first test, -0.5% luma BD-rate gain with an encoding time overhead of 110% are reported for AI. For the second test, -0.54% luma BD-rate gain with an encoding time overhead of 112% are reported for AI.

# Test CE3-4.1: Affine linear weighted intra prediction with a memory restriction of 8 Kilobyte and at most 4 multiplications per sample

The affine linear weighted intra prediction (ALWIP) modes tested in CE3-4.1 are the same as proposed in JVET-M0043 [1] under test CE3-1.2.2, except for the following changes:

- Number of ALWIP modes is reduced from 35 to 19 for all blocks W×H where maxW,H=8.

- Number of ALWIP modes is reduced from 35 to 11 for all blocks W×H where maxW,H>8.

- Linear interpolation of predicted samples is carried out in a single step per direction replacing iterative interpolation.

Additionally, two versions with the same syntax as in test CE3-4.1 but a faster encoder were also tested.

## Description of the method

For predicting the samples of a rectangular block of width W and height H, affine linear weighted intra prediction (ALWIP) takes one line of H reconstructed neighbouring boundary samples left of the block and one line of W reconstructed neighbouring boundary samples above the block as input. If the reconstructed samples are unavailable, they are generated as it is done in the conventional intra prediction. 	
The generation of the prediction signal is based on the following three steps:

- Out of the boundary samples, four samples in the case of W=H=4 and eight samples in all other cases are extracted by averaging.

- A matrix vector multiplication, followed by addition of an offset, is carried out with the averaged samples as an input. The result is a reduced prediction signal on a subsampled set of samples in the original block.

- The prediction signal at the remaining positions is generated from the prediction signal on the subsampled set by linear interpolation which is a single step linear interpolation in each direction.

The matrices and offset vectors needed to generate the prediction signal are taken from three sets S0, S1, S2 of matrices. The set S0 consists of 18 matrices A0i, i∈{0, …, 17} each of which has 16 rows and 4 columns and 18 offset vectors b0i, i∈{0, …, 17} each of size 16. Matrices and offset vectors of that set are used for blocks of size 4×4. The set S1 consists of 10 matrices A1i, i∈{0, …, 9}, each of which has 16 rows and 8 columns and 10 offset vectors  b1i, i∈{0, …, 9} each of size 16. Matrices and offset vectors of that set are used for blocks of sizes 4×8, 8×4 and 8×8. Finally, the set S2 consists of 6 matrices A2i, i∈{0, …, 5}, each of which has 64 rows and 8 columns and of 6 offset vectors  b2i, i∈{0, …, 5} of size 64. Matrices and offset vectors of that set or parts of these matrices and offset vectors are used for all other block-shapes.

The total number of multiplications needed in the computation of the matrix vector product is always smaller than or equal to 4⋅W⋅H. In other words, at most four multiplications per sample are required for the ALWIP modes.

## Averaging of the boundary

In a first step, the input boundaries bdrytop and bdryleft  are reduced to smaller boundaries bdryredtop and  bdryredleft. Here, bdryredtop and  bdryredleft both consists of 2 samples in the case of a 4×4-block and both consist of 4 samples in all other cases.

In the case of a 4×4-block, for 0≤i<2, one defines

bdryredtopi=j=01bdrytopi⋅2+j+1≫1

and defines  bdryredleft analogously.

Otherwise, if the block-width W is given as W=4⋅2k, for 0≤i<4, one defines

bdryredtopi=j=02k-1bdrytopi⋅2k+j+1≪k-1≫k

and defines bdryredleft analogously.	
The two reduced boundaries bdryredtop and bdryredleft are concatenated to a reduced boundary vector bdryred which is thus of size four for blocks of shape 4×4  and of size eight for blocks of all other shapes. If mode  refers to the ALWIP-mode, this concatenation is defined as follows:

bdryred=bdryredtop , bdryredleftfor W=H=4 and mode<18 bdryredleft, bdryredtop for W=H=4 and mode≥18bdryredtop , bdryredleftformaxW,H=8 and mode<10 bdryredleft, bdryredtop formaxW,H=8 and mode≥10bdryredtop , bdryredleftformaxW,H>8 and mode<6 bdryredleft, bdryredtop formaxW,H>8 and mode≥6.

Finally, for the interpolation of the subsampled prediction signal, on large blocks a second version of the averaged boundary is needed. Namely, if minW,H>8  and W≥H, one writes W=8*2l, and, for 0≤i<8, defines

bdryredIItopi=j=02l-1bdrytopi⋅2l+j+1≪l-1≫l.

If minW,H>8  and H>W, one defines bdryredIIleft analogously.

## Generation of the reduced prediction signal by matrix vector multiplication

Out of the reduced input vector bdryred one generates a reduced prediction signal predred. The latter signal is a signal on the downsampled block of width Wred and height Hred. Here, Wred and Hred are defined as:

Wred=4for maxW, H≤8min(W,8)for maxW, H>8

Hred=4for maxW, H≤8min(H,8)for maxW, H>8

The reduced prediction signal predred is computed by calculating a matrix vector product and adding an offset:

predred=A∙bdryred+b.

Here, A is a matrix that has Wred⋅ Hred rows and 4 columns if W=H=4 and 8 columns in all other cases. b is a vector of size Wred⋅ Hred.

The matrix A and the vector b are taken from one of the sets S0, S1, S2 as follows. One defines an index idx=idxW,H as follows:

idxW,H=0for W=H=41for maxW,H=82formaxW,H>8.

Moreover, one puts m as follows:

m=modefor W=H=4 and mode<18mode-17for W=H=4 and mode≥18modeformaxW,H=8 and mode<10mode-9formaxW,H=8 and mode≥10modeformaxW,H>8 and mode<6mode-5formaxW,H>8 and mode≥6.

Then, if idx≤1  or idx=2 and minW,H>4, one puts A=Aidxm and b=bidxm. In the case that  idx=2 and minW,H=4, one lets A be the matrix that arises by leaving out every row of Aidxm that, in the case W=4, corresponds to an odd x-coordinate in the downsampled block, or, in the case H=4, corresponds	 to an odd y-coordinate in the downsampled block.

Finally, the reduced prediction signal is replaced by its transpose in the following cases:

- W=H=4 and mode≥18

- maxW,H=8 and mode≥10

- maxW,H>8 and mode≥6

The number of multiplications required for calculation of predred  is 4 in the case of W=H=4 since in this case A has 4 columns and 16 rows. In all other cases, A has 8 columns and Wred⋅Hred rows and one immediately verifies that in these cases 8⋅Wred⋅Hred≤4⋅W⋅H multiplications are required, i.e. also in these cases, at most 4 multiplications per sample are needed to compute predred.

## Illustration of the entire ALWIP process

The entire process of averaging, matrix vector multiplication and linear interpolation is illustrated for different shapes in Figs. 1-4. Note, that the remaining shapes are treated as in one of the depicted cases.

- Given a 4×4  block, ALWIP takes two averages along each axis of the boundary. The resulting four input samples enter the matrix vector multiplication. The matrices are taken from the set S0. After adding an offset, this yields the 16 final prediction samples. Linear interpolation is not necessary for generating the prediction signal. Thus, a total of (4⋅16)/(4⋅4)=4 multiplications per sample are performed.

Figure 1: Illustration of ALWIP for 4×4 blocks

- Given an 8×8  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix vector multiplication. The matrices are taken from the set S1. This yields 16 samples on the odd positions of the prediction block. Thus, a total of (8⋅16)/(8⋅8)=2 multiplications per sample are performed. After adding an offset, these samples are interpolated vertically by using the reduced top boundary. Horizontal interpolation follows by using the original left boundary. The interpolation process does not require any multiplications in this case as pointed out in Section 1.6. Thus, a total of 2 multiplications per sample is required to calculate ALWIP prediction.

Figure 2: Illustration of ALWIP for 8×8 blocks

- Given an 8×4  block, ALWIP takes four averages along the horizontal axis of the boundary and the four original boundary values on the left boundary. The resulting eight input samples enter the matrix vector multiplication. The matrices are taken from the set S1. This yields 16 samples on the odd horizontal and each vertical positions of the prediction block. Thus, a total of (8⋅16)/(8⋅4)=4 multiplications per sample are performed. After adding an offset, these samples are interpolated horizontally by using the original left boundary. The interpolation process, in this case, does not add any multiplications as shown in Section 1.6. Thus, a total of 4 multiplications per sample are required to calculate ALWIP prediction.

Figure 3: Illustration of ALWIP for 8×4 blocks

The transposed case is treated accordingly.

- Given a 16×16  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix vector multiplication. The matrices are taken from the set S2. This yields 64 samples on the odd positions of the prediction block. Thus, a total of (8⋅64)/(16⋅16)=2 multiplications per sample are performed. After adding an offset, these samples are interpolated vertically by using eight averages of the top boundary. Horizontal interpolation follows by using the original left boundary. The interpolation process, in this case, does not add any multiplications as shown in Section 1.6. Therefore, totally, two multiplications per sample are required to calculate ALWIP prediction.

Figure 4: Illustration of ALWIP for 16×16 blocks

For larger shapes, the procedure is essentially the same and it is easy to check that the number of multiplications per sample is less than four.

For W×8 blocks with W>8, only horizontal interpolation is necessary as the samples are given at the odd horizontal and each vertical positions. In this case, (8⋅64)/(W⋅8)=64W multiplications per sample are performed to calculate the reduced prediction. In Section 1.6, it is pointed out that for W=16, in this case, no additional multiplications are required for linear interpolation. For W>16, the number of additional multiplications per sample required for linear interpolation is less than two. Thus, total number of multiplications per sample is less than or equal to four.

Finally for W×4 blocks with W>8, let Akbe the matrix that arises by leaving out every row that corresponds to an odd entry along the horizontal axis of the downsampled block. Thus, the output size is 32 and again, only horizontal interpolation remains to be performed. For calculation of reduced prediction, (8⋅32)/(W⋅4)=64W multiplications per sample are performed. For W=16, no additional multiplications are required while, for W>16, less than 2 multiplication per sample are needed for linear interpolation (see Section 1.6). Thus, total number of multiplications is less than or equal to four.

The transposed cases are treated accordingly.

## Single step linear interpolation

For a W×H block with maxW,H≥8, the prediction signal arises from the reduced prediction signal predred on Wred×Hred by linear interpolation. Depending on the block shape, linear interpolation is done in vertical, horizontal or both directions. If linear interpolation is to be applied in both directions, it is first applied in horizontal direction if W<H and it is first applied in vertical direction, else.

Consider without loss of generality a W×H block with maxW,H≥8 and W≥H. Then, the one-dimensional linear interpolation is performed as follows. Without loss of generality, it suffices to describe linear interpolation in vertical direction.  First, the reduced prediction signal is extended to the top by the boundary signal. Define the vertical upsampling factor Uver=H/Hred and write Uver=2uver>1. Then, define the extended reduced prediction signal by

predredx-1=bdryredtopxfor W=8bdryredIItopxfor W>8.

Then, from this extended reduced prediction signal, the vertically linear interpolated prediction signal is generated by

predredups,verxUver⋅y+k=Uver-k-1⋅predredxy-1+k+1⋅predredxy+Uver2≫uver

for 0≤x<Wred, 0≤y<Hred and 0≤k<Uver.

## Complexity assessment of single step interpolation

If an upsampling factor Uver or Uhor is two, linear interpolation in the corresponding direction does not add any multiplication since the factors in the equation of the previous Section 1.5 both evaluate to 0 or 1.

Second, by this equation, in all other cases linear interpolation does not add more than two multiplications per sample to be linear interpolated.

## Complexity assessment of the whole method

The parameters needed for all possible proposed intra prediction modes are comprised by the matrices and offset vectors belonging to the sets S0, S1, S2. Here, S0, consists of 18 matrices of shape 16×4 and of 18 offset vectors each of size 16. S1 consist of 10 matrices of shape 16×8 and of 10 offset vectors each of size 16. Finally, S2 consist of 6 matrices of shape 64×8 and of 6 offset vectors of size 64. Thus, a total number of 6336 parameters is required for the proposed method. Since each parameter is stored as a 10-bit value, this corresponds to 7.92 Kilobyte of memory.

The computational complexity for the calculation of the prediction signal remains at four multiplications per sample as in [1]. This can be seen as follows.

First, if on a W×H block one has maxW,H≤16. This follows directly from Section 1.3 and the fact that, due to Section 1.6, the linear interpolation does not add any multiplication in this case.

Second, if on a W×H block one has maxW,H>16, i.e. W=32 or W=64 or H =32 or H=64, then, by Section 1.3, for such a block, the number of multiplications per sample needed to generate the reduced prediction signal is always not more than 2. On the other hand, by Section 1.6, the number of multiplications per sample needed for the linear interpolation is not more than 2. Thus, also in these cases, not more than four multiplications per sample are required to generate the full prediction signal by our method. In fact, a closer study even shows that for large blocks in general much less than four multiplications per sample are required.

## Single step linear interpolation using only bit shifts

The single step linear interpolation operation can alternatively be implemented using solely bit shifts. The result is the same as in Section 1.5. This is done in the following way. First, for the vertical linear interpolation, define scaled above and below neighbors for each interpolated sample position.

predredabovexyk=predredxy-1≪uver-predred[x][y-1]for k=0predredtopxyk-1-predred[x][y-1]for k>0

predredbelowxyk=predredx[y]for k=0predredbelowxyk-1+pred_red[x][y]for k>0

Using these scaled neighbors, the equation from Section 1.5 becomes the following:

predredupsverxy≪uver+k=predredabovexyk+predredbelowxyk+1≪uver-1≫uver

The horizontal linear interpolation can be rewritten analogously. The proposed bit-shift-only linear interpolation algorithm does not require any multiplication.

## Signalization of the proposed intra prediction modes

For each Coding Unit (CU) in intra mode, a flag indicating if an ALWIP mode is to be applied on the corresponding Prediction Unit (PU) or not is sent in the bitstream. The signalization of the latter index is harmonized with MRL in the same way as in the previous CE ([1]). If an ALWIP mode is to be applied, the index predmode of the ALWIP mode is signaled using a MPM-list with 3 MPMS. 	
Here, the derivation of the MPMs is performed using the intra-modes of the above and the left PU as follows. There are three fixed tables map_angular_to_alwipidx, idx∈{0,1,2} that assign to each conventional intra prediction mode predmodeAngular an ALWIP mode

predmodeALWIP= map_angular_to_alwipidx[predmodeAngular].

For each PU of width W and height H one defines an index

idxPU=idx(W,H)∈{0,1,2}

that indicates from which of the three sets the ALWIP-parameters are to be taken as in Section 1.3 above.

If the above Prediction Unit PUabove is available, belongs to the same CTU as the current PU and is in intra mode, if idxPU=idx(PUabove) and if ALWIP is applied on PUabove with ALWIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwipidx(PUabove)[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode modeALWIPleft.

Finally, three fixed default lists listidx, idx∈{0,1,2} are provided, each of which contains three distinct ALWIP modes. Out of the default list listidx(PU) and the modes  modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.

## Adapted MPM-list derivation for conventional luma and chroma intra-prediction modes

The proposed ALWIP-modes are harmonized with the MPM-based coding of the conventional intra-prediction modes as follows. The luma and chroma MPM-list derivation processes for the conventional intra-prediction modes uses fixed tables map_alwip_to_angularidx, idx∈{0,1,2},   mapping an ALWIP-mode predmodeALWIP on a given PU to one of the conventional intra-prediction modes

predmodeAngular=map_alwip_to_angularidx(PU)[predmodeALWIP].

For the luma MPM-list derivation, whenever a neighboring luma block is encountered which uses an ALWIP-mode predmodeALWIP, this block is treated as if it was using the conventional intra-prediction mode predmodeAngular. For the chroma MPM-list derivation, whenever the current luma block uses an LWIP-mode, the same mapping is used to translate the ALWIP-mode to a conventional intra prediction mode.

## Experimental results

Evaluation of the test was performed according to the common test conditions JVET-M1010 [2], for the intra-only (AI) and random-access (RA) configurations with the VTM software version 4.0. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result of CE3-4.1 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.19% | -0.69% | -0.68% | 138% | 98% |
| Class A2 | -0.63% | -0.09% | -0.16% | 137% | 98% |
| Class B | -0.67% | -0.16% | -0.11% | 139% | 100% |
| Class C | -0.71% | -0.21% | -0.23% | 138% | 99% |
| Class E | -0.82% | -0.30% | -0.36% | 137% | 100% |
| Overall | -0.79% | -0.27% | -0.28% | 138% | 99% |
| Class D | -0.81% | -0.34% | -0.38% | 138% | 101% |
| Class F | -0.47% | -0.28% | -0.37% | 121% | 99% |

Table 2. Result of CE3-4.1 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.65% | -1.44% | -1.42% | 105% | 98% |
| Class A2 | -0.31% | -0.10% | 0.04% | 104% | 100% |
| Class B | -0.38% | -0.57% | -0.67% | 104% | 99% |
| Class C | -0.40% | -0.48% | -0.52% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.42% | -0.63% | -0.64% | 104% | 99% |
| Class D | -0.38% | -0.68% | -0.82% | 104% | 99% |
| Class F | -0.32% | -0.40% | -0.35% | 105% | 99% |

## Additional results with further encoder speedups

We additionally provide two further results for tests that relied on the same syntax as CE3-4.1 but with an optimized encoder search. This fast encoder search can be configured by the command line parameter  FastLwip. The configuration --FastLwip=0 gives the result from Table 1. Table 3 reports the results for --FastLwip=1. The software that supports FastLwip=1 was uploaded by the CE deadline and the results were cross-checked.

Table 3. Result of CE3-4.1 for VTM AI configuration in the speedup version --FastLwip=1

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.93% | -0.60% | -0.61% | 111% | 98% |
| Class A2 | -0.38% | 0.02% | -0.08% | 110% | 98% |
| Class B | -0.38% | -0.12% | -0.08% | 110% | 99% |
| Class C | -0.39% | -0.19% | -0.19% | 108% | 100% |
| Class E | -0.32% | -0.07% | -0.18% | 108% | 99% |
| Overall | -0.47% | -0.18% | -0.21% | 110% | 99% |
| Class D | -0.49% | -0.30% | -0.34% | 109% | 100% |
| Class F | -0.23% | -0.19% | -0.26% | 106% | 99% |

Table 4 reports results for the configuration --FastLwip=2. This configuration is a further development of the configuration --FastLwip=1 and should replace the latter. The software that supports --FastLwip=2 was uploaded by the document deadline and the results were cross-checked.

Table 4. Result of CE3-4.1 for VTM AI configuration in the speedup version --FastLwip=2

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.95% | -0.61% | -0.67% | 112% | 99% |
| Class A2 | -0.41% | 0.02% | -0.08% | 110% | 99% |
| Class B | -0.39% | -0.11% | -0.12% | 111% | 100% |
| Class C | -0.38% | -0.27% | -0.17% | 109% | 101% |
| Class E | -0.46% | -0.13% | -0.19% | 109% | 102% |
| Overall | -0.50% | -0.21% | -0.23% | 110% | 100% |
| Class D | -0.49% | -0.16% | -0.31% | 110% | 103% |
| Class F | -0.23% | -0.12% | -0.21% | 107% | 102% |

# Test CE3-4.2: Architectural variations of the affine linear weighted intra prediction complying with the 8 Kilobyte memory restriction

The technique tested in CE3-4.2 is related to “Affine Linear Intra Predictions” described in JVET-M0043 [1] under test CE3-1.2.2 and similar to the technique described in Section 1 above, but adds simplifications in terms of memory requirement and model architecture.

- Single set of matrices and offset vectors for all block shapes.

- Reduction of number of modes to 19 for all block shapes.

- Reduction of memory requirement to 5760 10-bit values, that is 7.20 Kilobyte.

- Linear interpolation of predicted samples is carried out in a single step per direction replacing iterative interpolation as in the first test.

Additionally, two versions with the same syntax as in test CE3-4.2 but a faster encoder were also tested.

## Description of the method

The method of affine linear intra prediction (ALWIP) is basically the same as described in Section 1.1 above.

The matrices and offset vectors needed to generate the prediction signal are taken from a single set S which consists of 10 matrices Ai,  i∈0,…, 9 each of which has 64 rows and 8 columns and 10 offset vectors bi,  i∈{0,…,9} each of size 64. Matrices and offset vectors of that set or parts of these matrices and offset vectors are used for blocks of all shapes

The total number of multiplications needed in the computation of the matrix-vector product is always smaller than or equal to 8⋅W⋅H. Moreover, the averaging operations on the boundary and the linear interpolation of the reduced prediction signal can be carried out by solely using additions and bit-shifts. In other words, at most eight multiplications per sample are needed for the ALWIP modes.

## Averaging of the boundary

Boundary averaging process is similar to the on described in Section 1.2. In a first step, the input boundaries bdrytop and bdryleft  are reduced to smaller boundaries bdryredtop and  bdryredleft. Here, bdryredtop and  bdryredleft both consist of 4 samples. Thus, in the case of W=4, no boundary averaging is necessary in horizontal direction while, in the case of H=4, no boundary averaging is needed in vertical direction.

For all block shapes, if the block width W is given as W=4⋅2k, for 0≤i<4, one defines

bdryredtopi=j=02k-1bdrytopi⋅2k+j+1≪k-1≫kfor W>4bndrytopifor W=4

and defines bdryredleft analogously.

The two reduced boundaries bdryredtop and bdryredleft are then concatenated to a reduced boundary vector bdryred which is thus of size eight for all block shapes. Here, if mode<10, one puts

bdryred=bdryredtop , bdryredleft.

If mode≥10, which corresponds to the transposed mode of mode-9, one puts

bdryred= bdryredleft, bdryredtop .

Finally, as in Section 1.2, for the interpolation of the subsampled prediction signal on large blocks, a second version of the averaged boundary is needed. Namely, if minW,H>8  and W≥H, one writes W=8⋅2l and, for 0≤i<8, defines

bdryredIItopi=j=02l-1bdrytopi⋅2l+j+1≪l-1≫l.

If minW,H>8  and H>W, one defines bdryredIIleft analogously.

## Generation of the reduced prediction signal by matrix-vector-multiplication

Out of the reduced input vector bdryred one generates a reduced prediction signal predred. The latter signal is a signal on the downsampled block of width

Wred=min(W,8)

and height

Hred=minH,8.

The reduced prediction signal predred is computed by calculating a matrix vector product and adding an offset:

predred=A∙bdryred+b.

Here, A is a matrix that has Wred⋅ Hred rows and 8 columns and b is a vector of size Wred⋅Hred.

The matrix A and the vector b are taken from the set S as follows. First, one puts m as follows:

m=modefor mode<10mode-9for mode≥10.

Here, mode refers to the ALWIP mode. Then, if minW,H>4, one puts A=Am and b=bm. In the case that minW,H=4, one lets A be the matrix that arises by leaving out every row of Aidxm that, in the case W=4, corresponds to an odd x-coordinate in the downsampled block, or, in the case H=4, corresponds to an odd y-coordinate in the downsampled block.

If mode≥10, one replaces the reduced prediction signal by its transpose.

In all cases, A has 8 columns and Wred⋅Hred rows and one immediately verifies that 8⋅Wred⋅Hred≤8⋅W⋅H multiplications are required, i.e. at most 8 multiplications per sample are needed to compute predred.

## Linear interpolation to generate the final prediction signal

For maxW,H>8, matrix vector multiplication described in the previous section generated only a subset predred of the samples to predict. Therefore, in these cases, samples at the sample positions that were left out in the generation process of predred are linearly interpolated from predred. This linear interpolation is not needed if maxW,H≤8. The interpolation process itself is the same process as described in 1.4.

## Illustration of the entire ALWIP process

The entire process of averaging, matrix vector multiplication and linear interpolation is illustrated for different shapes in Figs. 5-8. Note, that the remaining shapes are treated as in one of the depicted cases.

- Given a 4×4  block, ALWIP uses the original eight boundary samples as input for the matrix vector multiplication. During the multiplication every matrix row corresponding to an odd x-coordinate or y-coordinate in the resulting block is skipped. After adding an offset, this yields the 16 final prediction samples. Linear interpolation is not necessary for generating the prediction signal. Thus, a total of (8⋅16)/(4⋅4)=8 multiplications per sample are performed.

Figure 5: Illustration of ALWIP for 4×4 blocks

- Given an 8×8  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix vector multiplication. After adding an offset, this yields the final 16 prediction samples. Thus, a total of (8⋅64)/(8⋅8)=8 multiplications per sample are performed.

Figure 6: Illustration of ALWIP for 8×8 blocks

For 16×8 blocks, the same procedure is used followed by horizontal linear interpolation with an upsampling factor of two. Since this interpolation operation does not add any multiplications, the number of multiplications required reduces to (8⋅64)/(16⋅8)=4 multiplications per sample in this case.

For blocks W×8 with W≥32, the number of multiplications required for calculation of reduced prediction signal further reduces to (8⋅64)/(W⋅8)≤2 multiplications per sample. Horizontal linear interpolation with an upsampling factor greater than two increases that value by less than 2 multiplications per sample. Thus in these cases less than 4 multiplications are needed.

The transposed cases are treated accordingly.

- Given an 8×4  block, ALWIP takes four averages along the horizontal axis of the boundary and the four original boundary values on the left boundary. The resulting eight input samples enter the matrix vector multiplication. During the multiplication, matrix rows corresponding to odd rows in the resulting block are skipped. After adding an offset, this yields the final 32 prediction samples. Thus, a total of (8⋅32)/(8⋅4)=8 multiplications per sample are performed.

Figure 7: Illustration of ALWIP for 8×4 blocks

For 16×4 blocks, the same procedure is used followed by horizontal linear interpolation with an upsampling factor of two. Since this interpolation operation does not add any multiplications, the number of multiplications required reduces to (8⋅32)/(16⋅4)=4 multiplications per sample in this case.

For blocks W×4 with W≥32, the number of multiplications required for calculation of reduced prediction signal further reduces to (8⋅32)/(W⋅4)≤2 multiplications per sample. Horizontal linear interpolation with an upsampling factor greater than two increases that value by less than 2 multiplications per sample resulting in a total of less than 4 multiplications per sample required in these cases.

The transposed cases are treated accordingly.

- Given a 16×16  block, ALWIP takes four averages along each axis of the boundary. The resulting eight input samples enter the matrix vector multiplication. This yields 64 samples on the odd positions of the prediction block. After adding an offset, these samples are interpolated vertically by using eight averages of the top boundary. Horizontal interpolation follows by using the original left boundary. Since linear interpolation with an upsampling factor of two does not add any multiplications, a total of (8⋅64)/(16⋅16)=2 multiplications per sample are performed.

Figure 8: Illustration of ALWIP for 16×16 blocks

For other shapes with minW,H≥16, the procedure is essentially the same and it is easy to check that the number of multiplications per sample required for calculation of the reduced prediction signal is not more than two. Thus, the number of multiplications per sample required for calculating ALWIP prediction will be less than 4 in these cases.

The transposed cases are treated accordingly.

## Complexity assessment

The parameters needed for all possible proposed intra prediction modes are comprised by the matrices and offset vectors belonging to set S. All matrix coefficients and offset vectors are stored as 10-bit values. Thus, according to the above description, a total number of 5760 parameters, each in 10-bit precision, is needed for the proposed method. This corresponds to 7.20 Kilobyte of memory.

Computational complexity for calculation of reduced prediction signal is 8 multiplications per sample of reduced prediction block as pointed out in Section 2.3. Linear interpolation does not increase this value.

## Signalization of the proposed intra prediction modes

Signalization process basically matches the one described in 1.7 above. For generation of the MPM list, there are three fixed tables map_angular_to_alwipidxmap, idxmap∈{0,1,2} that assign to each conventional intra prediction mode predmodeAngular an ALWIP mode

predmodeALWIP= map_angular_to_alwipidxmap[predmodeAngular].

For each PU of width W and height H one defines an index

idxmapPU=idxmapW,H=0for W=H=41formaxW,H=82formaxW,H>8

that indicates from which of the three sets the ALWIP-parameters are to be taken.

If the above Prediction Unit PUabove is available, belongs to the same CTU as the current PU and is in intra mode, if idxPU=idx(PUabove) and if ALWIP is applied on PUabove with ALWIP-mode predmodeALWIPabove, one puts

modeALWIPabove=predmodeALWIPabove .

If the above PU is available, belongs to the same CTU as the current PU and is in intra mode and if a conventional intra prediction mode predmodeAngularabove is applied on the above PU, one puts

modeALWIPabove=map_angular_to_alwipidxmap(PUabove)[predmodeAngularabove].

In all other cases, one puts

modeALWIPabove=-1

which means that this mode is unavailable. In the same way but without the restriction that the left PU needs to belong to the same CTU as the current PU, one derives a mode modeALWIPleft.

Finally, seven fixed default lists listidxMPM, idxMPM∈{0, …, 6} are provided, each of which contains three distinct ALWIP modes. The index deciding which list to use for a PU is defined as follows.

idxMPMPU=idxMPMW,H=0for W=H=43for W=4 and H=85for W=8 and H=41for W=H=84forminW,H>8 and W<H6forminW,H>8 and W>H2forminW,H>8 and W=H

Out of the default list listidxMPM(PU) and the modes modeALWIPabove and modeALWIPleft, one constructs three distinct MPMs by substituting -1 by default values as well as eliminating repetitions.

## Adapted MPM-list derivation for conventional luma and chroma intra-prediction modes

The luma and chroma MPM-list derivation processes for the conventional intra-prediction modes uses fixed tables map_alwip_to_angularidxmap, idx∈{0, 1, 2},   mapping an ALWIP-mode predmodeALWIP on a given PU to one of the conventional intra-prediction modes

predmodeAngular=map_alwip_to_angularidxmap(PU)[predmodeALWIP]

as described in Section 1.10 above. The indices deciding which table to use are derived as in Section 2.7 above.

## Experimental results

Evaluation of the test was performed according to the common test conditions JVET-L1010 [2], for the intra-only (AI) and random-access (RA) configurations with the VTM software version 4.0. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 5. Result of CE3-4.2 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1.15% | -0.60% | -0.64% | 141% | 97% |
| Class A2 | -0.67% | -0.07% | -0.18% | 139% | 98% |
| Class B | -0.73% | -0.12% | -0.21% | 143% | 100% |
| Class C | -0.72% | -0.25% | -0.25% | 141% | 100% |
| Class E | -0.90% | -0.25% | -0.39% | 140% | 99% |
| Overall | -0.82% | -0.24% | -0.32% | 141% | 99% |
| Class D | -0.81% | -0.20% | -0.44% | 141% | 101% |
| Class F | -0.53% | -0.24% | -0.32% | 124% | 100% |

Table 6. Result of CE3-4.2 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.65% | -1.28% | -1.42% | 106% | 99% |
| Class A2 | -0.34% | -0.25% | -0.06% | 105% | 101% |
| Class B | -0.40% | -0.42% | -0.58% | 105% | 101% |
| Class C | -0.39% | -0.34% | -0.51% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.54% | -0.63% | 105% | 101% |
| Class D | -0.42% | -0.71% | -0.43% | 105% | 100% |
| Class F | -0.37% | -0.43% | -0.33% | 106% | 100% |

## Additional results with further encoder speedups

We additionally provide further results for tests that relied on the same syntax as CE3-4.2 but with an optimized encoder search. This fast encoder search can be configured by the command line parameter  FastLwip. The configuration --FastLwip=0 gives the result from Table 5. Table 7 reports the results for --FastLwip=1. The software that supports FastLwip=1 was uploaded by the CE deadline and the results were cross-checked.

Table 7. Result of CE3-4.2 for VTM AI configuration in the speedup version --FastLwip=1

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.90% | -0.63% | -0.56% | 113% | 97% |
| Class A2 | -0.42% | 0.02% | -0.09% | 113% | 99% |
| Class B | -0.45% | -0.17% | -0.08% | 113% | 100% |
| Class C | -0.40% | -0.28% | -0.19% | 111% | 100% |
| Class E | -0.44% | -0.16% | -0.29% | 110% | 103% |
| Overall | -0.51% | -0.24% | -0.22% | 112% | 100% |
| Class D | -0.49% | -0.26% | -0.32% | 112% | 101% |
| Class F | -0.24% | -0.17% | -0.19% | 109% | 100% |

Table 8 reports results for the configuration --FastLwip=2. This configuration is a further development of the configuration --FastLwip=1 and should replace the latter. The software that supports --FastLwip=2 was uploaded by the document deadline and the results were cross-checked.

Table 8. Result of CE3-4.2 for VTM AI configuration in the speedup version --FastLwip=2

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.92% | -0.51% | -0.52% | 114% | 97% |
| Class A2 | -0.46% | -0.06% | -0.08% | 112% | 98% |
| Class B | -0.46% | -0.13% | -0.13% | 113% | 100% |
| Class C | -0.42% | -0.26% | -0.17% | 110% | 101% |
| Class E | -0.55% | -0.19% | -0.26% | 111% | 100% |
| Overall | -0.54% | -0.22% | -0.22% | 112% | 99% |
| Class D | -0.50% | -0.25% | -0.31% | 111% | 102% |
| Class F | -0.27% | -0.21% | -0.24% | 108% | 100% |

# Cross-check

Tests CE3-4.1 and CE3-4.2 as well as the speedup --FastLwip=1 for both tests were cross-checked by Vasily Rufitskiy (Huawei). Tests CE3-4.1 and CE3-4.2 with the speedup --FastLwip=2 for both tests were cross-checked by Fabien Racape (Technicolor).

# Conclusion

The proposed affine linear intra prediction modes of test CE3-4.1 lead to coding gains of -0.79% resp. -0.42% in the AI resp. RA configuration while the proposed affine linear intra prediction modes of test CE3-4.2 lead to coding gains of -0.82% resp. -0.44% in the AI resp. RA configuration.

# References

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (test 1.2.1, test 1.2.2)”, JVET-M0043, Marrakech, Morocco, January 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, Morocco, January 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).