| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0219-v1 |
| --- | --- |

| Title: | Coding of 360° video in HEC using different numbers of tiles per face |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Johannes Sauer Max Bläser | Tel: Email: | +49241-80-27678 sauer@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document describes the impact of tiles on the coding of 360° video. For the anchor VTM 4.0.1 with 360Lib 9.0 was used and configured to use a single tile for the whole frame / all faces. Different number of tiles for a single cube face were used for comparison.

The BoG on coding of 360° video recommends disabling of loop filters at face boundaries. This can be achieved by configuring each face to be in a different tile group and setting loop_filter_across_tile_groups_enabled_flag to zero. Another approach is to implement a high level signaling of sample rows and columns at which loop filters shall be disabled.

The BD-Rate changes for different numbers of tiles per face, reported in this document, may be helpful for the ongoing discussion on this.

# Impact of the number of tiles per face for the coding of 360° video

VTM 4.0.1 with 360 Lib 9.0 was used for all simulations. The projection format was HEC. The simulations were done according to the CTC for coding of 360° video. The anchor was configured to use a single tile for the whole frame. The other configuration were using 1, 4, 9 and 16 tiles per face. It should be noted that for 9 tiles per face a face cannot be partitioned into exact 3x3 tiles, since the face resolution according to the CTC is 1280.

|  | End-to-end WS-PSNR |  |  | Tiles per face | Total tiles |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  |
| 1 tile per face | 0.33% | 0.12% | -0.13% | 1 | 6 |
| 4 tile per face | 1.68% | 1.22% | 0.67% | 4 | 24 |
| 9 tile per face | 3.05% | 2.49% | 1.79% | 9 | 54 |
| 16 tile per face | 4.52% | 4.05% | 3.34% | 16 | 96 |

# Patent rights declaration(s)

RWTH Aachen University does not have any current or pending patent rights relating to the technology described in this contribution.