| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0221 |
| --- | --- |

| Title: | Chroma Quantization Parameter QpC Table for HDR Signal |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu, fpu}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes that a chroma quantization parameter QpC table for HDR content be added to VVC to complement the existing chroma quantization parameter QpC table for SDR content. The values in the proposed HDR Qpc table are derived from subjective comparison to HDR CTC Anchor.

# Introduction

SDR and HDR are commercially deployed and will coexist for a long time. SDR content is typically coded as NCL Y'CbCr gamma in a BT. 709 or BT. 2020 container.  HDR content is typically coded as NCL Y'CbCr PQ, ICtCp PQ, or NCL Y'CbCr HLG in a BT.2020/BT. 2100 container.  In the current VVC specification, only one chroma quantization parameter (QpC) table is specified [1] (Table 8-12, reproduced below). The table was inherited from HEVC and designed only for SDR content. When HDR PQ content was first studied in JCT-VC, it was concluded that the default  QpC table results in chroma artefacts at low bitrate, especially in achromatic regions [2][3]. Consequently, encoder optimization using chromaQPOffset is suggested in HDR CTC [4] for HDR PQ content.

Table 8-12 – Specification of QpC as a function of qPi for ChromaArrayType equal to 1

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

VVC is a newly developing standard that will code both SDR and HDR content. Adding an HDR chroma quantization parameter QpC table in VVC would unify coding for all SDR and HDR content types and eliminate the need for different encoder optimizations for different content types.

This contribution proposes that an HDR chroma quantization parameter QpC table be included in VVC in addition to the existing SDR chroma quantization parameter QpC table.

# Proposal

## Signal characteristics

Common signal formats for SDR and HDR content are listed in Table 1 using the colour description syntax in HEVC specification.

Table 1 -- Common signal formats for SDR and HDR

| name | colour_primaries (for container) | transfer_characteristics | matrix_coeffs |
| --- | --- | --- | --- |
| SDR NCL Y'CbCr | BT.709(1)/BT. 2020(9) | gamma (1, 14, 15) | YCbCr 709 (1)/ YCbCr 2020 (9) |
| HDR NCL Y'CbCr PQ | BT. 2020/2100 (9) | PQ (16) | Y'CbCr 2020/2100 (9) |
| HDR NCL ICtCp PQ | BT. 2020/2100 (9) | PQ (16) | ICtCp (14) |
| HDR NCLY'CbCr HLG | BT. 2020/2100 (9) | HLG (18) | Y'CbCr (9) |

SDR CTC includes BT. 709 and BT. 2020 content. The default QpC table is used and no severe chroma colour artefact has been reported, which indicates that the default QpC table provides acceptable performance for both BT. 709 and BT. 2020 SDR content.

HDR CTC contains HDR NCL Y'CbCr PQ and HDR Y'CbCr HLG content. For PQ content, encoder-only modification with chromaQPOffset is used to reduce chroma artefact for low bitrate. For HLG content, the same setting as SDR is used. For ICtCp PQ content, the experimental results indicate that adding a positive constant value 6 to the value of chromaQPOffset used for Y’CbCr PQ provides acceptable performance.

We propose to add a QpC table for HDR NCL Y’CbCr PQ content to avoid the need for special encoder-only modification for HDR NCL Y'CbCr PQ content, and to unify the coding of all SDR and HDR signal formats.

## Design philosophy

The QpC table for HDR NCL Y’CbCr PQ content is derived from the encoder chromaQPOffset setting in HDR PQ CTC Anchor.

In HDR PQ CTC, a model is used that assigns Cb and Cr QP offsets based on the luma QP and a factor based on the capture colour space and the representation colour space [3][5]. The motivation is to do no adjustment, or only minor adjustment, for small values of QP when compression quality is good (visually lossless or near lossless); but do stronger adjustment for large QP when colour artefacts are prominent. The model is expressed as Equ.1

CbQpOffset = Clip3(-12, 0, round(c_cb*(k*QP+l)))

CrQpOffset = Clip3(-12, 0, round(c_cr*(k*QP+l))),       (Equ.1)

where k = -0.46, l = 9.26, and the values of c_cb and c_cr are shown in Table 2 for different combinations of capture colour space and representation colour spaces.

Table 2 -- Colour-space dependent parameters in Equ.1

| colour space |  | parameter |  |
| --- | --- | --- | --- |
| capture | representation | c_cb | c_cr |
| BT. 2020/BT.2100 | BT. 2020/BT.2100 | 1 | 1 |
| P3D65 |  | 1.04 | 1.39 |
| BT. 709 |  | 1.14 | 1.78 |

Equ. 1 generates different tables for Cb and Cr signals for each combination of capture and representation colour spaces as illustrated in Table 3 and Figure 1.

A single QpC table is preferable to multiple tables, which could be more confusing for users. Thus, the following approach to produce a single table is proposed:

- Compute CbQPOffset and CrQPOffset following Equ. 1 for both BT. 709 and P3D65 captured colour space cases.

- Compute quantization parameters QpC_Cb and QpC_Cr

- Generate final default QpC table: QpC = f(QpC_Cb_709, QpC_Cr_709, QpC_Cb_P3, QpC_Cr_P3), where f(.) is a function. f(.) could be any one of the following: select one (e.g., p3); average; or minimum of all QpC values.

Table 3 -- QpC Table for Cb and Cr in 709 or P3 colour space

| qPi | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC(709Cb) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 21 |  |
| QpC(709Cr) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 20 | 20 |  |
| QpC(P3Cb) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 21 |  |
| QpC(P3Cr) | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 20 | 21 |  |

| qPi | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC(709Cb) | 21 | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 29 | 29 | 30 | 30 |
| QpC(709Cr) | 21 | 21 | 21 | 21 | 21 | 22 | 22 | 22 | 22 | 22 | 22 | 23 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 29 | 30 |
| QpC(P3Cb) | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 29 | 29 | 30 | 31 | 31 |
| QpC(P3Cr) | 21 | 22 | 22 | 22 | 23 | 23 | 23 | 24 | 24 | 24 | 25 | 25 | 25 | 26 | 26 | 27 | 27 | 28 | 29 | 29 | 30 |

| qPi | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC(709Cb) | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |
| QpC(709Cr) | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |
| QpC(P3Cb) | 32 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |
| QpC(P3Cr) | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |

Figure 1 -- relationship between qPi and derived QpC (YCbCr case)

Note that it is not possible to match the exact results in HDR PQ CTC by using only one of the candidate QpC tables for the following reasons:

- ChromaQPOffset has different values for Cb, Cr and for different combinations of capture and representation colour spaces. (For example, if qPi = 37 then QpC_709_Cb = 28, QpC_709_Cr = 25, QpC_P3_Cb = 29, and QpC_P3_Cr = 26.)

- ChromaQPOffset is derived in PPS, which means that a constant value is used based on the input QP. The input QP may not be the same as the slice QP used in the codec. In RA CTC, there is QP modulation (JCTVC-X0038) based on hierarchical structure. (For example, when input QP is 37 then chromaQPOffset_709_Cr = -12 while the QP used in the corresponding I slice would be 34. Thus, the QP_Cr for I slice would be 34-12=22; but, if we were to use QpC_709_Cr mapping, then the QP_Cr for I slice would be 23.)

- In HDR CTC, the luma component uses the lumaDQP algorithm, which means QP for CTU would also be adjusted by average CTU luma value as explained in 2). This will cause further difference between the Anchor CTC results and the QpC table results.

## Syntax

In SPS, we add a new syntax element chroma_qp_table_idc. When the value of chroma_qp_table_idc is equal to 0, it refers to the original SDR QpC table in the VVC specification text. When the value of chroma_qp_table_idc is equal to 1, it refers to the new HDR QpC table proposed in this contribution.

chroma_qp_table_idc is an indication of an index to the array of QpC as a function of qPi for ChromaArrayType equal to 1. The value of chroma_qp_table_idc shall be in the range of 0 to 1, inclusive.

# Simulation results

We relied on subjective evaluations to choose the proposed QpC table for PQ Y’CbCr. Our subjective evaluation consisted of comparing HDR PQ Anchor with our proposal on a Pulsar display using in RA CTC. The settings are shown in Table 4.

Table 4 -- Settings for subjective evaluations

|  | LumaLevelToDeltaQPMode | WCGPPSEnable | QpC table |
| --- | --- | --- | --- |
| HDR PQ CTC Anchor | 1 | 1 | SDR QpC |
| HDR PQ Proposal | 1 | 0 | PQ QpC |

We tested the candidate functions shown inf Table 5 to derive a single HDR Qpc table: QpC = f(QPcb_709, QPcr_709, QPcb_p3, QPcr_p3).

Table 5 – Candidate functions for HDR QpC Table

| Test | HDR_QPc function |
| --- | --- |
| HDR_QpC_avg | AVERGE(QpC _709_Cb, QpC _709_Cr, QpC _P3_Cb, QpC _P3_Cr) |
| HDR_QpC_709avg | AVERGE(QpC_709_Cb, QpC_709_Cr) |
| HDR_QpC_P3avg | AVERGE(QpC_P3_Cb, QpC_P3_Cr) |
| HDR_QpC_avgmin | MIN(QpC_709_Cb, QpC_709_Cr, QpC_P3_Cb, QpC_P3_Cr) |
| HDR_QpC_709min | MIN(QpC_709_Cb, QpC_709_Cr) |
| HDR_QpC_P3min | MIN(QpC_P3_Cb, QpC_P3_Cr) |

For reference, we show BD-Rate results in Table 6. In this case, both solutions are compared with HDR PQ CTC setting but with WCGPPSEnable set as disabled; that is, using SDR QpC table without further encoder chromaQPOffset optimization.

Subjective evaluation tests were done for all functions compared to HDR CTC Anchors. We observed that if BDRate of DE100 gain is greater than 30%, the subjective difference in color between the proposal and HDR CTC Anchor was observed to be very minor even at low bitrate, which indicates QpC is an acceptable alternative for encoder optimization.

Based on subjective evaluation tests and objective results, we set the following selection criteria: BDRate DE100 gain greater than 30%; and both PSNRL100 and wPSNRY loss smaller or equal to the corresponding values for the HDR CTC Anchors. The following possible candidates met the criteria: HDR_QpC_avg, HDR_QpC_709avg, and HDR_QpC_P3min.

Table 6 -- BD-rate data in RA compared to modified HDR Anchor (i.e., without encoder chromaQPOffset optimization)

| Ref | Test | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_QpC | HDR_VTM4.0_Anchor | -42.5% | 5.0% | 5.8% | -50.0% | -70.4% |
|  | HDR_QpC_avg | -30.8% | 4.2% | 4.7% | -50.3% | -50.8% |
|  | HDR_QpC_709avg | -32.4% | 4.8% | 5.4% | -52.6% | -53.5% |
|  | HDR_QpC_P3avg | -26.7% | 3.4% | 3.9% | -44.1% | -44.4% |
|  | HDR_QpC_avgmin | -33.6% | 5.8% | 6.4% | -54.8% | -55.9% |
|  | HDR_QpC_709min | -38.4% | 7.8% | 8.4% | -62.4% | -63.6% |
|  | HDR_QpC_P3min | -31.8% | 4.9% | 5.4% | -52.0% | -52.6% |

It is expected that HDR PQ content will typically be created in the P3 colour space. We therefore recommend that HDR_QpC_P3min be used to derive the QpC table for HDR PQ content (Table 7).

Table 7 -- Specification of QpC as a function of qPi for ChromaArrayType equal to 1 when chroma_qp_table_idc = 1

| qPi | < 21 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 20 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 | 24 | 24 | 24 | 25 | 25 | 25 | 26 | 26 | 27 |

| qPi | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | > 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | 27 | 28 | 29 | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 26 | 26 | 27 | = qPi − 18 |

# Conclusion

This contribution proposes that an HDR chroma quantization parameter QpC table be included in VVC in addition to the existing SDR chroma quantization parameter QpC table.

Including an HDR chroma quantization parameter QpC table in VVC would unify codec design for SDR and HDR for different colour spaces and transfer characteristics, including: Y'CbCr gamma; Y'CbCr PQ; ICtCp PQ; and Y'CbCr HLG. Including an HDR chroma quantization parameter QpC table would also eliminate the need for different encoder optimizations for different content types.

# Acknowledgements

We would like to thank Technicolor for crosschecking results.

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-M1001-v5, Marrakech, MA, 0-18 Jan. 2019.

- D. Baylon, Z. Gu, A. Luthra, K. Minoo, P. Yin, F. Pu, T. Lu, T. Chen, W. Husak, Y. He, L. Kerofsky, Y. Ye, B. Yi, “Response to Call for Evidence for HDR and WCG Video Coding: ARRIS, Dolby and InterDigital”, Doc. m36264, July 2015, Warsaw, Poland

- K.Andersson, P. Hermansson, J. Samuelsson, J. Strom, M. Petersson, “Report for CE1.a (Chroma QP)”, Doc. m37179, Oct. 2015, Geneva, Switzerland.

- A. Segall, E. François, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-L1011, Macao, CN, 3–12 Oct. 2018.

- Supplement 15 to ITU-T H-series Recommendations: “Conversion and coding practices for HDR/WCG Y’CbCr 4:2:0 video with PQ transfer characteristics”

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).