| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0223-v1 |
| --- | --- |

| Title: | CE3-related: reference sample filtering simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, jin78.heo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution first reports mismatch between VVC specification (JVET-M1001-v6) and VTM4.0 when conditions for reference sample filtering are described. This contribution further proposes simplified conditions for reference sample filtering in intra prediction. The experiment results show -0.08% and -0.04% for AI in ClassA1 and ClassA2, and overall -0.02% and xx% luma BD-rate changes for AI and RA, respectively.

Introduction

Intra reference sample filter has proven to be beneficial in coding performance especially in large intra blocks. Currently, in VVC specification JVET-M1001-v6, conditions for applying reference sample filtering are different from VTM 4.0. According to VTM 4.0, following should be applied. Note that, highlights are different from SW and specification.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- predModeIntra is not equal to INTRA_DC

- one or more of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR and nTbW*nTbH is greater than 32

- nTbW is greater than nTbH and predModeIntra is greater than or equal to 2 and predModeIntra is less than ( whRatio > 1 )  ?  ( 8 + 2 * whRatio )  :  8

- nTbH is greater than nTbW and predModeIntra is less than or equal to 66 and predModeIntra is greater than ( whRatio > 1 )  ?  ( 60 − 2 * whRatio )  :  60

- The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1. The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ). The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4. The minDistVerHor is greater than intraHorVerDistThres[ nTbS ].

- predModeIntra is equal to INTRA_ANGULAR34

- predModeIntra is equal to INTRA_ANGULAR2 and nTbH is greater than or equal to nTbW

- predModeIntra is equal to INTRA_ANGULAR66 and nTbW is greater than or equal to nTbH

- Otherwise, filterFlag is set equal to 0.

- Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 | 14 | 2 | 0 | 0 | 0 |

Proposed Method

Simplification on reference sample filtering condition is proposed to give coding benefit to 4K sequences and simplify the specification text.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- one or more of the following conditions is true:

- nTbW * nTbH is less than or equal to 128 and predModeIntra is equal to one of INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR2..4, INTRA_ANGULAR32..36, INTRA_ANGULAR64..66

- nTbW * nTbH is greater than 128 and predModeIntra is neither INTRA_ANGULAR18 nor INTRA_ANGULAR50

- Otherwise, filterFlag is set equal to 0.

Proposed Specification Text

Please see attached specification document for detail.

Experimental Results

The proposed method is implemented on top of VTM 4.0, and evaluated for VTM configuration according to the common test conditions [1].

Table 1 Experiment result on all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.05% | 101% | 99% |
| Class A2 | -0.04% | 0.07% | -0.02% | 100% | 99% |
| Class B | 0.00% | 0.03% | 0.10% | 101% | 100% |
| Class C | 0.01% | -0.03% | 0.02% | 100% | 101% |
| Class E | 0.00% | 0.05% | -0.03% | 101% | 102% |
| Overall | -0.02% | 0.02% | 0.02% | 101% | 100% |
| Class D | -0.01% | 0.00% | -0.28% | 101% | 104% |
| Class F | 0.02% | -0.04% | -0.07% | 101% | 101% |

Table 2 Experiment result on random access (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | 0.03% | -0.04% | 101% | 101% |
| Class C | 0.01% | -0.03% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.07% | -0.25% | 100% | 101% |
| Class F | -0.01% | -0.10% | -0.16% | 100% | 101% |

Summary

This contribution proposes reference sample filtering simplification in intra prediction with specification text clean-up. The experiment results show -0.08% and -0.04% for AI in ClassA1 and ClassA2, and overall -0.02% and xx% luma BD-rate changes for AI and RA, respectively.

References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).