| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0223-v2 |
| --- | --- |

| Title: | CE3-related: reference sample filtering simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, jin78.heo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution first proposes fix of mismatch between VVC specification (JVET-M1001-v6) and VTM4.0 when conditions for reference sample filtering are described. This contribution further proposes simplified conditions for reference sample filtering in intra prediction. The experiment results show -0.08% and -0.04% for AI in ClassA1 and ClassA2, and overall -0.02% and -0.02% luma BD-rate changes for AI and RA, respectively.

Introduction

Intra reference sample filter has proven to be beneficial in coding performance especially in large intra blocks. Currently, in VVC specification JVET-M1001-v6, conditions for applying reference sample filtering are different from VTM 4.0. According to VTM 4.0, following should be applied. Note that, highlights are different from SW and specification.

Proposed Specification Fix

8.4.4.2.1 General intra sample prediction

…

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

…

3.	The reference sample filtering process as specified in clause 8.4.4.2.4 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the filterFlag and reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

…

The intra sample prediction process according to predModeIntra applies as follows:

…

- Otherwise, the corresponding intra prediction mode process specified in clause 8.4.4.2.7 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the coding block width nCbW and height nCbH, the colour component index cIdx, filterFlag, and the reference sample array p as inputs, and the modified intra prediction mode predModeIntra and the predicted sample array predSamples as outputs.

8.4.4.2.4 Reference sample filtering process

…

Outputs of this process are the filterFlag and reference samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- predModeIntra is not equal to INTRA_DC

- one or more of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR and nTbW*nTbH is greater than 32

- nTbW is greater than nTbH and predModeIntra is greater than or equal to 2 and predModeIntra is less than ( whRatio > 1 )  ?  ( 8 + 2 * whRatio )  :  8

- nTbH is greater than nTbW and predModeIntra is less than or equal to 66 and predModeIntra is greater than ( whRatio > 1 )  ?  ( 60 − 2 * whRatio )  :  60

- The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1. The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ). The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4. The minDistVerHor is greater than intraHorVerDistThres[ nTbS ].

- predModeIntra is equal to INTRA_ANGULAR34

- predModeIntra is equal to INTRA_ANGULAR2 and nTbH is greater than or equal to nTbW

- predModeIntra is equal to INTRA_ANGULAR66 and nTbW is greater than or equal to nTbH

- Otherwise, filterFlag is set equal to 0.

- Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 | 14 | 2 | 0 | 0 | 0 |

8.4.4.2.7 Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

…

Inputs to this process are:

- a variable filterFlag specifying reference sampling filtering

…

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nW is greater than 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 and predModeIntra is less than INTRA_ANGULAR34 and nH is greater than 8.

- Otherwise, filterFlag is set to 1-filterFlag the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ] or wideAngle is equal to 1, filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 16 | 14 | 2 | 0 | 0 | 0 |

Proposed Reference Sample Filtering Conditions

Simplification on reference sample filtering condition is proposed to give coding benefit to 4K sequences and simplify the specification text.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- one or more of the following conditions is true:

- nTbW * nTbH is less than or equal to 128 and predModeIntra is equal to one of INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR2..4, INTRA_ANGULAR32..36, INTRA_ANGULAR64..66

- nTbW * nTbH is greater than 128 and predModeIntra is neither INTRA_ANGULAR18 nor INTRA_ANGULAR50

- Otherwise, filterFlag is set equal to 0.

Proposed Specification Text

Please see attached specification document for detail.

Experimental Results

The proposed method is implemented on top of VTM 4.0, and evaluated for VTM configuration according to the common test conditions [1].

Table 1 Experiment result on all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.05% | 101% | 99% |
| Class A2 | -0.04% | 0.07% | -0.02% | 100% | 99% |
| Class B | 0.00% | 0.03% | 0.10% | 101% | 100% |
| Class C | 0.01% | -0.03% | 0.02% | 100% | 101% |
| Class E | 0.00% | 0.05% | -0.03% | 101% | 102% |
| Overall | -0.02% | 0.02% | 0.02% | 101% | 100% |
| Class D | -0.01% | 0.00% | -0.28% | 101% | 104% |
| Class F | 0.02% | -0.04% | -0.07% | 101% | 101% |

Table 2 Experiment result on random access (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.04% | -0.06% | 101% | 101% |
| Class A2 | -0.05% | -0.07% | 0.01% | 101% | 101% |
| Class B | -0.02% | 0.03% | -0.04% | 101% | 101% |
| Class C | 0.01% | -0.03% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.03% | 101% | 101% |
| Class D | 0.00% | -0.07% | -0.25% | 100% | 101% |
| Class F | -0.01% | -0.10% | -0.16% | 100% | 101% |

Summary

This contribution proposes reference sample filtering simplification in intra prediction with specification text clean-up. The experiment results show -0.08% and -0.04% for AI in ClassA1 and ClassA2, and overall -0.02% and -0.02% luma BD-rate changes for AI and RA, respectively. It is recommended to adopt simplified reference sample filtering condition to VVC.

References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).