| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0224-v1 |
| --- | --- |

| Title: | CE3-related: ISP support for 4:2:2 and 4:4:4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Moonmo Koo Jin Heo Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, jin78.heo, moonmo.koo, jaehyun.lim, junghak.nam seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes several solutions on ISP to support 4:2:2 and 4:4:4 chroma format.

- Method 1: apply ISP to chroma CUs which corresponds to luma ISP CU with size Nx128 and 128xN in single tree with chroma format 4:2:2 and 4:4:4

- Method 2: generally apply ISP to chroma CU with NumIntraSubPartitions /2

- Method 3: generally restrict ISP block size as ( W < MaxTbSizeY && H < MaxTbSizeY ) && ( W * H> MinTbSizeY * MinTbSizeY)

The experiment results show method 2 has overall xx% and xx% and method 3 has overall 0.01% and 0.01% luma BD-rate changes for AI and RA, respectively.

Introduction

Intra sub-partition prediction (ISP) was adopted in VVC. ISP is only applied to luma and splits luma intra CU to several intra TU either horizontally or vertically. Chroma TU is not split by ISP in both dual tree and single tree case. In single tree case, when an ISP luma intra block has width equal to 64 and height equal to 128, corresponding chroma blocks in various chroma format are shown in Figure 1.

Figure 1 Luma ISP block 64x128 corresponding chroma blocks in various chroma format

In 4:2:0 format, corresponding 32x64 chroma TU block is not split. In 4:2:2 and 4:4:4 format, corresponding 32x128 and 64x128 chroma TU blocks are not split. Since the maximum TU size in VTM 4.0 is 64, in these cases, chroma TU cannot be processed. Several solutions are proposed to resolve that problem.

Proposed Method 1

In method 1, in single tree, luma ISP block of size Nx128 and 128xN (for N<=64) corresponding chroma blocks apply ISP in 4:2:2 and 4:4:4 chroma format as shown in Figure 2.

Figure 2 Luma ISP block 64x128 corresponding chroma blocks apply ISP

By applying ISP to chroma blocks in 4:2:2 and 4:4:4, chroma 32x128 and 64x128 are split into 32x32 TUs and 64x32 TUs, thus less than or equal to maximum TU size 64.

Proposed syntax changes are described as follows.

Table 1 Syntax change for proposed method 1

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if (chromaArrayType != 0) { |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( ( subTuIndex = = NumIntraSubPartitions − 1 ) ¦¦ ((CbWidth>MaxTrSize ¦¦ CbHeight>MaxTrSize) && (chromaArrayType ==2 ¦¦ chromaArrayType ==3) )) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && ( (subTuIndex = = NumIntraSubPartitions − 1 ) ¦¦ ((CbWidth>MaxTrSize ¦¦ CbHeight>MaxTrSize) && (chromaArrayType ==2 ¦¦ chromaArrayType ==3) )) ) |  |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / SubWidthC2 |  |
| hC = CbHeight[ x0 ][ y0 ] / SubHeightC2 |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

Proposed Method 2

In method 2, ISP is applied to chroma CU generally with NumIntraSubPartitions divided by 2. As shown in Figure 3, when a luma intra CU with size of 16x16 has NumIntraSubPartitions as 4, corresponding chroma CU has NumIntraSubPartitions as 2 in any chroma format. Figure 4 shows the case when luma intra CU has NumIntraSubPartitions as 2. Only luma 4x8 and 8x4 intra CU can have NumIntraSubPartitions as 2. In these case, corresponding chroma block has NumIntraSubPartitions as 1, in other word, ISP is not applied.

Figure 3 Luma ISP block 16x16 corresponding chroma blocks apply ISP

Figure 4 Luma ISP block 4x8 corresponding chroma blocks apply ISP

Table 2 Syntax change for proposed method 2

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |
| if (chromaArrayType != 0) { |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions/2-1 ¦¦ subTuIndex== NumIntraSubPartitions – 1 )) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && (subTuIndex = = NumIntraSubPartitions/2-1 ¦¦ subTuIndex = = NumIntraSubPartitions − 1)) |  |
| xC = CbPosX[ x0 ][ y0 ] |  |
| yC = CbPosY[ x0 ][ y0 ] |  |
| wC = CbWidth[ x0 ][ y0 ] / SubWidthC |  |
| hC = CbHeight[ x0 ][ y0 ] / SubHeightC |  |
| } else |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

Proposed Method 3

In method 3, ISP is only applied to luma intra block with CU size as ( W < MaxTbSizeY && H < MaxTbSizeY ) && ( W * H> MinTbSizeY * MinTbSizeY).

Proposed syntax changes are described as follow.

Table 3 Syntax change for proposed method 3

coding_unit( x0, y0, cbWidth, cbHeight, treeType ) {

if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ) {

if( treeType != DUAL_TREE_CHROMA )

cu_skip_flag[ x0 ][ y0 ]

if( cu_skip_flag[ x0 ][ y0 ] = = 0 && tile_group_type != I )

pred_mode_flag

if( ( ( tile_group_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( tile_group_type != I && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_ibc_enabled_flag )

pred_mode_ibc_flag

}

if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) {

if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY )

pcm_flag[ x0 ][ y0 ]

if( pcm_flag[ x0 ][ y0 ] ) {

while( !byte_aligned( ) )

pcm_alignment_zero_bit

pcm_sample( cbWidth, cbHeight, treeType)

} else {

if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) {

if( ( y0 % CtbSizeY ) > 0 )

intra_luma_ref_idx[ x0 ][ y0 ]

if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && (cbWidth < MaxTbSizeY && cbHeight < MaxTbSizeY cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ))

intra_subpartitions_mode_flag[ x0 ][ y0 ]

if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY )

intra_subpartitions_split_flag[ x0 ][ y0 ]

if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 )

intra_luma_mpm_flag[ x0 ][ y0 ]

if( intra_luma_mpm_flag[ x0 ][ y0 ] )

intra_luma_mpm_idx[ x0 ][ y0 ]

else

intra_luma_mpm_remainder[ x0 ][ y0 ]

}

if (chromaArrayType != 0) {

if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA )

intra_chroma_pred_mode[ x0 ][ y0 ]

}

}

…

}

…

}

Proposed Specification Text

Please see attached specification document for detail.

Experimental Results

The proposed methods are implemented on top of VTM 4.0, and evaluated for VTM configuration according to the common test conditions [1].

Table 4 Experiment result of method 3 on all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.00% | 101% | 102% |
| Class A2 | 0.01% | 0.09% | 0.04% | 101% | 102% |
| Class B | 0.01% | 0.06% | 0.05% | 101% | 103% |
| Class C | 0.00% | 0.02% | -0.01% | 102% | 104% |
| Class E | 0.02% | -0.01% | -0.09% | 101% | 96% |
| Overall | 0.01% | 0.05% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.03% | 103% | 100% |
| Class F | 0.01% | 0.03% | 0.00% | 101% | 103% |

Table 5 Experiment result of method 3 on random access (RA)

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.17% | 0.07% | 101% | 101% |
| Class A2 | 0.00% | 0.19% | 0.03% | 100% | 100% |
| Class B | 0.01% | -0.07% | 0.04% | 99% | 102% |
| Class C | 0.00% | -0.06% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.02% | 100% | 101% |
| Class D | 0.00% | 0.07% | 0.06% | 100% | 101% |
| Class F | 0.03% | -0.07% | -0.10% | 100% | 102% |

Summary

This contribution proposes several solutions on ISP support for 4:2:2 and 4:4:4 chroma format. The proposed method 1 provides easy fix for supporting various chroma format. The proposed method 2 and method 3 provides general solution method 3 is proposed with insignificant coding impact.

References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).