| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0227-v2 |
| --- | --- |

| Title: | Maximum transform size signaling in HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li Moonmo Koo Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {l.li, moonmo.koo, jin78.heo, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to signal maximum transform block size in high level syntax.

Introduction

In current VVC specification (Draft 4), the luma minimum and maximum Tb size (MinTbSizeY and MaxTbSizeY) are fixed as 4 and 64. Several transform size related syntax are signaled such as MaxTsSize specifying maximum transform skip size, MaxSbtSize specifying maximum subblock transform size. Besides, maximum MTS transform size is fixed as 32.

In the scenario of real-time encoding such as Twitch, for low resolution video encoding, MaxTbSizeY as 64 would not be necessary. It would be desirable to have maximum transform block size to provide VVC more versatile functionality.

Proposed Aspect 1: HLS signaling

It is proposed to signal maximum transform block size in SPS as follow.

Table 1 Proposed MaxTbSizeY signaling

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_min_luma_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_luma_transform_block_size | ue(v) |
| … |  |
| } |  |

log2_min_luma_transform_block_size_minus2 plus 2 specifies the minimum luma Tb size. The variable MinTbLog2SizeY is set equal to log2_min_luma_transform_block_size_minus2 + 2. The CVS shall not contain data that result in MinTbLog2SizeY greater than to MinCbLog2SizeY

log2_diff_max_min_luma_transform_block_size specifies the difference between the maximum and minimum luma Tb size. The variable MaxTbLog2SizeY is set equal to log2_min_luma_transform_block_size_minus2 + 2 + log2_diff_max_min_luma_transform_block_size. The CVS shall not contain data that result in MaxTbLog2SizeY greater than Min( CtbLog2SizeY, 6 ).

The minimum and maximum luma Tb size are derived as follow.

MinTbSizeY = 1  <<  MinTbLog2SizeY

MaxTbSizeY = 1  <<  MaxTbLog2SizeY

Proposed Aspect 2

It is proposed to constrain MaxSbtSize such that MaxSbtSize is less than or equal to MaxTbSizeY.

Table 2 Existing MaxSbtSize signaling

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| … |  |
| } |  |

When sps_sbt_enabled_flag equal to 1, sps_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing subblock transform is 32 luma samples. sps_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing subblock transform is 64 luma samples.

MaxSbtSize is set equal to min(MaxSbtSize, MaxTbSizeY ) which means maximum SBT size cannot exceed the maximum TU size.

Proposed Aspect 3

It is proposed that the maximum CU size for CIIP should be less than or equal to MaxTbSizeY.

Table 3 Proposed CIIP and MaxTbSizeY

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

If a CIIP CU is larger than MaxTbSizeY, the CU will be split into several TU until split TU size is less than or equal to MaxTbSizeY. When such TU split happens, for coding CIIP CU, it introduces additional delay due to the dependency between intra TU blocks.

Summary

This contribution proposes to signal maximum transform block size in high level syntax. Signaling maximum transform block size would be desirable for versatile functionality.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).