| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0230 |
| --- | --- |

| Title: | CE3-related: Simplified intra mode candidates for ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Sunmi Yoo Ling Li Jungah Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM4.0, Intra Sub-Partitions (ISP) method requires two additional 6 MPM list construction processes (one for horizontal split mode and the other for vertical split mode) to derive the intra mode candidates. This contribution proposes to remove the two additional 6 MPM lists construction process which are dedicated to ISP mode, and to assign a default intra prediction mode list based on ISP split mode. More precisely, planar and horizontal intra mode are assigned for horizontal split mode, and planar and vertical intra mode are assigned for vertical split mode.

Experimental result shows 0.01% luma BD-rates with 99% decoding time in All Intra configuration, and it also shows -0.10% and -0.25 luma BD-rate in class F and class TGM sequences.

Introduction

The Intra Sub-Partitions (ISP) which was adopted in VTM4.0, divides luma intra-predicted blocks vertically or horizontally into 2 or 4 sub-partitions depending on the block size. All sub-partitions share the same intra mode, and the intra mode candidates for the ISP are derived adaptively with the similar way of 6 MPM list construction. As shown in Figure 1, when the luma block is coded with ISP, then the MPM list is constructed based on the split mode of ISP with the neighbor block search process, and it occurs the increased decoding complexity.

Figure 1 luma intra mode parsing flowchart in VTM4

Proposed method

In this proposal, only two fixed intra mode candidates are used for ISP and the information of selected intra mode of ISP is signaled with 1 bit flag, as shown in Figure 2.

The two fixed intra mode candidates and mode flag setting for ISP are made as below:

- ISP intra mode flag == 1 : PLANAR intra mode

- ISP intra mode flag == 0 : HORIZONTAL intra mode  if ISP split mode is horizontal.

VERTICAL intra mode     if ISP split mode is vertical

If ISP intra mode flag is equal to 1, PLANAR intra mode is selected in the current ISP sub-blocks. If ISP intra mode flag is equal to 0, then HORIZONTAL or VERTICAL intra mode is selected adaptively based on the ISP split mode of current block. ISP mode flag is coded with context modeling with only 1 context model.

The transform set (hor/ver) of selected PLANAR, HORIZONTAL or VERTICAL ISP intra mode is mapped as below:

- PLANAR intra mode		: DST7 in horizontal, DST7 in vertical

- HORIZONTAL intra mode	: DCT2 in horizontal, DST7 in vertical

- VERTICAL intra mode		: DST7 in horizontal, DCT2 in vertical.

If the current sub-block width (or height) is smaller than or equal to 2, or larger than or equal to 32, DCT2 is applied in horizontal (or vertical) direction, as same as in VTM4.0.

Figure 2 luma intra mode parsing flowchart with proposed method

Experimental results

The proposed method has been implemented using VTM4.0 as software base according to the common test conditions defined in JVET-M1010[1] and Core Experiment description in JVET-M1023[2]. The optional test of TGM sequences are also performed with the proposed method.

Table 1 and Table 2 show the test results in AI and RA configuration.

Table 1 Experimental results of proposed method for all-intra (AI) test condition; anchor is VTM4

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.07% | -0.02% | 101% | 99% |
| Class A2 | -0.04% | 0.04% | -0.06% | 101% | 100% |
| Class B | 0.04% | 0.12% | 0.08% | 101% | 99% |
| Class C | 0.06% | 0.06% | 0.03% | 100% | 99% |
| Class E | 0.06% | 0.27% | 0.20% | 99% | 99% |
| Overall | 0.01% | 0.08% | 0.05% | 100% | 99% |
| Class D | -0.07% | 0.06% | -0.07% | 101% | 99% |
| Class F | -0.10% | 0.09% | -0.09% | 100% | 99% |
| Class TGM | -0.25% | -0.10% | -0.12% | 101% | 99% |

Table 2 Experimental results of proposed method for random-access (RA) test condition; anchor is VTM4

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusion

To reduce the complexity of intra mode candidate construction in ISP, the simplified intra mode candidate for ISP is proposed with the almost same coding performance as VTM4. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1023, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).