| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0234_v3 |
| --- | --- |

| Title: | [AHG12/AHG17] Signaling of virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amith Dsouza Chirag Pujara Raj Narayana Gadde WoongIl Choi Kiho Choi Kwang Pyo Choi | Tel: Email: | amith.ds@samsung.com chirag.p@samsung.com raj.gadde@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

The in-loop filters in a conventional video coding system are used to reduce undesired artifacts and enhance coding efficiency. For a 360° video, the layout for a specific projection format can contain one or more discontinuous edges. Applying in-loop filters across these discontinuous edges directly may result in poor visual quality and coding efficiency because of the pixels involved (to be referenced and/or filtered) across discontinuous edges are jointly processed (filtered).

This contribution demonstrates the need to disable in-loop filters that not only span across the entire frame but also at arbitrary discontinuous edges within a picture. An alternate frame packing arrangement is demonstrated in JVET-N0233 that shows better compression efficiency as compared to the current frame packing method. The discontinuous boundaries in the alternate frame packing arrangement do not span across the entire picture. To address such problems, it is proposed to disable the in-loop filters at virtual boundaries at tile group level in the picture.

# Introduction

Omnidirectional videos are an important facet of Versatile Video Coding (VVC). To efficiently compress 360° videos, they can be represented in multiple projection formats as demonstrated in JVET-M1004 [1]. OMAF FDIS [3] has standardised two such projection formats, Equi-Rectangular Projection (ERP) and Cube-Map Projection (CMP). In CMP format and its derived variants (EAC, HEC, PHEC), the 360° video frame is divided into six faces. Packing the six faces into one frame causes synthetic discontinuities in the frame as shown in Fig 1 and Fig 2. When compressing the packed frames using VVC, loop filters are applied across these virtual boundaries, which cause bleeding artefacts across the virtual boundaries.  JVET-M0892 [5] proposes a signaling mechanism to signal virtual boundaries. The proposed mechanism can only handle discontinuities which span across the entire frame. However, discontinuities which do not span across the entire frame cannot be handled using this method.

Fig 1. PHEC Anchor with discontinuous boundary across centre of frame

Fig 2. Alternate face arrangement of PHEC with discontinuous boundaries marked in red

# Alternate Face Arrangement

In addition to the default packing arrangement for CMP, alternate face arrangement has been demonstrated in JVET-J0023[2] and  JVET-N0233[4]. It has been shown that the face arrangement demonstrated in JVET-N0233 shows higher compression efficiency as compared to the face arrangement used in AHG6 CTC for few sequences. It has been established in previous informal subjective quality evaluations that padding between continuous boundaries of HEC also shows subjective quality improvement [4].

## Tile groups for alternate face arrangement

CTU

Tile-group boundary

In-loop filters disabled

Virtual boundary

Fig 3. Tile groups for alternate PHEC arrangement

An example method of splitting the picture into tile groups for alternate arrangement of PHEC is as shown in Fig 3. The actual discontinuities in the images are shown in blue lines, whereas the virtual boundaries as defined by this contribution are shown in dotted lines. Tile groups are shown as red boxes.

# Signalling of Virtual Boundaries

The modifications of the in-loop filters disabled across virtual boundaries are highlighted in yellow.

#### 7.3.2.2	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| single_tile_per_tile_group_flag | u(1) |
| if( !single_tile_per_tile_group_flag ) |  |
| rect_tile_group_flag | u(1) |
| if( rect_tile_group_flag && !single_tile_per_tile_group_flag ) { |  |
| num_tile_groups_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| top_left_tile_idx[ i ] | u(v) |
| bottom_right_tile_idx[ i ] | u(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag ) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| tile_group_virtual_boundary_flag[i] | u(1) |
| if(tile_group_virtual_boundary_flag[i]){ |  |
| if( i>0 ){ |  |
| derive_tile_group_virtual_boundary_flag[i] | u(1) |
| } |  |
| if(!derive_tile_group_virtual_boundary_flag[i]){ |  |
| tile_group_num_vert_boundary[i] | u(2) |
| for(j=0;j< tile_group_num_vert_boundary[i];j++){ |  |
| tile_group_vert_boundary_ctu_idx[i][j] | ue(v) |
| tile_group_vert_boundary_ctu_offset[i][j] | ue(v) |
| } |  |
| tile_group_num_hor_boundary[i] | u(2) |
| for(j=0;j< tile_group_num_hor_boundary[i];j++){ |  |
| tile_group_hor_boundary_ctu_idx[i][j] | ue(v) |
| tile_group_hor_boundary_ctu_offset[i][j] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| rpl1_idx_present_flag | u(1) |
| init_qp_minus26 | se(v) |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_tile_group_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

…

#### 7.4.3.2	Picture parameter set RBSP semantics

…

tile_group_ virtual_boundary_flag[i] equal to 1 specifies that virtual boundaries are present in the ith tile group. The in-loop filtering operations are disabled across the virtual boundaries in those tile groups.

derive_tile_group_virtual_boundary_flag[i] equal to 1 specifies that the virtual boundaries in the current tile group are present at the same offset as the (i-1)th tile group. When derive_tile_group_virtual_boundary_flag[i] is not present, it is inferred to be equal to 0.

tile_group_num_vert_boundary[i] specifies the number of virtual boundaries syntax elements, tile_group_vert_boundary_ctu_idx[i][j] and tile_group_vert_boundary_ctu_offset[i][j] that are present in the tile group. The value of tile_group_num_vert_boundary[i] shall be in the range of 0 to 2, inclusive. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_num_vert_boundary[i] is inferred to be equal to tile_group_num_vert_boundary[i-1]. When tile_group_num_vert_boundary[i] is not present, it is inferred to be equal to 0.

tile_group_vert_boundary_ctu_idx[i][j] specifies the offset of the vertical virtual boundary in CtbSizeY units. The value of tile_group_vert_boundary_ctu_idx[i][j] shall be greater than the value of tile_group_vert_boundary_ctu_idx[i][j-1]. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_vert_boundary_ctu_idx[i][j] is inferred to be equal to tile_group_vert_boundary_ctu_idx[i-1][j].

tile_group_vert_boundary_ctu_offset[i][j] specifies the offset of the vertical boundary from the start of the CTU indicated by tile_group_vert_boundary_ctu_idx[i][j] in MinCbSizeY units. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_vert_boundary_ctu_offset[i][j] is inferred to be equal to tile_group_vert_boundary_ctu_offset[i-1][j].

tile_group_num_hor_boundary[i][j] specifies the number of virtual boundaries syntax elements, tile_group_hor_boundary_ctu_idx[i][j] and tile_group_hor_boundary_ctu_offset[i][j] that are present in the tile group. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_num_hor_boundary[i] is inferred to be equal to tile_group_num_hor_boundary[i-1].The value of tile_group_num_hor_boundary[i] shall be in the range of 0 to 2, inclusive. When tile_group_num_hor_boundary[i] is not present, it is inferred to be equal to 0.

tile_group_hor_boundary_ctu_idx[i] specifies the offset of the horizontal virtual boundary in CtbSizeY units. The value of tile_group_hor_boundary_ctu_idx[i] shall be greater than the value of tile_group_hor_boundary_ctu_idx[i - 1]. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_hor_boundary_ctu_idx[i][j] is inferred to be equal to tile_group_hor_boundary_ctu_idx[i-1][j].

tile_group_hor_boundary_ctu_offset[i] specifies the offset of the horizontal boundary from the start of the CTU indicated by tile_group_hor_boundary_ctu_idx[i] in MinCbSizeY units. When derive_tile_group_virtual_boundary_flag[i] is equal to 1, the value of tile_group_hor_boundary_ctu_offset[i][j] is inferred to be equal to tile_group_hor_boundary_ctu_offset[i-1][j].

The list vertBoundaries[i][j] is defined as follows

for(i=0;i<= num_tile_groups_in_pic_minus1;i++)

{

TileOffsetX = CtbAddrTsToRs [FirstCtbAddrTs[top_left_tile_idx[ i ] ] ] % PicWidthInCtbsY

for(j=0;j< tile_group_num_vert_boundary[i];j++)

{

vertBoundaries[i][j] = TileOffsetX+(tile_group_vert_boundary_ctu_idx[i][j]<< CtbLog2SizeY) + (tile_group_vert_boundary_ctu_offset[i][j]<< MinCbLog2SizeY)

}

}

The list horBoundaries[i][j] is defined as follows

for(i=0;i<= num_tile_groups_in_pic_minus1;i++)

{

TileOffsetY = CtbAddrTsToRs [FirstCtbAddrTs[top_left_tile_idx[ i ] ] ] / PicWidthInCtbsY

for(j=0;j< tile_group_num_hor_boundary[i];j++)

{

horBoundaries[i][j] = TileOffsetY +(tile_group_hor_boundary_ctu_idx[i][j]<< CtbLog2SizeY) + (tile_group_hor_boundary_ctu_offset[i][j]<< MinCbLog2SizeY)

}

}

…

8.8	In-loop Filter Process

…

### 8.8.2 Deblocking filter process

#### 8.8.2.1 General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

The vertical edges in a picture are filtered first. Then the horizontal edges in a picture are filtered with samples modified by the vertical edge filtering process as input. The vertical and horizontal edges in the CTBs of each CTU are processed separately on a coding unit basis. The vertical edges of the coding blocks in a coding unit are filtered starting with the edge on the left-hand side of the coding blocks proceeding through the edges towards the right-hand side of the coding blocks in their geometrical order. The horizontal edges of the coding blocks in a coding unit are filtered starting with the edge on the top of the coding blocks proceeding through the edges towards the bottom of the coding blocks in their geometrical order.

NOTE – Although the filtering process is specified on a picture basis in this Specification, the filtering process can be implemented on a coding unit basis with an equivalent result, provided the decoder properly accounts for the processing dependency order so as to produce the same output values.

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

–	Edges that are at the boundary of the picture,

–	Edges that coincide with tile boundaries when loop_filter_across_tiles_enabled_flag is equal to 0,

–  Edges that coincide with the virtual boundary position when tile_group_num_vert_boundary or tile_group_num_hor_boundary is not equal to 0

–	Edges that coincide with upper or left boundaries of tile groups with tile_group_loop_filter_across_tile_groups_enabled_flag equal to 0 or tile_group_deblocking_filter_disabled_flag equal to 1,

–	Edges within tile groups with tile_group_deblocking_filter_disabled_flag equal to 1,

–	Edges that do not correspond to 8x8 sample grid boundaries of the considered component,

–	Edges within chroma components for which both sides of the edge use inter prediction,

–	Edges of chroma transform blocks that are not edges of the associated transform unit.

–	Edges across the luma transform blocks of a coding unit that has an IntraSubPartitionsSplit value not equal to ISP_NO_SPLIT.

…

#### 8.5.2.2	Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the reconstructed picture prior to deblocking, i.e., the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are the modified reconstructed picture after deblocking, i.e:

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr.

For each coding unit with coding block width log2CbW, coding block height log2CbH and location of top-left sample of the coding block ( xCb, yCb ), when edgeType is equal to EDGE_VER and xCb % 8 is equal 0 or when edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The coding block width nCbW is set equal to 1  <<  log2CbW and the coding block height nCbH is set equal to 1  <<  log2CbH

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- Otherwise if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- Otherwise, filterEdgeFlag is set equal to 1.

[Ed. (BB): Adapt syntax once tiles are integrated.]

- All elements of the two-dimensional (nCbW)x(nCbH) array edgeFlags are initialized to be equal to zero.

- The derivation process of transform block boundary specified in clause 8.8.2.3 is invoked with the location ( xB0, yB0 ) set equal to ( 0, 0 ), the block width nTbW set equal to nCbW, the block height nTbH set equal to nCbH, the variable treeType, the variable filterEdgeFlag, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The derivation process of coding subblock boundary specified in clause 8.8.2.4 is invoked with the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The derivation process of virtual boundary specified in Clause 8.8.2.5 is invoked with the location (xCb, yCb), the coding block width nCbW, the coding block height nCbH, the array edgeFlags, and the variable edgeType as inputs, and the modified array edgeFlags as output.

- The picture sample array recPicture is derived as follows:

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, recPicture is set equal to the reconstructed luma picture sample array prior to deblocking recPictureL.

- Otherwise (treeType is equal to DUAL_TREE_CHROMA), recPicture is set equal to the reconstructed chroma picture sample array prior to deblocking recPictureCb.

- The derivation process of the boundary filtering strength specified in clause 8.8.2.5 is invoked with the picture sample array recPicture, the luma location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the variable edgeType, and the array edgeFlags as inputs, and an (nCbW)x(nCbH) array verBs as output.

- The edge filtering process is invoked as follows:

- If edgeType is equal to EDGE_VER, the vertical edge filtering process for a coding unit as specified in clause 8.8.2.6.1 is invoked with the variable treeType, the reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the array verBs as inputs, and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

Otherwise if edgeType is equal to EDGE_HOR, the horizontal edge filtering process for a coding unit as specified in clause 8.8.2.6.2 is invoked with the variable treeType, the modified reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the array horBs as inputs and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

…

#### 8.8.2.5	Derivation process of virtual boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a (tile_group_num_vert_boundary) array specifying the horizontal location of the first sample after the vertical virtual boundaries in current tile group relative to top-left sample of the current picture vertBoundaries,

- a (tile_group_num_hor_boundary) array specifying the vertical location of the first sample after the horizontal virtual boundaries in current tile group relative to top-left sample of the current picture horBoundaries.

Output of this process is the modified two-dimensional (nCbW)x(nCbH) array edgeFlags.

The number of vertical virtual boundaries in the current coding block numVertBoundary is derived as follows:

- if tile_group_num_vert_boundary is greater than 0,

- for every vertBoundaries[i], i in the range 0 to tile_group_num_vert_boundary-1, if xCb is less than vertBoundaries[i] and xCb + nCbW is greater than vertBoundaries[i], numVertBoundary is set as 1

- otherwise, set numVertBoundary as 0

The number of horizontal virtual boundaries in the current coding block numHorBoundary is derived as follows:

- if tile_group_num_hor_boundary is greater than 0,

- for every horBoundaries[i], i in the range 0 to tile_group_num_hor_boundary-1, if yCb is less than horBoundaries[i] and yCb + nCbH is greater than horBoundaries[i], numHorBoundary is set as 1

- otherwise, set numHorBoundary as 0

If numVertBoundary is 1, the vertical virtual boundary offset vertBoundaryOffset is derived as follows:

vertBoundaries[numVertBoundary -1] - xCb

If numVertBoundary is 1, the horizontal virtual boundary offset horBoundaryOffset is derived as follows:

horBoundaries[numHorBoundary -1] - yCb

Depending on the value of edgeType the following applies:

- If edgeType is equal to EDGE_VER and numVertBoundary is equal to 1, the following applies for i = vertBoundaryOffset, k = 0..nCbH − 1:

edgeFlags[ k ][ i ] = 0

- If edgeType is equal to EDGE_HOR and numHorBoundary is equal to 1, the following applies for j = horBoundaryOffset, k = 0..nCbW − 1:

edgeFlags[ j ][ k ] = 0

…

### 8.8.3	Sample adaptive offset process

…

#### 8.8.3.2	CTB modification process

Inputs to this process are:

–	the picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying the colour component index,

–	a pair of variables ( rx, ry ) specifying the CTB location,

–	the CTB width nCtbSw and height nCtbSh,

–	a pair of variables (tile_group_num_vert_boundary, tile_group_num_hor_boundary) specifying the number of vertical and horizontal boundaries in the current tile group,

–   a (tile_group_num_vert_boundary) array specifying the horizontal location of the first sample after the vertical virtual boundaries in current tile group relative to top-left sample of the current picture vertBoundaries,

–  a (tile_group_num_hor_boundary) array specifying the vertical location of the first sample after the horizontal virtual boundaries in current tile group relative to top-left sample of the current picture horBoundaries.

Output of this process is a modified picture sample array saoPicture for the colour component cIdx.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The location ( xCtb, yCtb ), specifying the top-left sample of the current CTB for the colour component cIdx relative to the top-left sample of the current picture component cIdx, is derived as follows:

( xCtb, yCtb ) = ( rx * nCtbSw, ry * nCtbSh )		(8-1125)

The sample locations inside the current CTB are derived as follows:

( xSi, ySj ) = ( xCtb + i, yCtb + j )		(8-1126)

( xYi, yYj ) = ( cIdx  = =  0 ) ? ( xSi, ySj ) : ( xSi * SubWidthC, ySj * SubHeightC )	(8-1127)

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, depending on the values of pcm_loop_filter_disabled_flag, pcm_flag[ xYi ][ yYj ] and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xSi ][ ySj ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xSi ][ ySj ] is not modified:

- pcm_loop_filter_disabled_flag and pcm_flag[ xYi ][ yYj ] are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 0.

- tile_group_num_vert_boundary is not equal to 0 and xYj is equal to vertBoundaries[ n ] for any n = 0.. tile_group_num_vert_boundary − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 1.

- tile_group_num_vert_boundary is not equal to 0 and xYj is equal to vertBoundaries[ n ] - 1 for any n = 0.. tile_group_num_vert_boundary − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 1.

- tile_group_num_hor_boundary is not equal to 0 and yYj is equal to horBoundaries[ n ] for any n = 0.. tile_group_num_hor_boundary − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

- tile_group_num_hor_boundary is not equal to 0 and yYj is equal to horBoundaries[ n ] - 1 for any n = 0.. tile_group_num_hor_boundary − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

### 8.8.4	Adaptive loop filter process

…

#### 8.8.4.2	Coding tree block filtering process for luma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process,

- a filtered reconstructed luma picture sample array alfPictureL,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture.

- a pair of variables (tile_group_num_vert_boundary, tile_group_num_hor_boundary) specifying the number of vertical and horizontal boundaries in the current tile group,

- a (tile_group_num_vert_boundary) array specifying the horizontal location of the first sample after the vertical virtual boundaries in current tile group relative to top-left sample of the current picture vertBoundaries,

- a (tile_group_num_hor_boundary) array specifying the vertical location of the first sample after the horizontal virtual boundaries in current tile group relative to top-left sample of the current picture horBoundaries.

Output of this process is the modified filtered reconstructed luma picture sample array alfPictureL.

The derivation process for filter index clause 8.8.4.3 is invoked with the location ( xCtb, yCtb ) and the reconstructed luma picture sample array recPictureL as inputs, and filtIdx[ x ][ y ] and transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1 as outputs.

For the derivation of the filtered reconstructed luma samples alfPictureL[ x ][ y ], each reconstructed luma sample inside the current luma coding tree block recPictureL[ x ][ y ] is filtered as follows with x, y = 0..CtbSizeY − 1:

  - The array of luma filter coefficients f[ j ] corresponding to the filter specified by filtIdx[ x ][ y ] is derived as follows with j = 0..12:

f[ j ] = AlfCoeffL[ filtIdx[ x ][ y ] ][ j ]		(8-1135)

  - The luma filter coefficients filterCoeff are derived depending on transposeIdx[ x ][ y ] as follows:

  - If transposeIndex[ x ][ y ] = = 1,

filterCoeff[ ] = { f[9], f[4], f[10], f[8], f[1], f[5], f[11], f[7], f[3], f[0], f[2], f[6], f[12] }	(8-1136)

  - Otherwise, if transposeIndex[ x ][ y ] = = 2,

filterCoeff[ ] = { f[0], f[3], f[2], f[1], f[8], f[7], f[6], f[5], f[4], f[9], f[10], f[11], f[12] }	(8-1137)

  - Otherwise, if transposeIndex[ x ][ y ] = = 3,

filterCoeff[ ] = { f[9], f[8], f[10], f[4], f[3], f[7], f[11], f[5], f[1], f[0], f[2], f[6], f[12] }	(8-1138)

  - Otherwise,

filterCoeff[ ] = { f[0], f[1], f[2], f[3], f[4], f[5], f[6], f[7], f[8], f[9], f[10], f[11], f[12] }	(8-1139)

  - The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x )	(8-1140)

vy = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y )	(8-1141)

  - The variable sum is derived as follows:

sum = filterCoeff[ 0 ]   * ( recPictureL[ hx, vy + 3 ] + recPictureL[ hx, vy − 3 ] ) + 
	filterCoeff[ 1 ]   * ( recPictureL[ hx + 1, vy + 2 ] + recPictureL[ hx − 1, vy − 2 ] ) + 
	filterCoeff[ 2 ]   * ( recPictureL[ hx, vy + 2 ] + recPictureL[ hx, vy − 2 ] ) + 
	filterCoeff[ 3 ]   * ( recPictureL[ hx − 1, vy + 2 ] + recPictureL[ hx + 1, vy − 2 ] ) + 
	filterCoeff[ 4 ]   * ( recPictureL[ hx + 2, vy + 1 ] + recPictureL[ hx − 2, vy − 1 ] ) + 
	filterCoeff[ 5 ]   * ( recPictureL[ hx + 1, vy + 1 ] + recPictureL[ hx − 1, vy − 1 ] ) + 
	filterCoeff[ 6 ]   * ( recPictureL[ hx, vy + 1 ] + recPictureL[ hx, vy − 1 ] ) + 	(8-1142)
	filterCoeff[ 7 ]   * ( recPictureL[ hx − 1, vy + 1 ] + recPictureL[ hx + 1, vy − 1 ] ) + 
	filterCoeff[ 8 ]   * ( recPictureL[ hx − 2, vy + 1 ] + recPictureL[ hx + 2, vy − 1 ] ) + 
	filterCoeff[ 9 ]   * ( recPictureL[ hx + 3, vy ] + recPictureL[ hx − 3, vy ] ) + 
	filterCoeff[ 10 ] * ( recPictureL[ hx + 2, vy ] + recPictureL[ hx − 2, vy ] ) + 
	filterCoeff[ 11 ] * ( recPictureL[ hx + 1, vy ] + recPictureL[ hx − 1, vy ] ) + 
	filterCoeff[ 12 ] *   recPictureL[ hx, vy ]

sum = ( sum + 64 ) >> 7		(8-1143)

The variable isVertBoundaryPixel is derived as follows:

  - If tile_group_num_vert_boundary is greater than 0 and if hx is greater than or equal to vertBoundaries[i]–3 and hx is less than vertBoundaries[i]+3, for all i in 0..tile_group_num_vert_boundary-1,  isVertBoundaryPixel is set to true

  - Otherwise, isVertBoundaryPixel is set to false

The variable isHorBoundaryPixel is derived as follows:

  - If tile_group_num_hor_boundary is greater than 0 and if vy is greater than or equal to horBoundaries[i]–3 and vy is less than horBoundaries[i]+3, for all i in 0..tile_group_num_hor_boundary-1,  isHorBoundaryPixel is set to true

  - Otherwise, isHorBoundaryPixel is set to false

  - The modified filtered reconstructed luma picture sample alfPictureL[ xCtb + x ][ yCtb + y ] is derived as follows:

  - If isHorBoundaryPixel is true or isVertBoundaryPixel is true,

alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, recPictureL[ hx, vy ])	(8-1144)

  - Otherwise

alfPictureL[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, sum )	(8-1146)

#### 8.8.4.3	Derivation process for ALF transpose and filter index for luma samples

Inputs of this process are:

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture,

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process.

- a pair of variables (tile_group_num_vert_boundary, tile_group_num_hor_boundary) specifying the number of vertical and horizontal boundaries in the current tile group,

- a (tile_group_num_vert_boundary) array specifying the horizontal location of the first sample after the vertical virtual boundaries in current tile group relative to top-left sample of the current picture vertBoundaries,

- a (tile_group_num_hor_boundary) array specifying the vertical location of the first sample after the horizontal virtual boundaries in current tile group relative to top-left sample of the current picture horBoundaries.

Outputs of this process are

- the classification filter index array filtIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1,

- the transpose index array transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1.

The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, x )		(8-1147)

vy = Clip3( 0, pic_height_in_luma_samples − 1, y )		(8-1148)

The variable isVertBoundaryPixel is derived as follows:

  - If tile_group_num_vert_boundary is greater than 0 and if hx is greater than or equal to vertBoundaries[i]–3 and hx is less than vertBoundaries[i]+3, for all i in 0..tile_group_num_vert_boundary-1,  isVertBoundaryPixel is set to true

  - Otherwise, isVertBoundaryPixel is set to false

The variable isHorBoundaryPixel is derived as follows:

  - If tile_group_num_hor_boundary is greater than 0 and if vy is greater than or equal to horBoundaries[i]–3 and vy is less than horBoundaries[i]+3, for all i in 0..tile_group_num_hor_boundary-1,  isHorBoundaryPixel is set to true

  - Otherwise, isHorBoundaryPixel is set to false

The classification filter index array filtIdx and the transpose index array transposeIdx are derived by the following ordered steps:

- The variables filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] with x, y = − 2..CtbSizeY + 1 are derived as follows:

- If both x and y are even numbers or both x and y are uneven numbers, the following applies:

filtH[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x−1, vyCtb+y ] − 	(8-1149)
	 recPicture[ hxCtb+x+1, vyCtb+y ]  )

filtV[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x, vyCtb+y−1 ] − 	(8-1150)
	 recPicture[ hxCtb+x, vyCtb+y+1 ]  )

filtD0[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x−1, vyCtb+y−1 ] − 	(8-1151)
	recPicture[ hxCtb+x+1, vyCtb+y+1 ]  )

filtD1[ x ][ y ] = Abs(  ( recPicture[ hxCtb+x, vyCtb+y ] << 1 ) − recPicture[ hxCtb+x+1, vyCtb+y−1 ] − 	(8-1152)
	recPicture[ hxCtb+x−1, vyCtb+y+1 ]  )

- If isHorBoundaryPixel is true filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] are set equal to 0

- If isVertBoundaryPixel is true, filtH[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] are set equal to 0

- Otherwise, filtH[ x ][ y ], filtV[ x ][ y ], filtD0[ x ][ y ] and filtD1[ x ][ y ] are set equal to 0.

- The variables varTempH1[ x ][ y ], varTempV1[ x ][ y ], varTempD01[ x ][ y ], varTempD11[ x ][ y ] and varTemp[ x ][ y ] with x, y = 0..( CtbSizeY  − 1 ) >> 2 are derived as follows:

sumH[ x ][ y ] = ij filtH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-1153)

sumV[ x ][ y ] = ij filtV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-1154)

sumD0[ x ][ y ] = ij filtD0[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-1155)

sumD1[ x ][ y ] = ij filtD1[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = −2..5	(8-1156)

sumOfHV[ x ][ y ] = sumH[ x ][ y ] + sumV[ x ][ y ]	(8-1157)

- The variables dir1[ x ][ y ], dir2[ x ][ y ] and dirS[ x ][ y ] with x, y = 0..CtbSizeY − 1 are derived as follows:

- The variables hv1, hv0 and dirHV are derived as follows:

- If sumV[ x >> 2 ][ y >> 2 ] is greater than sumH[ x >> 2 ][ y >> 2 ], the following applies:

hv1 = sumV[ x >> 2 ][ y >> 2 ]		(8-1158)

hv0 = sumH[ x >> 2 ][ y >> 2 ] 		(8-1159)

dirHV = 1		(8-1160)

- Otherwise, the following applies:

hv1 = sumH[ x >> 2 ][ y >> 2 ]		(8-1161)

hv0 = sumV[ x >> 2 ][ y >> 2 ]		(8-1162)

dirHV = 3		(8-1163)

- The variables d1, d0 and dirD are derived as follows:

- If sumD0[ x >> 2 ][ y >> 2 ] is greater than sumD1[ x >> 2 ][ y >> 2 ], the following applies:

d1 = sumD0[ x >> 2 ][ y >> 2 ]	(8-1162)

d0 = sumD1[ x >> 2 ][ y >> 2 ] 	(8-1163)

dirD = 0		(8-1164)

- Otherwise, the following applies:

d1 = sumD1[ x >> 2 ][ y >> 2 ]		(8-1165)

d0 = sumD0[ x >> 2 ][ y >> 2 ]		(8-1166)

dirD = 2		(8-1167)

- The variables hvd1, hvd0, are derived as follows:

hvd1 = ( d1 * hv0 > hv1 * d0 )  ?  d1  :  hv1	(8-1168)

hvd0 = ( d1 * hv0 > hv1 * d0 )  ?  d0  :  hv0	(8-1169)

- The variables dirS[ x ][ y ], dir1[ x ][ y ] and dir2[ x ][ y ] derived as follows:

dir1[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirD  :  dirHV	(8-1170)

dir2[ x ][ y ] = ( d1 * hv0 > hv1 * d0 )  ?  dirHV  :  dirD	(8-1171)

dirS[ x ][ y ] = ( hvd1 > 2 * hvd0 )  ?  1  :  ( ( hvd1 * 2 > 9 * hvd0 )  ?  2  :  0 )	(8-1172)

- The variable avgVar[ x ][ y ] with x, y = 0..CtbSizeY − 1 is derived as follows:

varTab[ ] = { 0, 1, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 4 }	(8-1173)

avgVar[ x ][ y ] = varTab[ Clip3( 0, 15, ( sumOfHV[ x >> 2 ][ y >> 2 ] * 64 ) >> ( 3 + BitDepthY ) ) ]	(8-1174)

- The classification filter index array filtIdx[ x ][ y ] and the transpose index array transposeIdx[ x ][ y ] with x = y = 0..CtbSizeY − 1 are derived as follows:

transposeTable[ ] = { 0, 1, 0, 2, 2, 3, 1, 3 }

transposeIdx[ x ][ y ] = transposeTable[ dir1[ x ][ y ] * 2 + ( dir2[ x ][ y ] >> 1 ) ]

filtIdx[ x ][ y ] = avgVar[ x ][ y ]

When dirS[ x ][ y ] is not equal 0, filtIdx[ x ][ y ] is modified as follows:

filtIdx[ x ][ y ] += ( ( ( dir1[ x ][ y ] & 0x1 ) << 1 ) + dirS[ x ][ y ] ) * 5	(8-1175)

#### 8.8.4.4	Coding tree block filtering process for chroma samples

Inputs of this process are:

- a reconstructed chroma picture sample array recPicture prior to the adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPicture,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top left sample of the current picture.

- a pair of variables (tile_group_num_vert_boundary, tile_group_num_hor_boundary) specifying the number of vertical and horizontal boundaries in the current tile group,

- a (tile_group_num_vert_boundary) array specifying the left sample of the vertical virtual boundaries in current tile group relative to top-left sample of the current picture vertBoundaries,

- a (tile_group_num_hor_boundary) array specifying the top sample of the horizontal virtual boundaries in current tile group relative to top-left sample of the current picture horBoundaries.

Output of this process is the modified filtered reconstructed chroma picture sample array alfPicture.

The size of the current chroma coding tree block ctbSizeC is derived as follows:

ctbSizeC = CtbSizeY / SubWidthC		(8-1176)

For the derivation of the filtered reconstructed chroma samples alfPicture[ x ][ y ], each reconstructed chroma sample inside the current chroma coding tree block recPicture[ x ][ y ] is filtered as follows with x, y = 0..ctbSizeC − 1:

  - The locations ( hx, vy ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x )	(8-1177)

vy = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y )	(8-1178)

  - The variable sum is derived as follows:

sum = AlfCoeffC[ 0 ] * ( recPicture[ hx, vy + 2 ] + recPicture[ hx, vy − 2 ] ) + 
	AlfCoeffC[ 1 ] * ( recPicture[ hx + 1, vy + 1 ] + recPicture[ hx − 1, vy − 1 ] ) + 
	AlfCoeffC[ 2 ] * ( recPicture[ hx, vy + 1 ] + recPicture[ hx, vy − 1 ] ) + 	(8-1179)
	AlfCoeffC[ 3 ] * ( recPicture[ hx − 1, vy + 1 ] + recPicture[ hx + 1, vy − 1 ] ) + 
	AlfCoeffC[ 4 ] * ( recPicture[ hx + 2, vy ] + recPicture[ hx − 2, vy ] ) + 
	AlfCoeffC[ 5 ] * ( recPicture[ hx + 1, vy ] + recPicture[ hx − 1, vy ] ) + 
	AlfCoeffC[ 6 ] * recPicture[ hx, vy ]

sum = ( sum + 64 ) >> 7		(8-1180)

The variable isVertBoundaryPixel is derived as follows:

  - If tile_group_num_vert_boundary is greater than 0 and if hx is greater than or equal to (vertBoundaries[i] / SubHeightC )–2 and hx is less than (vertBoundaries[i] / SubHeightC) +2, for all i in 0..tile_group_num_vert_boundary-1,  isVertBoundaryPixel is set to true

  - Otherwise, isVertBoundaryPixel is set to false

The variable isHorBoundaryPixel is derived as follows:

  - If tile_group_num_hor_boundary is greater than 0 and if vy is greater than or equal to (horBoundaries[i] / SubWidthC)–2 and vy is less than (horBoundaries[i] / SubWidthC)+2, for all i in 0..tile_group_num_hor_boundary-1,  isHorBoundaryPixel is set to true

  - Otherwise, isHorBoundaryPixel is set to false

  - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows:

  - If isHorBoundaryPixel is true or isVertBoundaryPixel is true,

alfPicture[ xCtb + x ][ yCtb + y ] = Clip3( 0, ( 1 << BitDepthY ) − 1, recPicture[ hx, vy ])	(8-1181)

  - Otherwise

alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum )	(8-1182)

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y. Ye, J. Boyce “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 9)” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1004, Jan. 2019, Marrakech, MA

- M. Bläser, J. Sauer, M. Wien (RWTH Aachen Univ.) “Description of SDR and 360° video coding technology proposal by RWTH Aachen University,” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0023, Apr. 2018, San Diego, US.

- “ISO/IEC FDIS 23090-2: Information technology coded representation of immersive media (MPEG-I) part 2: Omnidirectional media format,” Final Draft International Standard (FDIS), Feb 2018

- J. Boyce “BoG report on CE13 and CE13 related 360° video coding” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0874, Jan. 2019,  Marrakech, MA,

- S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju, P. Hanhart, Y. He  “CE-13 related: Loop filter disabled across virtual boundaries” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0892, Jan. 2019,  Marrakech, MA,

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).