| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0235 |
| --- | --- |

| Title: | CE2-related: Symmetric MVD Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong (Daniel) Luo Yuwen He 4 Research Way, Princeton, NJ 08540 | Tel: Email: | +1-609-436-0885 Daniel.Luo@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The SMVD is only allowed when the reference picture list satisfied a certain condition. This contribution proposes to signal a flag in SPS and a flag in tile group header respectively to enable/disable symmetric MVD (SMVD). The flag signaled in tile group header can avoid the CU parsing depending on the construction of reference list. It also gives encoder the freedom of switching off SMVD in tile group level. Simulation results show that on average no change in BD rate and complexity comparing to VTM-4.0

# Introduction

In VVC working draft 4, symmetric MVD (SMVD) mode [1] is allowed only when one of the following conditions on the reference picture list is satisfied:

a) when forward reference picture exists in list 0 and backward reference picture exists in list 1,

b) when backward reference picture exists in list 0 and forward reference picture exist in list 1.

When neither of the conditions is satisfied, the sym_mvd_flag is not present and implied to be 0.

In order for the decoder to perform the checking of these conditions, the reference picture lists of the current picture (i.e. list0 and list1) have to be constructed before CU parsing. This causes an undesirable parsing dependency.

In some cases, the encoder chooses not to use SMVD mode in order to save the encoding complexity. However, if any of the two conditions is true, then a flag in the CU level flag is required to be signaled, which is not efficient.

Based on the above reasons, this contribution proposes to signal a flag in SPS and a flag in tile group header, to enable or disable SMVD mode in sequence level and tile group level respectively.

# Simulation results

The proposed syntax has been implemented in VTM-4.0. The test condition described in JVET-M1010 [2] is used to verify the contribution. Table 4 reportedly shows that on average there is no BD rate and complexity difference comparing to VTM-4.0.

Table 4 Coding performance of proposed symmetric MVD over VTM3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% |

# References

- J. Luo, Y. He, “CE2-related: Simplified symmetric MVD based on CE4.4.3”, JVET-M0444, the 13th JVET meeting, Jan. 2019, Marrakech.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakech.

# Patent rights declaration(s)

InterDitigal may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).