| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0236 |
| --- | --- |

| Title: | CE2-related: Prediction refinement with optical flow for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiancong (Daniel) Luo Yuwen He 4 Research Way, Princeton, NJ 08540 | Tel: Email: | +1-609-436-0885 Daniel.Luo@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, prediction sample is refined by adding a difference derived by the optical flow equation, which is referred as prediction refinement with optical flow (PROF). The proposed method can achieve inter prediction in pixel level granularity without increasing the memory access bandwidth. The simulation result reportedly shows on average X.XX% and X.XX% BD rate saving for RA and LD, respectively, with X.XX% encoding time and X.XX% decoding time increase.

# Introduction

Affine motion model parameters can be used to derive the motion vector of each pixel in a CU. However, due to the high complexity and memory access bandwidth for generating pixel-based affine motion compensated prediction, the current VVC adopted a sub-block based affine motion compensation method, where a CU is divided into 4x4 sub-blocks, each of which is assigned with a MV derived from the affine CU’s control point MVs. The sub-block based affine motion compensation is a trade-off between coding efficiency, complexity and memory access bandwidth. It loses prediction accuracy due to sub-block based prediction.

# Prediction refinement with optical flow

To achieve a finer granularity of motion compensation, this contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation. The proposed PROF is described as following four steps.

Step 1) The sub-block-based affine motion compensation is performed to generate sub-block prediction Ii, j.

Step2) The spatial gradients gx(i,j) and gy(i,j) of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1].

gxi, j=Ii+1, j-Ii-1, j

gyi, j=Ii, j+1-Ii, j-1

The sub-block prediction is extended by one pixel on each side for the gradient calculation. To reduce the memory bandwidth and complexity, the pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. Therefore, additional interpolation for padding region is avoided.

Step 3) The luma prediction refinement is calculated by the optical flow equation.

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j)

where the Δv(i,j) is the difference between pixel MV computed for sample location (i,j), denoted by vi,j, and the sub-block MV of the sub-block to which pixel (i,j) belongs, as shown in Figure 1.

Figure 1. Sub-block MV VSB  and pixel Δv(i,j) (red arrow)

Since the affine model parameters and the pixel location relative to the sub-block center are not changed from sub-block to sub-block, Δvi,j can be calculated for the first sub-block, and reused for other sub-blocks in the same CU.  Let x and y be the horizontal and vertical offset from the pixel location to the center of the sub-block, Δvx,y can be derived by the following equation,

Δvxx,y=c*x+d*yΔvy(x, y)=e*x+f*y

For 4-parameter affine model,

c=f=v1x-v0xwe=-d=v1y-v0yw

For 6-parameter affine model,

c=v1x-v0xwd=v2x-v0xhe=v1y-v0ywf=v2y-v0yh

where (v0x,v0y), (v1x,v1y), (v2x,v2y) are the top-left, top-right and bottom-left control point motion vectors, w and h are the width and height of the CU.

Step 4) Finally, the luma prediction refinement is added to the sub-block prediction Ii, j. The final prediction I’ is generated as the following equation.

I'(i,j)= Ii,j+ΔI(i,j)

# Simulation results

The contribution has been implemented on top of VTM-4.0. The test condition described in JVET-M1010 [1] is used in the simulation. Two tests are performed in this contribution. In test 1, affine sub-block size is 4x4. In test 2, affine sub-block size is 4x4 for uni-prediction and 8x8 for bi-prediction. For test 2, it is no longer the bi-prediction but the uni-prediction the worst case for memory access for affine mode, whose worst-case bandwidth is 27% smaller than that of the non-affine mode. Results for RA/LB configurations are summarized in Table 1 and Table 2. Additional test (test 3) results for those affine sequences provided by Huawei is also provided in Table 3.

Additional tests are conducted to test an alterative way to generate gradient: the 11×11 integer reference samples before interpolation is fetched for the interpolation of 4×4 sub-block, a 13×13 reference samples is generated by extending the 11×11 block with the border copied from adjacent inner samples, and is used for the interpolation of 6×6 prediction, which is used to calculate the 4x4 gradient. Result of this test for RA/LD configurations are summarized in Table 4 and 5 for 4x4 sub-block (test 4) and 8x8 sub-block for bi-prediction (test 5), respectively.

Table 1.  Affine sub-block size 4x4 test on CTC sequence

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | - | - | - | - | - |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | - | - | - | - | - |
| Class B | -0.58% | -0.23% | -0.06% | 103% | 102% | - | - | - | - | - |
| Class C | -0.27% | -0.06% | -0.15% | 103% | 102% | - | - | - | - | - |
| Class E |  |  |  |  |  | - | - | - | - | - |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | - | - | - | - | - |
| Class D | -0.59% | -0.31% | -0.23% | 102% | 102% | - | - | - | - | - |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | - | - | - | - | - |

Table 2. Affine sub-block size 4x4 for uni-prediction and 8x8 for bi-prediction

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | - | - | - | - | - | - | - | - | - | - |
| Class C | - | - | - | - | - | - | - | - | - | - |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | - | - | - | - | - | - | - | - | - | - |
| Class D | - | - | - | - | - | - | - | - | - | - |
| Class F | - | - | - | - | - | - | - | - | - | - |

Table 3  Additional test on affine sequence provided by Huawei, with test 1 condition

|  | Random Access Main 10 |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  | Over VTM 4.0 |  |  |
|  | Y | U | V | Y | U | V |
| BlueSkyPart | -1.68% | -0.62% | -0.66% | - | - | - |
| JetParts | -0.27% | 0.88% | -0.28% | - | - | - |
| ShieldsPart | -1.18% | -0.57% | -0.85% | - | - | - |
| SpincalendarPart | -2.72% | -1.66% | -1.77% | - | - | - |
| StationKtaPart | -2.49% | -0.49% | -0.32% |  |  |  |
| TractorPart | -2.89% | -0.40% | -0.44% | - | - | - |
| Overall | -1.87% | -0.48% | -0.72% | - | - | - |

Table 4.  Affine sub-block size 4x4 test on CTC sequence with alternative gradient calculation

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.10% | 0.15% | 105% | 102% | - | - | - | - | - |
| Class A2 | -0.90% | 0.29% | 0.30% | 107% | 103% | - | - | - | - | - |
| Class B | -0.63% | -0.27% | -0.17% | 105% | 103% | - | - | - | - | - |
| Class C | -0.35% | -0.04% | -0.10% | 105% | 102% | - | - | - | - | - |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | -0.52% | -0.06% | 0.01% | 105% | 102% | - | - | - | - | - |
| Class D | -0.59% | -0.19% | -0.23% | 105% | 102% | - | - | - | - | - |
| Class F | -0.98% | -0.48% | -0.45% | 105% | 101% | - | - | - | - | - |

Table 5. Affine sub-block size 4x4 for uni-prediction and 8x8 for bi-prediction with alternative gradient calculation

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 4.0 |  |  |  |  | Over VTM 4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.10% | 0.16% | 104% | 100% | - | - | - | - | - |
| Class A2 | -0.55% | 0.49% | 0.52% | 106% | 100% | - | - | - | - | - |
| Class B | -0.45% | -0.12% | 0.00% | 105% | 101% | - | - | - | - | - |
| Class C | -0.17% | 0.11% | -0.10% | 104% | 102% | - | - | - | - | - |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | -0.33% | 0.11% | 0.11% | 105% | 101% | - | - | - | - | - |
| Class D | -0.31% | 0.10% | 0.00% | 104% | 102% | - | - | - | - | - |
| Class F | -0.68% | -0.14% | -0.14% | 104% | 101% | - | - | - | - | - |

# Subjective tests

Subjective differences between PROF on and off are shown here. Figure 2 shows frame 170 of SlideShow encoded with QP 37. The blocking artifact is effectively reduced by PROF. Figure 3 shows frame 21 of CatRobot encoded with QP 37. Image is sharper with PROF on.

Crop area

| PROF off | PROF on |
| --- | --- |

Figure 2. Frame 170 of SlideShow encoded with QP 37.

Crop area

| PROF off | PROF on |
| --- | --- |

Figure 3. Frame 21 of SlideShow encoded with QP 37.

# Conclusions

PROF can improve sub-block based affine prediction accuracy. The simulations for those affine sequences show that it is especially effective for affine mode. When sub-block size is changed from 4x4 to 8x8 for bi-prediction, PROF also shows coding gain, which means it can effectively reduce the affine memory bandwidth.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018.

# Patent rights declaration(s)

InterDitigal may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).