| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 Mar. 2019 | Document: JVET-N0238-v3 |
| --- | --- |

| Title: | AHG13: ISP Tool-off Tests in VTM-4.0.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document presents the results of the Intra Sub-partitions (ISP) coding mode tool-off tests on top of VTM-4.0.1.

# Introduction

This document presents the results of the Intra Sub-partitions (ISP) coding mode tool-off tests on top of VTM-4.0.1. With respect to VTM-4.0, VTM-4.0.1 introduces a new speed-up related to ISP that can be enabled with the configuration file parameter "ISPFast". The reference document for ISP is JVET-M0102 [1]. Two different tests have been designed:

- Test 1: VTM-4.0.1 with ISP disabled against VTM-4.0.1 with ISPFast set to 0.

- Test 2: VTM-4.0.1 with ISP disabled against VTM-4.0.1 with ISPFast set to 1.

# Experimental results

According to the common test conditions [2], Test 1 and Test 2 are evaluated for intra-only (AI), random-access (RA) and low-delay-b (LDB) configuration with the VTM-4.0.1 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler.

## Test 1

Table I: Results for Test 1 using the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 with ISPFast disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,21% | 0,16% | 0,18% | 84% | 97% |
| Class A2 | 0,31% | 0,33% | 0,22% | 85% | 97% |
| Class B | 0,57% | 0,46% | 0,54% | 83% | 96% |
| Class C | 0,86% | 0,60% | 0,71% | 83% | 94% |
| Class E | 1,06% | 0,67% | 0,49% | 84% | 96% |
| Overall | 0,61% | 0,45% | 0,46% | 83% | 96% |
| Class D | 0,71% | 0,55% | 0,47% | 83% | 96% |
| Class F | 0,96% | 0,61% | 0,76% | 90% | 96% |

Table II: Results for Test 1 using the RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 with ISPFast disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,16% | 0,25% | 0,16% | 97% | 100% |
| Class A2 | 0,22% | 0,28% | -0,03% | 98% | 100% |
| Class B | 0,27% | 0,29% | -0,02% | 98% | 101% |
| Class C | 0,37% | 0,32% | 0,08% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,26% | 0,29% | 0,04% | 98% | 100% |
| Class D | 0,25% | 0,12% | -0,26% | 98% | 98% |
| Class F | 0,66% | 0,33% | 0,28% | 98% | 99% |

Table III: Results for Test 1 using the LDB configuration

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 with ISPFast disabled |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,05% | -0,08% | -0,38% | 99% | 100% |
| Class C | 0,13% | 0,72% | 0,39% | 99% | 99% |
| Class E | 0,26% | 0,72% | -0,53% | 99% | 99% |
| Overall | 0,13% | 0,39% | -0,16% | 99% | 100% |
| Class D | -0,03% | -0,04% | 0,24% | 100% | 99% |
| Class F | 0,35% | 0,34% | 0,71% | 99% | 100% |

## Test 2

Table IV: Results for Test 2 using the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,18% | 0,16% | 0,11% | 86% | 98% |
| Class A2 | 0,28% | 0,21% | 0,21% | 87% | 98% |
| Class B | 0,54% | 0,47% | 0,48% | 86% | 96% |
| Class C | 0,82% | 0,64% | 0,74% | 87% | 94% |
| Class E | 1,00% | 0,63% | 0,47% | 88% | 95% |
| Overall | 0,58% | 0,44% | 0,43% | 87% | 96% |
| Class D | 0,69% | 0,57% | 0,61% | 87% | 97% |
| Class F | 0,92% | 0,53% | 0,67% | 92% | 96% |

Table V: Results for Test 2 using the RA configuration

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,14% | 0,25% | 0,14% | 98% | 99% |
| Class A2 | 0,22% | 0,27% | -0,02% | 99% | 100% |
| Class B | 0,26% | 0,40% | 0,13% | 99% | 100% |
| Class C | 0,36% | 0,58% | 0,49% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,25% | 0,39% | 0,20% | 98% | 100% |
| Class D | 0,25% | 0,24% | -0,37% | 99% | 99% |
| Class F | 0,64% | 0,45% | 0,32% | 98% | 99% |

Table VI: Results for Test 2 using the LDB configuration

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,05% | 0,27% | -0,55% | 99% | 100% |
| Class C | 0,13% | 0,24% | 0,18% | 99% | 99% |
| Class E | 0,21% | -1,03% | -0,97% | 99% | 99% |
| Overall | 0,12% | -0,06% | -0,41% | 99% | 99% |
| Class D | 0,01% | 0,00% | -0,22% | 100% | 99% |
| Class F | 0,41% | 0,41% | 0,06% | 99% | 100% |

# Cross-check

# Cross-checks have been performed by Tencent.

# Conclusion

It can be observed that Test 2 obtains a 4% reduction in the encoding run-time in exchange for a 0.03% loss in the coding performance for the AI configuration with respect to Test 1. The differences between Test 1 and Test 2 for the RA and LDB cases are not significant: they produce a loss of 0.01% but do not alter the encoding run-time.

# References

| [1] | S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)," in Document JVET-M0102, Marrakech, MA, 2019. |
| --- | --- |
| [2] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. (. Sühring, "JVET common test conditions and software reference configurations for SDR video," Document JVET-M1010, Marrakech, MA, 2019. |