##### Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << 1, yTbC << 1 )		(8-156)

The variables availL, availT and availTL are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC, yTbC − 1 ) as inputs, and the output is assigned to availT.

- The availability of top-left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC − 1 ) as inputs, and the output is assigned to availTL.

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC + x, yTbC − 1 ) as inputs, and the output is assigned to availableTR

- When availableTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC + y ) as inputs, and the output is assigned to availableLB

- When availableLB is equal to TRUE, numLeftBelow is incremented by one.

The number of available neighbouring chroma samples on the top and top-right numTopSamp and the number of available neighbouring chroma samples on the left and left-below nLeftSamp are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT  ?  nTbW  :  0		(8-157)

numSampL = availL  ?  nTbH  :  0		(8-158)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM )  ?  ( nTbW + numTopRight )  :  0	(8-159)

numSampL = ( availL  &&  predModeIntra = =  INTRA_L_CCLM )  ?  ( nTbH + numLeftBelow )  :  0	(8-160)

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC & ( 1 << ( CtbLog2SizeY − 1 ) − 1 )  = =  0 )  ?  TRUE  :  FALSE. 	(8-161)

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both numSampL and numSampT are equal to 0, the following applies:

predSamples[ x ][ y ] = 1 << ( BitDepthC − 1 )		(8-162)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * 2 − 1, y= 0..nTbH * 2 − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples samples pY[ x ][ y ] are derived as follows:

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..2 * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..2 * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x ][ 2 * y − 1 ] +
	 pY[ 2 * x − 1 ][ 2 * y ] + 4 * pY[ 2 * x ][ 2 * y ] + pY[ 2 * x + 1 ][ 2 * y ] + 	(8-163)
		pY[ 2 * x ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] +
	 pY[ −1 ][ 2 * y ] + 4 * pY[ 0 ][ 2 * y ] + pY[ 1 ][ 2 * y ] + 	(8-164)
		pY[ 0 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] + 2 * pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 2 ) >> 2	(8-165)

    - If availT is equal to TRUE, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x ][ −1 ] +
	 pY[ 2 * x − 1 ][ 0 ] + 4 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 	(8-166)
		pY[ 2 * x ][ 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x − 1 ][ 0 ] + 2 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 2 ) >> 2	(8-167)

    - If availL is equal to TRUE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] +
	 pY[ −1 ][ 0 ] + 4 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 	(8-168)
		pY[ 0 ][ 1 ] + 4 ) >> 3

    - Otherwise if availL is equal to TRUE and availT is equal to FALSE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ −1 ][ 0 ] + 2 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 2 ) >> 2	(8-169)

    - Otherwise if availL is equal to FALSE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] + 2 * pY[ 0 ][ 0 ] + pY[ 0 ][ 1 ] + 2 ) >> 2	(8-170)

    - Otherwise (availL is equal to FALSE and availT is equal to FALSE), pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = pY[ 0 ][ 0 ]		(8-171)

    - Otherwise, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x − 1 ][ 2 * y ] + 	pY[ 2 * x − 1 ][ 2 * y + 1 ] + 
	2* pY[ 2 * x ][ 2 * y ] +	 2*pY[ 2 * x ][ 2 * y + 1 ] + 	(8-172)
		pY[ 2 * x + 1 ][ 2 * y ] + 	pY[ 2 * x + 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ 0 ][ 2 * y ] + 2*pY[ 0 ][ 2*y + 1 ] + 	(8-173)
		pY[ 1 ][ 2 * y ] +	 pY[ 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 1 ) >> 1	(8-174)

  - When numSampL is greater than 0, the down-sampled neighbouring left luma samples pLeftDsY[ y ] with y = 0..numSampL − 1 are derived as follows:

    - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

    - pLeftDsY[ y ] with y = 1..nTbH − 1 is derived as follows:

pLeftDsY[ y ] = ( pY[ −2 ][ 2 * y − 1 ] +
		 pY[ −3 ][ 2 * y ] + 4 * pY[ −2 ][ 2 * y ] + pY[ −1 ][ 2 * y ] + 	(8-175)
		pY[ −2][ 2 * y + 1 ] + 4 ) >> 3

    - If availTL is equal to TRUE, pLeftDsY[ 0 ] is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −2 ][ −1 ] +
		 pY[ −3 ][ 0 ] + 4 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 	(8-176)
		pY[ −2 ][ 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −3 ][ 0 ] + 2 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 2 ) >> 2	(8-177)

    - Otherwise, the following applies:

pLeftDsY[ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ −2 ][ 2 * y ] + 2*pY[ −2 ][ 2 * y + 1 ] + 	(8-178)
		pY[ −3 ][ 2 * y ] + 	pY[ −3 ][ 2 * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the down-sampled neighbouring top luma samples pTopDsY[ x ] with x = 0..numSampT − 1 are specified as follows:

    - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

    - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

    - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x ][ −3 ] +
		pY[ 2 * x − 1 ][ −2 ] + 4 * pY[ 2 * x ][ −2 ] + pY[ 2 * x + 1 ][ −2 ] + 	(8-179)
		pY[ 2 * x ][ −1 ] + 4 ) >> 3

    - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-180)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

    - pTopDsY[ 0 ] is derived as follows:

    - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] +
		pY[ −1 ][ −2 ] + 4 * pY[ 0 ][ −2 ] + pY[ 1 ][ −2 ] + 	(8-181)
		pY[ 0 ][ −1 ] + 4 ) >> 3

    - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ −1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-182)
		pY[ 1 ][ −1 ] + 2 ) >> 2

    - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] + 2 * pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 2 ) >> 2	(8-183)

    - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]		(8-184)

    - Otherwise, the following applies:

    - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

    - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −2 ] + pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −2 ] +	 2*pY[ 2 * x ][ −1 ] + 	(8-185)
		pY[ 2 * x + 1 ][ −2 ] + pY[ 2 * x + 1 ][ −1 ] + 4 ) >> 3

    - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-186)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

    - pTopDsY[ 0 ] is derived as follows:

    - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −2 ] + pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −2 ] + 2*pY[ 0 ][ −1 ] + 	(8-187)
		pY[ 1 ][ −2 ] + pY[ 1 ][ −1 ] + 4 ) >> 3

    - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-188)
		pY[ 1 ][ −1 ] + 2 ) >> 2

    - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 1 ) >> 1	(8-189)

    - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]		(8-190)

  - The variables nS, xS, yS are derived as follows:

    - If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

nS = ( ( availL && availT ) ? Min( nTbW, nTbH ) : ( availL ? nTbH : nTbW ) ) 	(8-191)

xS = 1 << ( ( ( nTbW > nTbH ) && availL && availT ) ? ( Log2( nTbW) − Log2( nTbH ) ) : 0 ) 	(8-192)

yS = 1 << ( ( ( nTbH > nTbW ) && availL && availT ) ? ( Log2( nTbH) − Log2( nTbW ) ) : 0 ) 	(8-193)

    - Otherwise if predModeIntra is equal to INTRA_L_CCLM, the following applies:

nS = numSampL		(8-194)

xS = 1		(8-195)

yS = 1		(8-196)

    - Otherwise (predModeIntra is equal to INTRA_T_CCLM), the following applies:

nS = numSampT		(8-197)

xS = 1		(8-198)

yS = 1		(8-199)

  - The variables avgY, avgC, T, minY, maxY, minC and maxC are derived as follows:

    - The variables avgY and avgC are set to 0, the variables T and minY are set equal to 1 << (BitDepthY) + 1 and the variable maxY is set equal to −1.

    - If availT is equal to TRUE, the variables minY, maxY, minC and maxC with x = 0..nS − 1 are derived as follows:

    - The variable avgY and avgC are updated as follows:

avgY += pTopDsY[ x * xS ]	(8-142)

avgC += p[ x * xS ][ −1 ]	(8-143)

    - If minY is greater than pTopDsY[ x * xS ], the following applies:

minY = pTopDsY[ x * xS ]	(8-200)

minC = p[ x * xS ][ −1 ]	(8-201)

    - If maxY is less than pTopDsY[ x * xS ], the following applies:

maxY = pTopDsY[ x * xS ]	(8-202)

maxC = p[ x * xS ][ −1 ] 	(8-203)

    - If availL is equal to TRUE, the variables minY, maxY, minC and maxC with y = 0..nS − 1 are derived as follows:

    - The variable avgY and avgC are updated as follows:

avgY += pLeftDsY[ y * yS ]	(8-142)

avgC += p[ −1 ][ y * yS ]	(8-143)

    - If minY is greater than pLeftDsY[ y * yS ], the following applies:

minY = pLeftDsY[ y * yS ]	(8-204)

minC = p[ −1 ][ y * yS ]	(8-205)

    - If maxY is less than pLeftDsY[ y * yS ], the following applies:

maxY = pLeftDsY[ y * yS ]	(8-206)

maxC = p[ −1 ][ y * yS ]	(8-207)

  - The variables T, a, b, a2, b2, k and k2 are derived as follows:

In the following, divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-221)

    - If numSampL is equal to 0, and numSampT is equal to 0, the following applies:

k = 0		(8-208)

a = 0		(8-209)

b = 1 << ( BitDepthC − 1)		(8-210)

    - Otherwise, if nTbW * nTbH is greater than or equal to 16, the following applies:

iCountShift = Log2( nS ) + ( ( availL && availT )  ?  1  :  0 )	(8 155)

avgY = ( avgY + ( 1 << ( iCountShift − 1 ) ) ) >> iCountShift	(8 155)

avgC = ( avgC + ( 1 << ( iCountShift − 1 ) ) ) >> iCountShift	(8 155)

avgY += ( 2 * avgY − minY − maxY ) >> 2	(8 155)

avgC += ( 2 * avgC − minC − maxC ) >> 2	(8 155)

diff1 = avgY − minY	(8-155)

diff2 = maxY − avgY	(8-155)

      - If both diff1 and diff2 are greater than 8, the following applies:

    - T is set to avgY

    - First set of parameters are computed as follows (diff1 is above 0):

diffC = avgC − minC	(8-212)

x = Floor( Log2( diff1 ) )	(8-213)

normDiff = ( ( diff1 << 4 ) >> x ) & 15	(8-214)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-215)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-216)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-217)

k = ( ( 3 + x − y )  <  1 )  ?  1  :  3 + x − y	(8-218)

a = ( ( 3 + x − y )  <  1 )  ?  Sign( a ) * 15  :  a	(8-219)

    - Second set of parameters are computed as follows (diff2 is above 0):

diffC = maxC − avgC	(8-212)

x = Floor( Log2( diff2 ) )	(8-213)

normDiff = ( ( diff2 << 4 ) >> x ) & 15	(8-214)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-215)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-216)

a2 = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-217)

k2 = ( ( 3 + x − y )  <  1 )  ?  1  :  3 + x − y	(8-218)

a2 = ( ( 3 + x − y )  <  1 )  ?  Sign( a2 ) * 15  :  a2	(8-219)

    - If a is 0, a is set to a2 and k set to k2

    - If a2 is 0, a2 is set to a and k2 set to k

    - b and b2 are computed as follows:

b = avgC − ( ( a * avgY ) >> k )	(8-220)

b2 = avgC − (  ( a2 * avgY ) >> k2 )	(8-220)

      - If either diff1 or diff2 is strictly below 8, then next point applies.

    - Otherwise, the following applies:

diff = maxY − minY	(8-211)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-212)

x = Floor( Log2( diff ) )	(8-213)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-214)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-215)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-216)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-217)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-218)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-219)

b = minC − ( ( a * minY ) >> k )	(8-220)

divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-221)

    - Otherwise (diff is equal to 0), the following applies:

k = 0	(8-222)

a = 0	(8-223)

b = minC	(8-224)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

    - If pDsY[ x ][ y ] is less than T, then:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-225)

    - Otherwise:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a2 ) >> k2 ) + b2 )	(8-161)

