| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0241 |
| --- | --- |

| Title: | CE3: Results of Test CE3-1.8 on multiple model LM using piecewise linear model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Guillaume Laroche Jonathan Taquet Patrice Onno | Tel: Email: | christophe.gisquet@crf.canon.fr |
| Source: | Canon Research France |  |  |

_____________________________

# Abstract

At the 13th JVET meeting in Marrakech, a new method was proposed to perform the multiple-model linear mode using straight lines. This goes through the definition of a knee point between the two lines, derived from the luma and chroma sample averages and extremum points. The method only operates on larger blocks to not affect the linear model worst case. The present contribution reports the results for two different sets of block size constrains.

# Technical content

While the Multiple-Model LM mode presents attractive coding efficiency, several elements of its design were subject to complexity considerations, which are discussed hereafter.

## Processing stage / dependency problem

Computing averages (first stage) over the samples must be performed prior to any other pass over them to e.g. classify them or update whatever computations (second stage) are needed for deriving the model(s) parameters. Depending on the implementation, this can require additional buffering, and/or several stages in the pipeline.

## Discontinuity problem in unconstrained MMLM

The “MMLM” mode, and derivatives, currently being studied in CE3 determines two linear models, one per segment, for two segments defined by the average of the downsampled reconstructed luma samples over a selection of the block borders. Initially based on LMS solutions, a source of inefficiency is the discontinuity between the two lines. A solution is to determine a point in the middle, where either one or the two models are potentially the most wrong.

Figure 1: knee in the cloud

In the above, that point, T, indicated by an arrow to the knee between the 2 lines, offers a continuity, as well as define the new threshold used to select one model or the other.

This knee is simply determined by a computation based on the luma and chroma averages together with their minimum and maximum, in a single pass, thus solving both the discontinuity and stage issues. The formulae used to determine T are simply:

YT = Ymean + ( (2*Ymean – Ymax – Ymin) >> 2 )

CT = Cmean + ( (2*Cmean – Cmax – Cmin) >> 2 )

## Throughput requirements

As the original MMLM mode applies to all block sizes, it increases the complexity of cross-component prediction method worst case (smallest block size). The above ‘LM in the middle’ method can be applied to any regular CCLM mode solely based on the block size being processed.

Indeed, the above method is more likely to work if indeed CCLM in the middle is wrong, which is more likely the smaller the luma interval is. The method therefore suffers less from block size restrictions. Current smallest number of samples for any CCLM-coded block is 2x2, setting their required throughput. Relatively, the ‘LM in the middle’ method suffers minimally from working only on blocks of larger size, thereby not impacting the throughput worst case.

As a result, the average block size for the CCLM modes increases, meaning these modes become more realistic for an encoder that is restricted in the depth of its analysis.

## Other considerations

Beside the block size restrictions, if either segment between min and middle, or middle and max, is too small (below or equal to 8), then the operation reverts to a single model. Additionally, if one slope is 0, it is set to the other value of the other one.

Finally, the switch to MMLM is implicit, based on block size, meaning no additional signalling is present.

## Test definitions

Test 1 uses the formulae of section 1.2, and a minimal MMLM block size of 16 samples (i.e. blocks of sizes 8x2, 2x8 and 4x4).

Test 2 further constrains normatively the minimal CCLM size to 16 samples as a potential other operating point from a decoder standpoint. As a consequence, the minimal MMLM block size is increased to 64 samples. It should be noted that only the MMLM methods are proposed here, test 2 being meant more as an example of more stringent restrictions for all chroma-from-luma modes. It is in particular not proposed to constrain normatively the minimal CCLM size to 16 samples.

## Complexity in terms of number of samples

|  | Comparisons | Additions | Divisions |
| --- | --- | --- | --- |
| CCLM | 2N (Min1, Max2) | 0 | 0 |
| MMLM (JVET-M0097) | 4N = 2N (Min1, Max2) + N (demux) + N (Min2, Max1) | N−1 (Yavg) | 1 (table) |
| CE3-1.8 | 2N (Min1, Max2) | 2*(N-1) (Yavg, Cavg) | 1 (table) |

# Results for VTM4.0

In general, decoder runtime is heavily fluctuant and should not be considered reliable.

## Test 1

## Test 2

# Conclusion

Test1 showed a competitive performance with a contained complexity, with a throughput asserted to be similar to the CCLM mode. In addition, test 2 showcases a reduction in complexity, where all minimal block sizes for CCLM methods are increased, while keeping similar coding efficiency. It is therefore suggested to adopt test1, independently of any decision on block size restrictions, at this meeting.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).