| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0242 |
| --- | --- |

| Title: | CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear Adaptive Loop Filter. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution gives the results of the 4 tests (CE5-3.1, CE5-3.2, CE5-3.3, CE5-3.4) related to the Non-Linear Adaptive Loop filtering as presented in JVET-M0385 where adaptive clipping operation on the input samples values are applied during the Adaptive Loop Filtering in VTM4.0. The goal of this adaptive clipping is to cap the difference between the input sample value to be filtered and the neighboring input samples of the filter.

It is reported that for test CE5-3.1, the average BDR gains and enc./dec. runtimes are respectively -0.45%/-0.35%/-0.59% - 101%/103% for the AI configuration, -0.67%/-1.01%/-1.12% - 100%/104% for the RA configuration, -0.75%/-0.52%/-0.86% - 103%/101% for the Low Delay B configuration, and -1.07%/-1.43%/-1.32% - 108%/102% for the Low Delay P configuration.

It is reported that for test CE5-3.2, the average BDR gains and enc./dec. runtimes are respectively -0.44%/-0.49%/-0.69% - 101%/102% for the AI configuration, -0.70%/-1.00%/-1.04% - 100%/103% for the RA configuration, -0.72%/-0.94%/-1.22% - 104%/102% for the Low Delay B configuration, and -1.02%/-1.44%/-1.58% - 108%/104% for the Low Delay P configuration.

It is reported that for test CE5-3.3, the average BDR gains and enc./dec. runtimes are respectively -0.35%/-0.39%/-0.62% - 100%/100% for the AI configuration, -0.50%/-0.96%/-0.99% - 99%/99% for the RA configuration, -0.51%/-0.51%/-0.33% - 101%/99% for the Low Delay B configuration, and -0.75%/-1.37%/-0.99% - 104%/101% for the Low Delay P configuration.

It is reported that for test CE5-3.4, the average BDR gains and enc./dec. runtimes are respectively +0.12%/-0.43%/-0.68% - 99%/99 for the AI configuration, +0.17%/-1.01%/-1.09% - 98%/99 for the RA configuration, 0.00%/-0.37%/-0.90% - 100%/97 for the Low Delay B configuration, and -0.11%/-1.32%/-1.45% - 102%/98% for the Low Delay P configuration.

# Introduction

## Adaptive Loop Filtering

In VTM4.0, the filtering process of the Adaptive Loop Filter, is performed as follows:

Ox,y=i,jwi,j.Ix+i,y+j,	(1)

where samples I(x+i,y+j) are input samples, O(x,y) is the filtered output sample (i.e. filter result), and wi, j denotes the filter coefficients. In practice, in VTM4.0 it is implemented using integer arithmetic for fixed point precision computations:

Ox,y=i=-L2L2j=-L2L2wi, j.Ix+i,y+j+64≫7,	(2)

where L denotes the filter length, and where wi, j are the filter coefficients in fixed point precision.

# Overview of the Non-linear ALF proposed in JVET-M0385.

## Filtering reformulation

Equation (1) can be reformulated, without coding efficiency impact, in the following expression:

Ox,y=Ix,y+(i,j)≠(0,0)wi, j.Ix+i,y+j-Ix,y,	(3)

where wi, j are the same filter coefficients as in equation (1) [excepted w0, 0 which is equal to 1 in equation (3) while it is equal to 1-(i,j)≠(0,0)wi, j in equation (1)].

## Modified filter

Using this above filter formula of (3), we can easily introduce non linearity to make ALF more efficient by using a simple clipping function to reduce the impact of neighbor sample values (Ix+i,y+j) when they are too different with the current sample value (Ix,y) being filtered.

In this proposal, the ALF filter is modified as follows:

O'x,y=Ix,y+(i,j)≠(0,0)wi, j.KIx+i,y+j-Ix,y,k(i,j),	(4)

where Kd,b=min⁡(b,max-b,d) is the clipping function, and k(i,j) are clipping parameters, which depends on the(i,j) filter coefficient. The encoder performs the optimization to find the best k(i,j).

In the CE5.3 implementation, the clipping parameters ki,j are specified for each ALF filter, one clipping value is signaled per filter coefficient. It means that up to 12 clipping values can be signalled in the bitstream per Luma filter and up to 6 clipping values for the Chroma filter.

In order to limit the signaling cost and the encoder complexity, we limit the evaluation of the clipping values to a small set of possible values. In the proposal, we only use 4 fixed values which are the same for INTER and INTRA tile groups.

Because the variance of the local differences is often higher for Luma  than for Chroma, we use two different sets for the Luma and Chroma filters. We also include the maximum sample value (here 1024 for 10 bits bit-depth) in each set, so that clipping can be disabled if it is not necessary.

The sets of clipping values used in the CE5-3 tests are provided in the Table 1. The 4 values have been selected by roughly equally splitting, in the logarithmic domain, the full range of the sample values (coded on 10 bits) for Luma, and the range from 4 to 1024 for Chroma.

More precisely, the Luma table of clipping values have been obtained by the following formula:

AlfClipL=roundM1NN-n+1  for n∈ 1..N], with M=210 and N=4.

Similarly, the Chroma tables of clipping values is obtained according to the following formula:

AlfClipC=roundA.MA1N-1N-n  for n∈ 1..N], with M=210, N=4 and A=4.

|  | INTRA/INTER tile group |
| --- | --- |
| LUMA | { 1024, 181, 32, 6 } |
| CHROMA | { 1024, 161, 25, 4 } |

Table 1: Authorized clipping values

The selected clipping values are coded in the “alf_data” syntax element by using a Golomb encoding scheme corresponding to the index of the clipping value in the above Table 1. This encoding scheme is the same as the encoding scheme for the filter index.

## Encoder modifications

At encoder side, instead of computing one covariance matrix E and one cross-covariance matrix y per CTU and per frame, we compute NK×NK clipping covariance matrices Ecab and NK clipping cross-covariance matrices, yc[c]. NK is the number of allowed clipping values. Ecab is the clipping covariance matrix between inputs using a-th clipping value (same clipping for each input difference, no clipping for center value) and inputs using b-th clipping value. yc[c] is the cross-covariance matrix between expected output and inputs using c-th clipping value (same clipping for each input difference, no clipping for center value).

# CE5.3 description

## Test CE5-3.1

This test corresponds to the non-linear ALF as proposed in JVET-M0385 with SIMD implementation in VTM4.0. See the corresponding VVC draft text in VTM4.0 is attached in Draft_text-CE5.3.1_all_v1.0.pdf.

## Test CE5-3.2

Test CE5-3.2 uses the non-linear ALF of test CE5-3.1 with additional NL-ALF signaling flags allowing switching between linear and non-linear filters, so that when non-linear filter is not used, unnecessary clipping parameters are not signalled. A new encoder optimization algorithm exploits this additional signaling possibility to get the best BDR performance. One flag is used to enable the NL-ALF filter for the Luma component. Another flag is used to enable or disable the NL-ALF for the two Chroma components.

This provides a slightly better rate distortion compromise compared to the test CE5-3.1 while increasing a bit the encoder complexity. However, it does not change the decoder complexity: the number of ALF filtering operations remains exactly the same as the test CE5-3.1. This test offers a different compromise between encoder complexity and coding efficiency compared to test CE5-3.1. See the corresponding VVC draft text in VTM4.0 is attached in Draft_text-CE5.3.2_all_v1.0.pdf.

## Test CE5-3.3

Test CE5-3.3 is based on the test CE5-3.1 where the clipping operations performed on the input samples are significantly reduced. For the 7x7 Luma filter, the number of clipping operations is reduced from 24 clippings to 8 clippings. For the 5x5 Chroma filter, the clipping operations are also reduced, and the clipping operations are only applied on the 8 outer samples. This enables to reduce the number of clippings from 12 clippings to 8 clippings for the Chroma filter. The positions of the reduced number of clipping operations are illustrated by an ‘x’ in the filter patterns shown in Figure 1. This test offers a different compromise between decoding complexity and coding efficiency compared to test CE5-3.1. See the corresponding VVC draft text in VTM4.0 is attached in Draft_text-CE5.3.3_all_v1.0.pdf.

Figure 1 – Positions of the clipping operations with CE5-3.3 test for Luma (left) and for Chroma (right).

## Test CE5-3.4

Test CE5-3.4 is based on the test CE5.3.1 where the clipping operations performed on the input samples are also significantly reduced. Instead of using a 7x7 non-linear Luma filter, a 5x5 non-linear Luma filter is used. The number of clipping operations is thus reduced from 12 clippings to 8 clippings. Reducing the size of the filter from 7x7 to 5x5 also reduces the number of multiplications required by the NL-ALF filtering from 12 to 6 and allows reducing the number of samples line buffers. This test offers another compromise between decoding complexity and coding efficiency compared to test CE5-3.1. See the corresponding VVC draft text in VTM4.0 is attached in Draft_text-CE5.3.4_all_v1.0.pdf.

# Results

## Test CE5-3.1

## Test CE5-3.2

## Test CE5-3.3

## Test CE5-3.4

## Complexity comparisons

In this section average BDR gains versus Encoder & Decoder runtime increase of CE5-3 tests are compared together in the CTC configurations, using VTM4.0 as a reference, and. using average BDR gain obtained with a 14/16 weight for Luma channel BDR gain and a 1/16 weight for each Chroma channel BDR gain.

Figure 2 – BDR gains versus Encoder (left) / Decoder (right) runtime increase of CE5-3 tests compared to VTM4.0 in CTC
All Intra configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 3 – BDR gains versus Encoder (left) / Decoder (right) runtime increase of CE5-3 tests compared to VTM4.0 in CTC
Random Access configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 4 – BDR gains versus Encoder (left) / Decoder (right) runtime increase of CE5-3 tests compared to VTM4.0 in CTC
Low Delay B configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 5 – BDR gains versus Encoder (left) / Decoder (right) runtime increase of CE5-3 tests compared to VTM4.0 in CTC
Low Delay P configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

## Complexity analysis

### Memory for storing filters

- CE5-3.1/CE5-3.2:

25 classes x 12 coefficients x 8 bits (=2400)

+ 25 classes x 12 clipping x 2 bits (index) (= 600)

+ 1 chroma filter x 6 coefficients x 8 bits (=48)

+ 1 chroma filter x 6 clipping x 2 bits (index) (=12)

= 3060 bits = 382 bytes

382 bytes x 32 ALF filter sets (APS) = 12224 bytes.

- CE5-3.3:

25 classes x 12 coefficients x 8 bits (=2400)

+ 25 classes x 4 clipping x 2 bits (index) (=200)

+ 1 chroma filter x 6 coefficients x 8 bits (=48)

+ 1 chroma filter x 4 clipping x 2 bits (index) (=8)

= 2656 bits = 332 bytes

332 bytes x 32 ALF filter sets (APS) = 10624 bytes.

- CE5-3.4:

25 classes x 6 coefficients x 8 bits (=1200)

+ 25 classes x 6 clipping x 2 bits (index) (=300)

+ 1 chroma filter x 6 coefficients x 8 bits (=48)

+ 1 chroma filter x 6 clipping x 2 bits (index) (=12)

= 1560 bits = 195 bytes

195 bytes x 32 ALF filter sets (APS) = 6240 bytes.

### Operations of filtering process per Luma sample

- CE5-3.1/CE5-3.2:

3 Comp (transposeIdx)

+ 24 Add (24 neighbor – curr)

+ 48 Comp (24 Clip)

+ 12 Add (addition of symmetric positions)

+ 12 Mult (filter coefficient multiplication)

+ 11 Add (addition of multiplication result for each coefficient)

+ 1 Add (addition of fixed point 0.5)

+ 1 Shift (round fixed point value to integer)

+ 1 Add (addition of center value * center filter coefficient=1)

+ 1 Comp (final clipping of positive value)

=

| Add | 49 |
| --- | --- |
| Comp | 52 |
| Mult | 12 |
| Shift | 1 |

- CE5-3.3:

3 Comp (transposeIdx)

+ 24 Add (24 neighbor – curr)

+ 16 Comp (8 Clip)

+ 12 Add (addition of symmetric positions)

+ 12 Mult (filter coefficient multiplication)

+ 11 Add (addition of multiplication result for each coefficient)

+ 1 Add (addition of fixed point 0.5)

+ 1 Shift (round fixed point value to integer)

+ 1 Add (addition of center value * center filter coefficient=1)

+ 1 Comp (final clipping of positive value)

=

| Add | 49 |
| --- | --- |
| Comp | 20 |
| Mult | 12 |
| Shift | 1 |

- CE5-3.4:

3 Comp (transposeIdx)

+ 12 Add (12 neighbor – curr)

+ 24 Comp (12 Clip)

+ 6 Add (addition of symmetric positions)

+ 6 Mult (filter coefficient multiplication)

+ 5 Add (addition of multiplication result for each coefficient)

+ 1 Add (addition of fixed point 0.5)

+ 1 Shift (round fixed point value to integer)

+ 1 Add (addition of center value * center filter coefficient=1)

+ 1 Comp (final clipping of positive value)

=

| Add | 25 |
| --- | --- |
| Comp | 28 |
| Mult | 6 |
| Shift | 1 |

### Operations of filtering process per Chroma sample

- CE5-3.1/CE5-3.2/CE5-3.4:

12 Add (12 neighbor – curr)

+ 24 Comp (12 Clip)

+ 6 Add (addition of symmetric positions)

+ 6 Mult (filter coefficient multiplication)

+ 5 Add (addition of multiplication result for each coefficient)

+ 1 Add (addition of fixed point 0.5)

+ 1 Shift (round fixed point value to integer)

+ 1 Add (addition of center value * center filter coefficient=1)

+ 1 Comp (final clipping of positive value)

=

| Add | 25 |
| --- | --- |
| Comp | 25 |
| Mult | 6 |
| Shift | 1 |

- CE5-3.3:

12 Add (12 neighbor – curr)

+ 16 Comp (8 Clip)

+ 6 Add (addition of symmetric positions)

+ 6 Mult (filter coefficient multiplication)

+ 5 Add (addition of multiplication result for each coefficient)

+ 1 Add (addition of fixed point 0.5)

+ 1 Shift (round fixed point value to integer)

+ 1 Add (addition of center value * center filter coefficient=1)

+ 1 Comp (final clipping of positive value)

=

| Add | 25 |
| --- | --- |
| Comp | 17 |
| Mult | 6 |
| Shift | 1 |

# Conclusion

These four CE5.3 tests demonstrate different compromise between the performance and the computation complexity of the Non-Linear Adaptive Loop Filter proposed in JVET-M0385.

Test CE5.3.1 and CE5.3.2 deliver the best BDR performance without significantly impacting the encoder and decoder runtimes.

Test CE5.3.3 reduces the complexity by limiting the number of clipping operation to be performed on the Luma and Chroma samples. Basically, the BDR performance dropped by 20% to 30% compared to Tests CE5-3.1 and CE5-3.2 with negligible runtime increase compared to VTM4.0.

Test CE5.3.4 reduces the number of the clipping operations by using a 5x5 filter instead of a 7x7 for the Luma samples. From a BDR point of view, the performance are slightly below those of VTM4.0. It is also important to note that this test enables to reduce the size of the line buffer for ALF.

We recommend to adopt the non–linear ALF in the next version of the VVC draft by considering the different complexity/performance compromises illustrated by these CE tests.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).