| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0243 |
| --- | --- |

| Title: | Non-CE5: Non-Linear ALF with alternative filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to extend the Non-Linear ALF concept that was introduced in JVET-M0385. More precisely, this contribution proposes to add some syntax elements to enable the use of alternative ALF Luma filter sets and alternative Chroma filters, and to select them on a per filter, per CTU basis. For each CTU, if the ALF filtering is enabled on Luma component, an additional Luma filter set alternative index is signalled for each ALF Luma filter index. Similarly, for each CTU, for each Chroma component on which ALF filtering is enabled, an alternative Chroma filter index is signalled to select a given filter for the Chroma components.

It is reported that against the VTM4.0, the average Luma/Cb/Cr BDR gains are: 
with Test-1 -0.37%/-1.22%/-1.81% for AI, -0.85%/-2.90%/-3.24% for  RA, -1.23%/-3.78%/-4.46% for LB, -1.69%/-4.94%/-4.89% for LP; 
with Test-2 -0.37%/-1.22%/-1.81% for AI, -0.78%/-2.94%/-3.27% for RA, -1.04%/-3.98%/-4.58% for LB, -1.52%/-4.92%/-5.07% for LP; and 
with Test-3 -0.27%/-1.24%/-1.82% for AI, -0.64%/-2.89%/-3.19% for RA, -0.96%/-3.74%/-4.35% for LB, -1.39%/-4.99%/-5.22% for LP.

# Introduction

In JVET-M0385, an adaptive clipping operation is performed on the input samples values of the Adaptive Loop Filter. The goal of this adaptive clipping is to introduce non-linearity to cap the difference between the central input sample value and the other neighbor input sample values used during the ALF filtering process.

In VTM4.0, the ALF filtering can be expressed as follows:

Ox,y=i,jwi,j.Ix+i,y+j,	(1)

where samples I(x+i,y+j) are input samples, O(x,y) is the filtered output sample (i.e. filter result), and wi, j denotes the filter coefficients. In practice, in VTM4.0 it is implemented using integer arithmetic for fixed point precision computations:

Ox,y=i=-L2L2j=-L2L2wi, j.Ix+i,y+j+64≫7,	(2)

where L denotes  the filter length, and where wi, j are the filter coefficients in fixed point precision.

Equation (1) can be reformulated, without coding efficiency impact, in the following expression:

Ox,y=Ix,y+(i,j)≠(0,0)wi, j.Ix+i,y+j-Ix,y,	(3)

where wi, j are the same filter coefficients as in equation (1) [excepted w0, 0 which is equal to 1 in equation (3) while it is equal to 1-(i,j)≠(0,0)wi, j in equation (1)].

Using this above filter formula of (3), we can easily introduce nonlinearity to make ALF more efficient by using a simple clipping function to reduce the impact of neighbor sample values (Ix+i,y+j) when they are too different with the current sample value (Ix,y) being filtered.

In JVET-M0385, the ALF filter is modified as follows:

O'x,y=Ix,y+(i,j)≠(0,0)wi, j.KIx+i,y+j-Ix,y,k(i,j),	(4)

where Kd,b=min⁡(b,max-b,d) is the clipping function, and k(i,j) are clipping parameters, which depends on the(i,j) filter coefficient. The encoder performs the optimization to find the best k(i,j).

With ALF in VTM4.0, the total number of operations per filtered Luma sample is 13 multiplications and 13 additions per filtered Luma sample; and the total number of operations per filtered Chroma sample it 7 multiplications, 7 additions. We may also add one addition and one bitshift for fixed point computational rounding, plus one clipping of the final result.

Using JVET-M0385 NL-ALF, the total number of operations per filtered Luma sample can be: 12 multiplications, 13 additions, 24 subtractions, 24 clippings; and the total number of operations per filtered Chroma sample can be 6 multiplications, 7 additions, 12 subtractions, 12 clippings. We may also add one addition and one bitshift for fixed point computational rounding, plus one clipping of the final result.

# Proposed changes

In this contribution, it is proposed, in addition to NL-ALF, to

- enable signaling and using alternative Luma filter sets, to provide more than one set of Luma filters for the NL-ALF processing of the Luma component of a tile group,

- enable signaling and using alternative Chroma filters, to provide more than one Chroma filter for the NL-ALF processing of each Chroma component of a tile group,

- enable selecting the alternative filters to use in each CTU.

## Alternative Luma filter sets

The number of alternative filter sets is indicated in the tile group header (it could be in APS when APS will be implemented in VTM), as well as the filter coefficients (and the associated clipping values) of each filter for each alternative Luma filter set. For this contribution, the maximum number of alternative Luma filter sets is fixed to MAX_NUM_ALF_ALTERNATIVES_LUMA = 8 per tile group.

The signaling of each alternative Luma filter set is performed in a similar but not exact same way as the signaling of the set of Luma filters in JVET-M0385 (i.e. as the signaling of the set of Luma filter coefficients in VTM4.0, plus the signaling of the clipping values of each filter).

The signaling of the first alternative Luma filter set is the same as the signaling of the set of Luma filters in JVET-M0385. Thus each class index is mapped to a filter index in the first alternative luma filter set using a 25 entries lookup table (which is equivalent to ‘alf_luma_coeff_delta_idx[ filtIdx ]’ lookup table in VVC draft 4, with ‘filtIdx’ corresponding to the class index in VTM software).

From the second alternative Luma filter set, each current alternative Luma filter set is constrained to start from the same class/filter merging as its previous Luma filter set. Then, instead of mapping each class index to a filter index, each filter index of the previous alternative Luma filter set is mapped to a filter index in the current alternative Luma filter set using a lookup table with a number of entries equal to the number of filters in the previous alternative Luma filter set. Thus, with each alternative Luma filter set alf_luma_coeff_al_merge_idx[ altIdx ][ filtIdx ] provides a table indicating a merging of the filter indexes. Such kind of merging is illustrated with an example in Figure 1.

Figure 1 – Example illustrating the proposed ‘alf_luma_coeff_alt_merge_idx’ signaling as a replacement of ‘alf_luma_coeff_delta_idx’ signaling for alternative Luma filter sets.

The class index to filter index mapping ‘alf_luma_coeff_delta_idx[ altIdx ][ filtIdx ]’ is derived as follow:

- alf_luma_coeff_delta_idx[ 0 ][ filtIdx ] is first set equal to alf_luma_coeff_alt_merge_idx[ 0 ][ filtIdx ] for filtIdx = 0..24 – 1.

- Then for altIdx = 1..alf_luma_num_alts_minus1,

  - for filtIdx = 0..24,

    - alf_luma_coeff_delta_idx[ altIdx ][ filtIdx ] is set equal to alf_luma_coeff_alt_merge_idx[ altIdx ][ alf_luma_coeff_delta_idx[ altIdx – 1 ][ filtIdx ] ].

## Alternative Chroma filters

The number of alternative Chroma filters is indicated in the tile group header (it could be in APS when APS will be implemented in VTM), as well as the filter coefficients (and the associated clipping values) of each alternative Chroma filter. For this contribution, the maximum number of alternative Chroma filters is fixed to MAX_NUM_ALF_ALTERNATIVES_CHROMA = 8 per tile group.

## Alternative filters selection

In each CTU, if ALF is enabled on Luma component, a Luma filter set alternative index is signalled for each filter index of the first Luma filter set alternative (e.g. for the example of Figure 1, N0-1 alternative index are provided in each CTU). And in each CTU, for each Chroma component on which ALF is enabled, a Chroma filter alternative index is signalled.

The signaling of a filter (set) alternative index is performed, by entropy with CABAC the bits of its unary representation. In total, three CABAC contexts are used. The first context is shared with all the ALF Luma filter indexes and is used for the signaling of the Luma filter set alternative indexes. The second context is used for the signaling of the Cb Chroma alternative filter index, and the third context is used for the signaling of the Cr Chroma alternative filter index.

For Luma component, if ALF is enabled on a CTU and the classification index of a 4x4 Luma samples block is classIdx, the filter index in first Luma alternative filter set is filtIdx = alf_luma_coeff_delta_idx[ 0 ][ classIdx ]. altIdx is the alternative Luma filter index provided in the CTU for the filter index filtIdx.

The filter coefficients and clipping values to use for processing the 4x4 Luma samples block are the ones provided for the filter with index alf_luma_coeff_delta_idx[ altIdx ][ classIdx ] in the alternative Luma filter set with index altIdx.

For a Chroma component, if ALF is enabled on a CTU and the alternative filer index is altIdx, the filter coefficients and clipping values to use for processing the CTU are the ones provided for the alternative Chroma filter with index altIdx.

# Proposed tests

This contribution provides three tests to show achievable BDR gains with the proposed NL-ALF extension.

- Test-1 and Test-2 add the proposed syntax to signal alternative filter sets to an additional signalling where linear ALF vs non-linear ALF (NL-ALF) can be used. Test-1 and Test-2 only differ by the encoder complexity in order to reach different BDR gains/complexity trade-offs,

- Test-3 adds the proposed syntax to signal alternative filter sets to an additional signalling and a reduced number of clipping operations for NL-ALF.

# Results

In the following sections, the results of the three tests, are provided, as well as runtime comparisons of the three tests.

## Test-1

## Test-2

## Test-3

## Runtime comparisons

In this section average BDR gains versus Encoder & Decoder runtime increase of Test-1, Test-2 and Test3 are compared together in the CTC configurations, using VTM4.0 as a reference, and. using average BDR gain obtained with a 14/16 weight for Luma channel BDR gain and a 1/16 weight for each Chroma channel BDR gain.

Figure 2 – BDR gains versus Encoder and Decoder runtime increase of JVET-N0243 tests compared to VTM4.0 in CTC All Intra configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 3 – BDR gains versus Encoder and Decoder runtime increase of JVET-N0243 tests compared to VTM4.0 in CTC Random Access configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 4 – BDR gains versus Encoder and Decoder runtime increase of JVET-N0243 tests compared to VTM4.0 in CTC Low Delay.B configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 5 – BDR gains versus Encoder and Decoder runtime increase of JVET-N0243 tests compared to VTM4.0 in CTC Low Delay.P configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

# Conclusion

In this contribution, in order to improve the non-linear ALF perfomance, it is proposed to enable signaling of alternative filter sets for Luma and alternative filters for Chroma. The three tests in this contribution demonstrate the BDR gains using the syntax for enabling alternative Luma filter sets and alternative Chroma filters.

It is recommended to adopt this Non-Linear ALF extension in the next version of the VVC draft by considering the good complexity/performance compromise illustrated by  the three tests presented herein.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).