| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0243 |
| --- | --- |

| Title: | Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to extend the Non-Linear ALF concept that was introduced in JVET-M0385. More precisely, this contribution proposes to add some syntax elements to enable the use of alternative ALF Luma filter sets and alternative Chroma filters, and to select them on a per filter, per CTU basis. For each CTU, if the ALF filtering is enabled on Luma component, an additional Luma filter set alternative index is signalled for each ALF Luma filter index. Similarly, for each CTU, for each Chroma component on which ALF filtering is enabled, an alternative Chroma filter index is signalled to select a given filter for the Chroma components.

It is reported that against the VTM4.0, the average Luma/Cb/Cr BDR gains are: 
with Test-1 -0.37%/-1.22%/-1.81% for AI, -0.85%/-2.90%/-3.24% for  RA, -1.23%/-3.78%/-4.46% for LB, -1.69%/-4.94%/-4.89% for LP; 
with Test-2 -0.37%/-1.22%/-1.81% for AI, -0.78%/-2.94%/-3.27% for RA, -1.04%/-3.98%/-4.58% for LB, -1.52%/-4.92%/-5.07% for LP; and 
with Test-3 -0.27%/-1.24%/-1.82% for AI, -0.64%/-2.89%/-3.19% for RA, -0.96%/-3.74%/-4.35% for LB, -1.39%/-4.99%/-5.22% for LP.

# Introduction

In JVET-M0385, an adaptive clipping operation is performed on the input samples values of the Adaptive Loop Filter. The goal of this adaptive clipping is to introduce non-linearity to cap the difference between the central input sample value and the other neighbor input sample values used during the ALF filtering process.

In VTM4.0, the ALF filtering can be expressed as follows:

Ox,y=i,jwi,j.Ix+i,y+j

I(x+i,y+j) O(x,y)wi, j

Ox,y=i=-L2L2j=-L2L2wi, j.Ix+i,y+j+64≫7

wi, j

Ox,y=Ix,y+(i,j)≠(0,0)wi, j.Ix+i,y+j-Ix,y

wi, j w0, 01-(i,j)≠(0,0)wi, j

On=wc.Inc+i=0i<cwi.Ini-+Ini+.

On is the output sample of the filter at a position n. Inc is the filter input sample at the same position n as the output sample (and is the centre input sample in the filter pattern). Ini- is the i-th (in raster scan order) neighbouring input sample of Inc in the filter pattern. Ini+ is the neighbouring input sample at a mirrored spatial position of Ini- about the centre position of Inc. wi is the ALF filter coefficient associated with the neighbouring input samples Ini- and Ini+. wc is the ALF filter coefficient for the centre input sample Inc . And c is the number of encoded filter coefficients. The values of i correspond to the index values and the associated positions of Ini- and Ini+ correspond to the index values with their superscript sign (“+” or “-“) in the filter patterns of Figure 1: for Luma filters, c=12 and for Chroma filters, c=6.

Figure 1 – ALF filter patterns for Luma (left) and for Chroma (right).

Using same notations, NL-ALF filtering in JVET-M0385 can be expressed as follows:

On=Inc+i=0i<cwi.KIni--Inc,ki+KIni+-Inc,ki.

where ki is a filter clipping Ix+i,y+j)Ix,y

parameter associated with the filter

O'x,y=Ix,y+(i,j)≠(0,0)wi, j.KIx+i,y+j-Ix,y,k(i,j)

coefficient wi, and where Kd,b=min⁡(b,max-b,d)minb,max-b, d is the clipping functionk(i,j). The filter clipping parameters(i,j)k(i,j) ki for i in 0; c-1 are provided/encoded for each filter.

With ALF in VTM4.0, the total number of operations per filtered Luma sample is 13 multiplications and 13 additions per filtered Luma sample; and the total number of operations per filtered Chroma sample it 7 multiplications, 7 additions. We may also add one addition and one bitshift for fixed point computational rounding, plus one clipping of the final result.

Using JVET-M0385 NL-ALF, the total number of operations per filtered Luma sample can be: 12 multiplications, 13 additions, 24 subtractions, 24 clippings; and the total number of operations per filtered Chroma sample can be 6 multiplications, 7 additions, 12 subtractions, 12 clippings. We may also add one addition and one bitshift for fixed point computational rounding, plus one clipping of the final result.

In CE5-3, four tests were performed, showing different complexity versus coding gains tradeoffs. In this contribution, we focus on the first three tests (and more especially on CE5-3.2 and CE5-3.3):

| Test | Description |
| --- | --- |
| CE5-3.1 | Non-linear ALF (NL-ALF) as in JVET-M0385. |
| CE5-3.2 | Test CE5-3.1 + additional NL-ALF signaling + encoder optimization. |
| CE5-3.3 | TEST CE5-3.1 + reduced number of clipping operations for NL-ALF. |

Test CE5-3.1 corresponds to the non-linear ALF as proposed in JVET-M0385 with SIMD implementation in VTM4.0.

Test CE5-3.2 uses the non-linear ALF of test CE5-3.1 with additional NL-ALF signaling flags allowing switching between linear and non-linear filters (L-ALF vs NL-ALF). Using linear filter enable not signaling clipping parameters when the cost of that signaling is estimated to be too high by the encoder optimization algorithm. This provides a better rate distortion compromise compared to the test CE5-3.1. It slightly increases the encoder complexity, but it does not change the decoder complexity: the number of ALF filtering operations remains exactly the same as the test CE5-3.1. This is because, in that test, the implementation of linear and non-linear ALF is unified so that linear filters are a special case of non-linear filters, with all clipping values set to 1024 (i.e. clipping does nothing with 10 bits internal bitdepth). This test offers a different compromise between encoder complexity and coding efficiency compared to test CE5-3.1

Test CE5-3.3 is based on the test CE5-3.1 where the clipping operations performed on the input samples are significantly reduced. For the 7x7 Luma filter, the number of clipping operations is reduced from 24 clippings to 8 clippings. For the 5x5 Chroma filter, the clipping operations are also reduced, and the clipping operations are only applied on the 8 outer samples. This enables to reduce the number of clippings from 12 clippings to 8 clippings for the Chroma filter. The positions of the reduced number of clipping operations are illustrated by an ‘x’ in the filter patterns shown in Figure 2.This test offers a different compromise between decoding complexity and coding efficiency compared to test CE5-3.1.

Figure 2 – Positions of the clipping operations with CE5-3.3 test for Luma (left) and for Chroma (right).

# Proposed changes

In this contribution, it is proposed, in addition to NL-ALF, to

- enable signaling and using alternative Luma filter sets, to provide more than one set of Luma filters for the NL-ALF processing of the Luma component of a tile group,

- enable signaling and using alternative Chroma filters, to provide more than one Chroma filter for the NL-ALF processing of each Chroma component of a tile group,

- enable selecting the alternative filters to use in each CTU.

## Alternative Luma filter sets

The number of alternative filter sets is indicated in the tile group header (it could be in APS when APS will be implemented in VTM), as well as the filter coefficients (and the associated clipping values) of each filter for each alternative Luma filter set. For this contribution, the maximum number of alternative Luma filter sets is fixed to MAX_NUM_ALF_ALTERNATIVES_LUMA = 8 per tile group.

The signaling of each alternative Luma filter set is performed in a similar but not exact same way as the signaling of the set of Luma filters in CE5-3 (i.e. as the signaling of the set of Luma filter coefficients in VTM4.0, plus the signaling of the clipping values of each filter).

The signaling of the first alternative Luma filter set is the same as the signaling of the set of Luma filters in CE5-3. Thus each class index is mapped to a filter index in the first alternative luma filter set using a 25 entries lookup table (which is equivalent to ‘alf_luma_coeff_delta_idx[ filtIdx ]’ lookup table in VVC draft 4, with ‘filtIdx’ corresponding to the class index in VTM software).

From the second alternative Luma filter set, each current alternative Luma filter set is constrained to start from the same class/filter merging as its previous Luma filter set. Then, instead of mapping each class index to a filter index, each filter index of the previous alternative Luma filter set is mapped to a filter index in the current alternative Luma filter set using a lookup table with a number of entries equal to the number of filters in the previous alternative Luma filter set. Thus, with each alternative Luma filter set alf_luma_coeff_al_merge_idx[ altIdx ][ filtIdx ] provides a table indicating a merging of the filter indexes. Such kind of merging is illustrated with an example in Figure 3.

Figure 3 – Example illustrating the proposed ‘alf_luma_coeff_alt_merge_idx’ signaling as a replacement of ‘alf_luma_coeff_delta_idx’ signaling for alternative Luma filter sets.

The class index to filter index mapping ‘alf_luma_coeff_delta_idx[ altIdx ][ filtIdx ]’ is derived as follow:

- alf_luma_coeff_delta_idx[ 0 ][ filtIdx ] is first set equal to alf_luma_coeff_alt_merge_idx[ 0 ][ filtIdx ] for filtIdx = 0..24.

- Then for altIdx = 1..alf_luma_num_alts_minus1,

  - for filtIdx = 0..24,

    - alf_luma_coeff_delta_idx[ altIdx ][ filtIdx ] is set equal to alf_luma_coeff_alt_merge_idx[ altIdx ][ alf_luma_coeff_delta_idx[ altIdx – 1 ][ filtIdx ] ].

## Alternative Chroma filters

The number of alternative Chroma filters is indicated in the tile group header (it could be in APS when APS will be implemented in VTM), as well as the filter coefficients (and the associated clipping values) of each alternative Chroma filter. For this contribution, the maximum number of alternative Chroma filters is fixed to MAX_NUM_ALF_ALTERNATIVES_CHROMA = 8 per tile group.

## Alternative filters selection

In each CTU, if ALF is enabled on Luma component, a Luma filter set alternative index is signalled for each filter index of the first Luma filter set alternative (e.g. for the example of Figure 3, N0-1 alternative index are provided in each CTU). And in each CTU, for each Chroma component on which ALF is enabled, a Chroma filter alternative index is signalled.

The signaling of a filter (set) alternative index is performed, by entropy with CABAC the bits of its unary representation. In total, three CABAC contexts are used. The first context is shared with all the ALF Luma filter indexes and is used for the signaling of the Luma filter set alternative indexes. The second context is used for the signaling of the Cb Chroma alternative filter index, and the third context is used for the signaling of the Cr Chroma alternative filter index.

For Luma component, if ALF is enabled on a CTU and the classification index of a 4x4 Luma samples block is classIdx, the filter index in first Luma alternative filter set is filtIdx = alf_luma_coeff_delta_idx[ 0 ][ classIdx ]. altIdx is the alternative Luma filter index provided in the CTU for the filter index filtIdx.

The filter coefficients and clipping values to use for processing the 4x4 Luma samples block are the ones provided for the filter with index alf_luma_coeff_delta_idx[ altIdx ][ classIdx ] in the alternative Luma filter set with index altIdx.

For a Chroma component, if ALF is enabled on a CTU and the alternative filer index is altIdx, the filter coefficients and clipping values to use for processing the CTU are the ones provided for the alternative Chroma filter with index altIdx.

# Proposed tests

This contribution provides three tests to show achievable BDR gains with the proposed NL-ALF extension.

- Test-1 and Test-2 add the proposed syntax to signal alternative filter sets to an additional signalling on top of  the CE5-3.2 test, where linear ALF vs non-linear ALF (NL-ALF) can be used. Test-1 and Test-2 only differ by the encoder complexity in order to reach different BDR gains/complexity trade-offs (see below for details),

- Test-3 adds the proposed syntax to signal alternative filter sets to an additional signalling on top of a combination of CE5-3.2 and CE5-3.3 tests, where ALF vs NL-ALF can be used and NL-ALF uses a reduced number of clipping operations for NL-ALF.

In the three tests, each component type, the encoder starts with a number of tested - Luma filter set / Chroma filter – alternatives equal to a maximum number of tested - Luma filter set / Chroma filter – alternatives (see section 3.1).

- It starts by associating to each alternative a sub-set of the CTU for performing a first filter(s) optimization of each alternative.

- Then while iteratively optimizing the CTB alf enable flag(s), it also associates to each CTB the best alternative index(es). Letting the CTB alf enable flag(s), and filter(s) converge (The number of convergence loop iterations, compared to the ones used in VTM4.0 for optimizing CTB alf enable flag(s) only, is increased proportionally to the number of tested alternatives).

The encoder decreases the number of tested alternatives and repeat 1) and 2) until only one alternative has been optimized. It selects the best Rate/Distortion compromise.

## Restriction of alternative Luma filter sets

To limit the encoder complexity in this contribution, the maximum number of tested alternatives depends on the number of CTU in the tile group (i.e. in the frame).

In Test-1, Test-2 and Test-3, the maximum number of tested alternative Luma filter sets depends on the tile group type. On INTRA tile groups, only one filter set is tested (it seemed to proponents that on INTRA tile groups, more than one alternative filter sets does not provide a good complexity versus gains trade off).

On non-INTRA tile groups, for obtaining the maximum number of tested alternative Luma filter sets, the floored log2 of the number of CTU is computed:

log2_floor=log2(m_numCTUsInPic).

In Test-1 and Test-3, the maximum number of tested alternative Luma filter sets is:

min( MAX_NUM_ALF_ALTERNATIVES_LUMA, max( 1, log2_floor - 2 ) ).

In Test-2, the maximum number of tested alternative Luma filter sets is:

min( MAX_NUM_ALF_ALTERNATIVES_LUMA, max( 1, log2_floor - 3 ) ).

## Restriction of alternatives Chroma filters

In Test-1, Test-2 and Test-3, the maximum number of tested alternative Chroma filters is set equal to:

min( MAX_NUM_ALF_ALTERNATIVES_CHROMA, m_numCTUsInPic * 2 ).

## Summary of alternative filter sets

| Class (NumCTUs) | Luma |  | Chroma |
| --- | --- | --- | --- |
|  | Test-1 and Test-3 | Test-2 | All Tests |
| Class A (510) | Intra: 1 Non-intra: 6 | Intra: 1 Non-intra: 5 | 8 |
| Class B (135) | Intra: 1 Non-intra: 5 | Intra: 1 Non-intra: 4 | 8 |
| Class C (28) | Intra: 1 Non-intra: 2 | 1 | 8 |
| Class D (8) | 1 | 1 | 8 |
| Class E (60) | Intra: 1 Non-intra: 3 | Intra: 1 Non-intra: 2 | 8 |
| Class F (28) | Intra: 1 Non-intra: 2 | 1 | 8 |
| Class F (60) | Intra: 1 Non-intra: 3 | Intra: 1 Non-intra: 2 | 8 |
| Class F (135) | Intra: 1 Non-intra: 5 | Intra: 1 Non-intra: 4 | 8 |

Table 1 - Maximum number of tested alternatives

# Results

In the following sections, the results of the three tests, are provided, as well as runtime comparisons of the three tests and the three related CE5-3.x tests.

## Test-1

## Test-2

## Test-3

## Runtime comparisons

In this section average BDR gains versus Encoder & Decoder runtime increase of Test-1, Test-2, Test3, CE5-3.1, CE5-3.2, and CE5-3.3 tests are compared together in the CTC configurations, using VTM4.0 as a reference, and. using average BDR gain obtained with a 14/16 weight for Luma channel BDR gain and a 1/16 weight for each Chroma channel BDR gain.

Figure 4 – BDR gains versus Encoder and Decoder runtime increase of CE5-3 tests and JVET-N0243 tests compared to VTM4.0 in CTC All Intra configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 5 – BDR gains versus Encoder and Decoder runtime increase of CE5-3 tests and JVET-N0243 tests compared to VTM4.0 in CTC Random Access configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 6 – BDR gains versus Encoder and Decoder runtime increase of CE5-3 tests and JVET-N0243 tests compared to VTM4.0 in CTC Low Delay.B configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

Figure 7 – BDR gains versus Encoder and Decoder runtime increase of CE5-3 tests and JVET-N0243 tests compared to VTM4.0 in CTC Low Delay.P configuration
(14/16 was used for Luma channel BDR gain and 1/16 was used for each Chroma channel BDR gain)

# Conclusion

In this contribution, in order to improve the non-linear ALF perfomance, it is proposed to enable signaling of alternative filter sets for Luma and alternative filters for Chroma. The three tests in this contribution demonstrate the BDR gains using the syntax for enabling alternative Luma filter sets and alternative Chroma filters.

It is recommended to adopt this Non-Linear ALF extension in the next version of the VVC draft by considering the good complexity/performance compromise illustrated by the CE5-3 tests and the three tests presented herein.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).