| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0246 |
| --- | --- |

| Title: | Modified dequantization scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karl Sharman Steve Keating | Email: | karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

In VVC WD4, when the size of a transform block is not a power of 4, the transform coefficients are modified by a √2 factor, to compensate for an implicit scaling by the transform process.

It is proposed that the scaling be applied by a modification to the QP or QP levelScale table rather than by multiplication by 181/256 (or 181/128).

The proposal reportedly produces negligible BD-rate changes. It is a continuation of JVET-M0119, and is independent, but related to JVET-L0095.

# Introduction

The transformation processes require the resulting coefficients to be scaled by

factor= 1√MN

where M and N are the width and height of the transform block.

In VVC, the dimensions of the block are powers of 2, i.e. M=2m and N=2n. This means that when M=N, or indeed when MN=4f, the factor can be applied by a right-shift. However, when MN is not a power of 4, a simple right-shift is not applicable. Instead, in the current VVC Draft 4 (JVET-M1001-v6) and in VTM4.0, the coefficients are scaled by a 181/256 factor as an additional stage of the dequantization process (and similarly for the quantization process).

There are several alternative methods to remove this additional compensation stage. They all relate to the fact that the QP system is logarithmic, and that QP+6 is an additional division by 2, and QP+3 is an additional division by √2.

## Method 1: “HM_QTBT_AS_IN_JEM_QUANT==0”

VTM4.0 already includes some code, which can be enabled by setting “HM_QTBT_AS_IN_JEM_QUANT” to 0. The change causes QP’ to be adjusted by 3 for blocks that require the √2 compensation. However, the implemented code clips the adjusted QP’ to be in the range [0, MAX_QP+qpBDOffset].

The clipping prevents the qP from going less than 0; the effects of this are only noticeable at the extremely low QPs, as will be seen later.

Note that due to recent software merges, some minor bug fixes are required if this macro is disabled.

## Method 2: “HM_QTBT_AS_IN_JEM_QUANT==0” without clipping

The clipping introduced in method 1 can be removed from the calculation. The requirement for this, however, is that if the qP is less than 0, the bit shifts resulting from a negative (qP / 6) must be modified, as must the (qP % 6), which must produce a value in the range [0, 5].

The result would be indistinguishable for normal QP ranges, but results in the system functioning as expected at the lowest QPs.

## Method 3

Another method to consider is to increase the size of the “levelScale” look-up table. The QPper and QPrem are calculated without regard of the block shape. Then, the QPrem index is used to look-up the inverse quantization scale factor, except that the look-up stage accounts for the block shape. This can be achieved by using a second table for non-power-of-4 blocks:

| qP % 6 |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qscale | Normal blocks |  |  | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| invQScale |  |  |  | 40 | 45 | 51 | 57 | 64 | 72 |
| Qscale | Compensating blocks |  |  | 36158 | 32768 | 29127 | 26214 | 23302 | 20560 |
| invQScale |  |  |  | 29 | 32 | 36 | 40 | 45 | 51 |

An alternative, but equivalent implementation could use a single table, with 9 elements, due to the significant overlap between the two sets of coefficients.

This is effectively combining the normalisation multiplication (181/256) and the levelScale table into one table, without increasing the bit-depth of the per-coefficient calculation.

## Method 4

Alternatively, instead of compensating by using a smaller inverse quantisation scaler, a larger value could be used, followed by a larger shift right. The table therefore becomes:

| qP % 6 |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qscale | Normal blocks |  |  | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| invQScale |  |  |  | 40 | 45 | 51 | 57 | 64 | 72 |
| Qscale | Compensating blocks |  |  | 18396 | 16384 | 14564 | 12945 | 11523 | 10280 |
| invQScale |  |  |  | 57 | 64 | 72 | 81 | 91 | 102 |

This has the advantage that no new value (encoder or decoder) crosses a power of 2 boundary (in method 3, the new encoder values crossed a power-of-2 boundary).

## Method 5

Another method is derived upon Method 3, but with the new decoder values of the table being half (rounded down) of the existing values. This potentially reduces the size of table:

| qP % 6 |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qscale | Normal blocks |  |  | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| invQScale |  |  |  | 40 | 45 | 51 | 57 | 64 | 72 |
| Qscale | Compensating blocks |  |  | 37449 | 32768 | 29127 | 26214 | 23302 | 20560 |
| invQScale |  |  |  | 28 | 32 | 36 | 40 | 45 | 51 |

## Method 6

Finally, a similar adjustment, as was applied on method 3 to give method 5, can be applied to method 4. The table then becomes:

| qP % 6 |  |  |  | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qscale | Normal blocks |  |  | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| invQScale |  |  |  | 40 | 45 | 51 | 57 | 64 | 72 |
| Qscale | Compensating blocks |  |  | 18396 | 16384 | 14564 | 13107 | 11651 | 10280 |
| invQScale |  |  |  | 57 | 64 | 72 | 80 | 90 | 102 |

However, this is for information only, since the “normal block” invQScale values in method 4 or 6 could have instead have been derived from the “compensating blocks” values with a simple left shift.

# Proposal

It is proposed than any of the methods are selected, as they all reduce the size of the multiplicand during the coefficient decoding process.

The simplest method to select is the one already present in the software (method 1). However, it does have a potential issue at qP’ < 3. This issue can be resolved with method 2, without any significant effort, and the software integration for this is minimal.

Methods 3 and 5 would instead be likely to be implemented with a multiplexor on the selection on the invQScale value (levelScale). Methods 4 and 6 seem to introduce a complexity that is not necessary.

The text changes required for each of these methods can be seen in Section 4.

# Experimental results

The proposed methods have been implemented on top of VTM4.0. Some experiments are currently running, and this contribution/document may be updated once results have completed.

## CTCs

Experiments have been run using the CTCs as described in JVET-M1010. As expected, and shown below, the results have negligible impact on BD-rate, and are also similar to those indicated in JVET-L0095 (which was implemented on top of VTM2.0) and JVET-M0119 (which was implemented on VTM3.0).

### Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Method 1 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | 0.19% | 0.05% | 99% | 100% |
| Class B | -0.01% | 0.05% | 0.16% | 100% | 101% |
| Class C | 0.02% | -0.05% | 0.05% | 100% | 100% |
| Class E | 0.00% | 0.05% | 0.01% | 101% | 103% |
| Overall | 0.00% | 0.05% | 0.07% | 100% | 101% |
| Class D | 0.00% | -0.03% | -0.11% | 100% | 101% |
| Class F | -0.01% | 0.10% | 0.06% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Method 1 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | 0.12% | 100% | 99% |
| Class B | 0.00% | 0.04% | -0.16% | 100% | 100% |
| Class C | 0.00% | 0.03% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | -0.09% | 100% | 100% |
| Class D | 0.03% | -0.14% | -0.08% | 100% | 99% |
| Class F | -0.04% | -0.19% | -0.28% | 100% | 100% |

### Method 2

The results for method 2 with the common test conditions are identical to those of method 1; they are therefore not repeated here.

### Method 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Method 3 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.10% | 0.08% | 99% | 99% |
| Class A2 | -0.03% | 0.26% | 0.12% | 100% | 100% |
| Class B | -0.02% | 0.14% | 0.22% | 99% | 99% |
| Class C | 0.01% | 0.00% | 0.09% | 100% | 100% |
| Class E | 0.00% | 0.12% | 0.07% | 99% | 100% |
| Overall | -0.01% | 0.12% | 0.13% | 100% | 100% |
| Class D | -0.02% | 0.10% | -0.02% | 102% | 102% |
| Class F | -0.02% | 0.09% | 0.07% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Method 3 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.03% | 101% | 101% |
| Class A2 | -0.04% | 0.13% | 0.03% | 101% | 101% |
| Class B | 0.00% | 0.01% | 0.05% | 100% | 100% |
| Class C | 0.03% | 0.06% | -0.20% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | -0.03% | 100% | 101% |
| Class D | 0.00% | -0.22% | -0.34% | 100% | 100% |
| Class F | -0.01% | -0.23% | -0.30% | 100% | 101% |

### Method 4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Method 4 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | 0.01% | 100% | 101% |
| Class A2 | -0.02% | 0.15% | 0.05% | 99% | 100% |
| Class B | -0.01% | 0.00% | 0.09% | 100% | 100% |
| Class C | 0.02% | -0.14% | -0.04% | 100% | 100% |
| Class E | 0.00% | 0.01% | 0.02% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.03% | 100% | 100% |
| Class D | 0.03% | 0.08% | -0.21% | 101% | 101% |
| Class F | 0.00% | 0.05% | -0.08% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Method 4 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | 0.07% | 102% | 102% |
| Class B | 0.00% | -0.01% | -0.17% | 100% | 100% |
| Class C | 0.01% | -0.07% | -0.09% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.07% | 100% | 101% |
| Class D | 0.01% | -0.09% | 0.03% | 100% | 100% |
| Class F | -0.02% | -0.15% | -0.23% | 101% | 101% |

### Method 5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Method 5 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 99% | 100% |
| Class A2 | 0.00% | 0.08% | -0.04% | 100% | 101% |
| Class B | 0.00% | -0.01% | 0.08% | 100% | 100% |
| Class C | 0.02% | -0.09% | 0.04% | 102% | 102% |
| Class E | -0.01% | 0.02% | -0.03% | 99% | 99% |
| Overall | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.00% | -0.05% | -0.15% | 101% | 101% |
| Class F | 0.00% | 0.02% | -0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Method 5 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.01% | 99% | 99% |
| Class A2 | 0.00% | -0.14% | 0.02% | 101% | 101% |
| Class B | 0.00% | -0.22% | -0.20% | 100% | 100% |
| Class C | 0.05% | 0.00% | -0.22% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.12% | -0.12% | 100% | 100% |
| Class D | 0.04% | -0.17% | -0.35% | 99% | 100% |
| Class F | 0.00% | -0.17% | -0.27% | 101% | 101% |

### Method 6

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Method 6 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | 0.18% | 0.04% | 101% | 101% |
| Class B | -0.01% | 0.04% | 0.17% | 100% | 100% |
| Class C | 0.02% | -0.05% | 0.05% | 101% | 101% |
| Class E | 0.00% | 0.05% | 0.01% | 99% | 99% |
| Overall | 0.00% | 0.05% | 0.07% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.11% | 100% | 100% |
| Class F | -0.01% | 0.10% | 0.06% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Method 6 Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | -0.04% | 0.11% | 101% | 101% |
| Class B | 0.00% | 0.03% | -0.17% | 100% | 100% |
| Class C | 0.00% | 0.03% | -0.21% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.09% | 100% | 100% |
| Class D | 0.03% | -0.14% | -0.08% | 99% | 100% |
| Class F | -0.04% | -0.19% | -0.28% | 101% | 101% |

## Low QP study

At qP’ < 3, the clipping of method 1 becomes apparent. To show this the all-intra CTCs have been repeated, but with QPs of -12, -11, -10 and -9. In the graph below, the dipping of Method 1 at the lowest QPs is apparent; all other methods show negligible difference compared with the anchor.

# Draft text

## Method 1

### 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Max(0, Qp′Y − (rectNonTsFlag ? 3 : 0) )		(8-1018)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Max(0, Qp′Cb − (rectNonTsFlag ? 3 : 0) )		(8-1019)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Max(0, Qp′Cr − (rectNonTsFlag ? 3 : 0) )		(8-1020)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  8  :  0 ) + 		(8-1022)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

rectNorm = rectNonTsFlag  ?  181  :  1	(8-1023)

bdOffset = ( 1 << bdShift ) >> 1		(8-1024)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(8-1025)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ qP % 6 ] ) << ( qP / 6 )	(8-1026)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-1027)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-1028)

## Method 2

### 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y − (rectNonTsFlag ? 3 : 0)		(8-1018)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb − (rectNonTsFlag ? 3 : 0)		(8-1019)

- Otherwise (cIdx is equal to 2), the following applies:

qP =Qp′Cr − (rectNonTsFlag ? 3 : 0)		(8-1020)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  8  :  0 ) + 		(8-1022)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag
	+ ( qP < 0 ? ( 5 −qP ) /6 : 0)

rectNorm = rectNonTsFlag  ?  181  :  1	(8-1023)

bdOffset = ( 1 << bdShift ) >> 1		(8-1024)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ (qP + 17) % 6 ] ) << ( qp < 0 ? 0 : ( (qP + 1) / 6 ) )	(8-1025)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ (qP + 6) % 6 ] ) << ( qp < 0 ? 0 :  (qP / 6) )	(8-1026)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-1027)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-1028)

## Methods 3 & 5

Note in method 5, the table levelScale redefined below would need to modified as set out above.

### 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-1018)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-1019)

- Otherwise (cIdx is equal to 2), the following applies:

qP =Qp′Cr		(8-1020)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  8  :  0 ) + 		(8-1022)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

rectNorm = rectNonTsFlag  ?  181  :  1	(8-1023)

bdOffset = ( 1 << bdShift ) >> 1		(8-1024)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 29, 32, 36, 40, 45, 51 } } with j = 0 or 1, and k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1 % 6 ] ) << ( (qP + 1) / 6 )	(8-1025)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << (qP / 6)	(8-1026)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-1027)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-1028)

## Methods 4 & 6

Note in method 6, the table levelScale redefined below would need to modified as set out above.

### 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-1018)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-1019)

- Otherwise (cIdx is equal to 2), the following applies:

qP =Qp′Cr		(8-1020)

The variable rectNonTsFlag is derived as follows:

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 && 	(8-1021)
		transform_skip_flag[ xTbY ][ yTbY ] = = 0 )

The variables  bdShift, rectNorm and bdOffset are derived as follows:

bdShift = bitDepth + ( ( rectNonTsFlag  ?  81  :  0 ) +		(8-1022)
	( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + dep_quant_enabled_flag

rectNorm = rectNonTsFlag  ?  181  :  1	(8-1023)

bdOffset = ( 1 << bdShift ) >> 1		(8-1024)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 81, 91, 102 } } with j = 0 or 1, and k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is set equal to 16.

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If dep_quant_enabled_flag is equal to 1, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1 % 6 ] ) << ( (qP + 1) / 6 )	(8-1025)

–	Otherwise (dep_quant_enabled_flag is equal to 0), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << (qP / 6)	(8-1026)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = 			(8-1027)
	( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * ls[ x ][ y ] * rectNorm +bdOffset )  >>  bdShift

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(8-1028)

# Conclusion

It is proposed that the method to compensate for blocks that are not a power-of-4 in size (in terms of coefficients) is modified, from the additional scaling factor method in VVC WD4 / VTM4.0. Six different methods are discussed, with the simplest (method 1) being that already present (but disabled) in VTM4.0. If operation at low QPs is a significant concern, then a small adjustment (method 2) is suggested. All methods show a negligible BD-rate change with the CTCs defined in JVET-M1010; at low QPs, (qP’<3), the difference between method 1 and methods 2/3/4/5/6 can be seen.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).