| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0248 |
| --- | --- |

| Title: | Non-CE3: Combined-Hypothesis Intra-Prediction with Unified Intra Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gosala Kulupana Andre Seixas Dias Saverio Blasi 201 Wood Lane, The Lighthouse W12 7TQ, London, UK | Tel: Email: | gosala.kulupana@bbc.co.uk andre.seixasdias@bbc.co.uk saverio.blasi@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution presents an Intra prediction technique based on JVET-M0458, where two intra-prediction hypotheses are computed for a block, one obtained by means of PLANAR prediction and the other obtained using one of the angular intra-predictions. The two hypotheses are averaged on a sample-by-sample basis to obtain a combined-hypothesis intra-prediction. This contribution further proposes to combine this technique with the method proposed in JVET-M0528, where non-angular modes (PLANAR or DC) are encoded using specific syntax elements independent from the computation of an MPM list. By combining the two techniques, the combined hypothesis intra-prediction flag is only signalled in case the block is predicted using an angular mode, avoiding the parsing dependency identified in JVET-M0458. Average BD-rates of -0.22% (Y), -0.15% (U) and -0.13% (V) are reported for AI with 105% average encoding time and 101% decoding time with respect to VTM 4.0.

# Introduction

In VTM 4.0 [1], a given sample can be predicted using one of the intra-prediction modes which consist of PLANAR, DC and 65 angular modes. In this contribution, Combined-Hypothesis Intra-Prediction as presented in [2] is considered in VTM 4.0. A set of 65 additional combined hypothesis intra-prediction modes are considered. Each additional mode computes a prediction by averaging an intra-prediction hypothesis obtained by predicting the block using a given angular mode, with another intra-prediction hypothesis obtained by predicting the same block using PLANAR intra-prediction. The combined-hypothesis, PComb, for a given sample (i,j) is computed by combining the corresponding angular prediction (PAng) with the PLANAR prediction (PPlanar) of the same sample as:

Pcombi,j=(PAngi,j+PPlanari,j+ 1)≫1

Combined-hypothesis intra prediction modes are signalled as a modification of existing angular modes. In case this modification is signalled, then the corresponding angular mode is combined with PLANAR prediction as above. Signalling is performed by means of an additional binary flag combined_hypothesis_intra_flag which is sent for each angular intra-predicted luma block to indicate whether to use the combined hypothesis intra prediction mode or not. In order to avoid parsing dependencies, the luma intra mode coding presented in [3] is considered to be used for signalling of luma intra modes. When using this modification, a flag intra_luma_angular_mode_flag is introduced to determine whether a block is predicted using non-angular modes (PLANAR or DC) or angular modes. When using the combined-hypothesis intra-prediction approach, the value of such flag is checked for an intra-predicted block. If intra_luma_angular_mode_flag==1, then combined_hypothesis_intra_flag is parsed from the bitstream, otherwise, combined_hypothesis_intra_flag is inferred to be 0 and no combined-hypothesis intra prediction is used. The combined-hypothesis modes are applied only on luma samples. Additional restrictions are applied to limit usage of the new modes only on specific block sizes. In particular, the additional modes are only allowed if both the height and width of the block are larger than 8, and at least one of height and width is larger than or equal to 32. Moreover, the method is not used in combination with the multi-line reference samples [4] mode or the intra-subpartition mode [5].

The proposed changes to the relevant part of the draft specification text are highlighted below.

| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| --- | --- |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_angular_mode_flag[ x0 ][ y0 ] = = 0) |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 && intra_luma_angular_mode_flag[ x0 ][ y0 ]) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| if(intra_luma_angular_mode_flag[ x0 ][ y0 ] && intra_luma_ref_idx[ x0 ][ y0] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] == 0 && (cbWidth >= 8 && cbHeight >= 8) && (cbWidth > 32 ¦¦ cbHeight > 32) ) |  |
| combined_hypothesis_intra [ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |

# Performance results

The performance of the proposed contribution is summarised in Table 1. The tests were performed using the JVET common test conditions [6] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 4.0, compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1: Performance of the proposed combined-hypothesis intra.

|  |  |  | AI |  |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.43% | -0.35% | 104% | 99% |  |  |  |  |  |
| Class A2 | -0.19% | -0.06% | -0.10% | 106% | 102% |  |  |  |  |  |
| Class B | -0.20% | -0.15% | -0.09% | 105% | 101% |  |  |  |  |  |
| Class C | -0.03% | -0.11% | -0.08% | 105% | 100% | -0.01% | 0.08% | -0.21% | 99% | 99% |
| Class E | -0.13% | -0.01% | -0.08% | 106% | 101% |  |  |  |  |  |
| Overall | -0.22% | -0.15% | -0.13% | 105% | 101% |  |  |  |  |  |
| Class D | -0.02% | -0.08% | -0.20% | 104% | 102% | 0.02% | -0.37% | -0.10% | 100% | 95% |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusions

In this contribution, combined-hypothesis intra-prediction as in JVET-M0458 was proposed in combination with the unified luma intra mode coding as in in JVET-M0528. When using combined-hypothesis intra-prediction, two intra-prediction hypotheses obtained by predicting the same block using PLANAR prediction and one of the angular prediction modes are combined. The method is signalled as an additional syntax element in case a block is predicted using one of the available angular modes in VTM 4.0. The performance of the proposed method when implemented in VTM 4.0 demonstrates an average BD-rates of -0.22% (Y) with particularly significant gains in Class A1 (-0.64% (Y)) for an average 105% encoder complexity and  101% decoder complexity.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001-v5, January 2019

- G. Kulupana, A. Seixas Dias, S. Blasi, “Non-CE3: Combined-Hypothesis Intra-Prediction”, in JVET-M0458, January 2019

- F. Bossen, K. Misra, “Non-CE3: A unified luma intra mode list construction process”, in JVET-M0528, January 2019

- B. Bross, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, L. Zhao, X. Zhao, X. Li, S. Liu,Y-J. Chang, H-Y. Jiang, P-H. Lin, C-C. Kuo, C-C. Lin, C-L. Lin “CE3: Multiple reference line intra”, in JVET-L0283, October 2018

- S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Intra Sub-Partitions Coding Mode”, in JVET-M0102, January 2019

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” in JVET-M1010, January 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).