| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0250 |
| --- | --- |

| Title: | Non-CE8: Reference memory reduction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents solutions to reduce on-chip memory for intra block copy. Solution A provides 20% memory reduction with BD-rate of -0.13% for 4:2:0TGM. Solution B provides 6.7% memory reduction with BD-rate of -2.57% for 4:2:0TGM.

# Introduction

Intra block copy, i.e. IBC (or current picture referencing, i.e. CPR previously) coding mode, was adopted into VTM at the 12th JVET meeting. It was known that IBC needs additional on-chip memory to store nearby reconstructed samples before filtering for reference. Currently that additional memory size is 3 VPDUs, i.e. 3*(64x64*1.5)*10/(1024*8)=22.5Kbytes. Given the cost of on-chip memory, it is desired to reduce the memory requirement while keeping the coding efficiency.

# Proposed schemes

Currently the internal bit-depth for decoding is 10-bit and so is the IBC reference. However, usually for screen content, the input bit-depth is less than 10-bit. We propose to reduce the bit-depth for IBC reference so that the additional memory brought by IBC can be reduced. Two specific schemes are described below.

## Solution A

Figure 1. Memory reuse in the current design.

In the current IBC design, memory reuse is applied and 3 additional 64x64 blocks are added to store IBC reference samples of previous 64x64 blocks.

In this solution, we propose to use 8-bit for those 64x64 blocks’ memory. When decoding of the current 64x64 block is finished, the reconstructed at 10-bit will be converted and copied to those 3 8-bit 64x64 buffer. Compared with the current design, the saving is 20% for additional IBC memory.

## Solution B

In this solution, instead of allowing IBC reference from the current 64x64 memory being decoded, we propose to use a dedicated 128x128 7-bit buffer to store IBC reference samples. One benefit is that because decoding buffer is not mixed with IBC buffer so that IBC buffer can store 4 64x64 reconstructed blocks, which corresponds to CE8.1.2d, i.e., CU level memory reuse, presented at the 13th JVET meeting. In this solution, after decoding a CU, its reconstructed will be written to the corresponding 7-bit IBC buffer.

The saving for additional IBC memory is 100%-128x128*7/(3*64*64*10)=6.7%.

# Experimental results

We compare the proposed solutions with VTM-4.0 with IBC on for all sequences. Table I shows the performance for solution A and Table II shows the performance for solution B.

Table I. Performance of Solution A. IBC is on for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.08% | 0.01% | 125% | 113% |
| Class E | -0.10% | -0.05% | -0.01% | 128% | 116% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.04% | 0.05% | 122% | 117% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| 4:2:0 TGM | -0.13% | -0.16% | -0.12% | 99% | 120% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.17% | -0.25% | 108% | 120% |
| Class C | 0.00% | 0.27% | -0.06% | 111% | 115% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.07% | 0.22% | 113% | 124% |
| Class F | -0.07% | -0.10% | -0.12% | 104% | 127% |
| 4:2:0 TGM | -0.62% | -0.46% | -0.54% | 105% | 126% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.09% | 0.95% | 0.59% | 118% | 116% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| 4:2:0 TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table II. Performance of Solution B. IBC is on for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.01% | 0.03% | 129% | 111% |
| Class E | -0.13% | -0.16% | -0.04% | 131% | 115% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | 0.04% | 0.12% | 125% | 118% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| 4:2:0 TGM | -2.57% | -2.73% | -2.67% | 102% | 122% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.10% | -0.22% | 110% | 121% |
| Class C | -0.01% | 0.17% | 0.12% | 112% | 116% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.05% | 0.16% | 0.20% | 115% | 126% |
| Class F | -0.71% | -0.77% | -0.77% | 105% | 126% |
| 4:2:0 TGM | -1.81% | -1.65% | -1.64% | 106% | 125% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | 1.07% | 0.18% | 120% | 119% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| 4:2:0 TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).