| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N252 |
| --- | --- |

| Title: | CE4-1.4: Syntax changes of merge data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution provides the simulation results of CE4-1.4. It is proposed to adjust the order of merge data syntaxes based on the type of prediction partition. The test 4-1.4 can achieve 0.01% and 0.03% BD-rate gain in random access and low-delay B configuration, respectively.

# Introduction

In the current draft of VVC, there are three kinds of prediction partitions such as a block, subblocks, and two triangles. It is one of the key features that are different from conventional video coding standards.

From this point of view, the merge lists can be categorized as CU merge list, subblock merge list, and triangle merge list [1]. More specifically, there are MMVD list and general CU merge list for CU-level. In the current draft, the order of syntax parsing for merge data is mmvd, subblock merge, multi hypothesis prediction, triangle prediction, and general CU-level merge.

# Proposed methods

In CE4-1.4, three tests are conducted as shown in below list:

- Test 4-1.4.a: Signal a merge_subblock_flag and a merge_triangle_flag consecutively

- Test 4-1.4.b: combination of test 4.1.4.a with 4.1.1

- Test 4-1.4.c: combination of test 4.1.4.a with 4.1.3

In test 4-1.4.a, it is proposed to adjust the order based on the type of prediction partition [2]. By arranging the merge_subblock_flag and the merge_triangle_flag, which are syntaxes related to the type of prediction partition, sequentially, the proposed method can provide flexibility for further syntax changes. The reason that the merge_subblock_flag is positioned at the front is that the coding efficiency of affine prediction and sbTMVP is the highest. In addition, the merge_triangle_flag is placed after that to determine block-level merge and triangle-level merge. In order to maintain consistency according to the type of prediction partitions, the test 4-1.4.a changes the order of the syntaxes as Table1.

In addition, this CE also tests the performance of the combination of two other CE tests related to syntax changes of merge mode. In test 4-1.4.b, it is reported the simulation results of the combination of test 4-1.4.a with 4-1.1 [3]. In test 4-.1.4.c, it is reported the simulation results of the combination of test 4-1.4.a with 4-1.3 [4].

Table 1. Proposed changes in Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_triangle_enabled_flag && tile_group_type = = B && cbWidth * cbHeight >= 64 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| ciip_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| ciip_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Performance evaluation

The proposed methods were evaluated under JVET common test conditions (CTC) [5]. The proposed methods were integrated on VTM-4.0. Detailed results for each test are reported in Table 2, Table 3, and Table4, respectively. The encoder and decoder times might be unreliable.

Table 2. Test 4-1.4.a

|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.03% | -0.02% | -0.05% | 102% | 102% |  |  |  |  |  |
| Class B | -0.01% | -0.10% | 0.00% | 103% | 102% | -0.03% | -0.06% | 0.13% | 96% | 95% |
| Class C | -0.02% | -0.18% | -0.08% | 101% | 101% | -0.03% | 0.12% | -0.05% | 95% | 85% |
| Class E |  |  |  |  |  | -0.04% | 0.00% | -0.53% | 93% | 86% |
| Overall | -0.01% | -0.09% | -0.03% | 102% | 102% | -0.03% | 0.02% | -0.10% | 95% | 89% |
| Class D | -0.04% | -0.05% | -0.19% | 100% | 101% | -0.09% | -0.03% | -0.40% | 100% | 99% |

Table 3. Test 4-1.4.b

|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.02% | 0.17% | 102% | 103% |  |  |  |  |  |
| Class A2 | -0.03% | 0.09% | 0.18% | 103% | 103% |  |  |  |  |  |
| Class B | 0.00% | 0.08% | 0.03% | 103% | 101% | -0.04% | -0.08% | 0.05% | 102% | 97% |
| Class C | -0.01% | 0.04% | -0.07% | 102% | 102% | -0.07% | 0.05% | 0.10% | 93% | 81% |
| Class E |  |  |  |  |  | -0.05% | 0.11% | -0.41% | 92% | 81% |
| Overall | -0.01% | 0.06% | 0.06% | 102% | 102% | -0.05% | 0.01% | -0.05% | 96% | 87% |
| Class D | 0.04% | 0.06% | 0.11% | 101% | 100% | 0.06% | 0.11% | -0.23% | 99% | 94% |

Table 3. Test 4-1.4.c

|  | Over VTM-4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.09% | 102% | 104% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.05% | 103% | 104% |  |  |  |  |  |
| Class B | -0.02% | -0.15% | -0.06% | 102% | 102% | -0.01% | -0.20% | -0.44% | 93% | 85% |
| Class C | -0.03% | -0.10% | -0.25% | 103% | 103% | -0.04% | 0.16% | -0.24% | 92% | 80% |
| Class E |  |  |  |  |  | 0.00% | -0.17% | -0.50% | 91% | 80% |
| Overall | -0.01% | -0.10% | -0.06% | 103% | 103% | -0.02% | -0.07% | -0.39% | 92% | 82% |
| Class D | -0.03% | -0.06% | -0.30% | 102% | 102% | -0.06% | 0.17% | -0.36% | 99% | 93% |

# References

- B. Bross, J. Chen, and S. Liu, “Versatile video coding (Draft 4),” JVET-M1001, 13th JVET meeting, Marrakech, MA, Jan. 2019.

- Y. Ahn and D. Sim, “CE4-related: Syntax changes of merge data,” JVET-M0369, 13th JVET meeting, Marrakech, MA, Jan. 2019.

- E. Sasaki, T. Chujoh, and T. Ikai, “Non-CE4: Syntax change of MMVD,” JVET-M0069, 13th JVET meeting, Marrakech, MA, Jan. 2019.

- G. Ko, D. Kim, J. Jung, J. Son, and J. Kwak, “Non-CE4: Modification of merge data syntax,” JVET-M0359, 13th JVET meeting, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th JVET meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).