| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0266 |
| --- | --- |

| Title: | CE2-related: Disabling bi-prediction or inter-prediction for small blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC [1], bi-predicted 4*16, 16*4, 4*8 and 8*4 coding units (CU) and uni-predicted 4*4 CUs require much higher memory bandwidth than 8*8 bi-predicted CUs. This contribution provides the results of disabling inter-prediction for 4*4 CUs and bi-prediction for 4*N and N*4 CUs. It is reported that:

- Disable inter prediction for 4*4 CUs: there is 0.03% and 0.03% coding loss in RA and LDB Main10 configurations respectively.

- Disable inter prediction for 4*4 CUs and disable bi-prediction for 4*8 and 8*4 CUs: there is 0.xx% and 0.xx% coding loss in RA and LDB Main10 configurations respectively.

- Disable inter prediction for 4*4 CUs and disable bi-prediction for 4*8, 8*4, 4*16 and 16*4 CUs: there is 0.25% and 0.30% coding loss in RA and LDB Main10 configurations respectively.

# Introduction

In current VVC, bi-predicted 4*16, 16*4, 4*8 and 8*4 coding unit (CU) and uni-predicted 4*4 CU require higher memory bandwidth than 8*8 bi-predicted CU. Tool provided in JVET-M0862 [2] is used to analyze the memory bandwidth required by these CUs, and the results are summarized in Table 1 for YCbCr 4:2:0 case. Both luma and chroma components are considered in the analysis.

Compared with HEVC wherein 8*8 bi-predicted CU requires the highest memory bandwidth, the worst-case memory bandwidth of VVC is increased by around 46% and 100% for 2*1 memory pattern and 16*2 memory pattern respectively.

Table 1: Memory bandwidth comparison of different block sizes.

| CUs | 2*1 memory pattern |  | 16*2 memory pattern |  |
| --- | --- | --- | --- | --- |
|  | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs | Memory bandwidth | Bandwidth / bandwidth of 8*8 bi-predicted CUs |
| 8*8 Bi | 20.19 | 100% | 36.00 | 100% |
| 4*8 Bi | 29.38 | 146% | 72.00 | 200% |
| 8*4 Bi | 29.38 | 146% | 54.00 | 150% |
| 4*16 Bi | 22.69 | 112% | 54.00 | 150% |
| 16*4 Bi | 22.69 | 112% | 36.00 | 100% |
| 4*4 Uni | 21.38 | 106% | 54.00 | 150% |

# Proposed method

To suppress the problem mentioned above, this contribution test disabling bi-prediction or inter-prediction for small CUs. Specifically, we run the following tests:

- Test #1: disabling inter-prediction for 4*4 CU.

- Test #2: Test #1 + disabling bi-prediction for 4*8 and 8*4 CUs.

- Test #3: Test #1 + disabling bi-prediction for 4*8, 8*4, 4*16 and 16*4 CUs, including CUs coded in triangle mode.

In Test #2, the worst-case memory bandwidth of VVC is reduced to 112% and 150% of that of the 8*8 bi-predicted CU, for 2*1 memory pattern and 16*2 memory pattern respectively. In Test #3, the worst-case memory bandwidth of VVC is reduced to that of the 8*8 bi-predicted CU.

# Experimental results

The proposed method was implemented on top of reference software VTM-4.0 [3], and simulations were performed following the common test conditions [4]. The results are shown in Table 2~Table 4.

Table 2: Coding performance of Test #1.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | 0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.02% | 0.02% | 0.19% | 99% | 100% |  |  |  |  |  |
| Class B | 0.03% | 0.11% | 0.16% | 98% | 100% | 0.03% | 0.00% | 0.06% | 100% | 99% |
| Class C | 0.06% | 0.49% | 0.47% | 97% | 99% | 0.08% | 0.48% | 0.25% | 97% | 94% |
| Class E |  |  |  |  |  | -0.02% | 0.27% | -0.51% | 100% | 98% |
| Overall | 0.03% | 0.19% | 0.22% | 98% | 100% | 0.03% | 0.23% | -0.02% | 99% | 97% |
| Class D | 0.16% | 0.69% | 0.56% | 96% | 92% | 0.24% | 0.77% | 0.39% | 99% | 100% |
| Class F | 0.13% | 0.31% | 0.29% | 98% | 98% | 0.28% | 1.06% | 0.99% | 98% | 98% |

Table 3: Coding performance of Test #2.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 4: Coding performance of Test #3.

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.25% | 98% | 100% |  |  |  |  |  |
| Class A2 | 0.22% | 0.61% | 0.80% | 98% | 99% |  |  |  |  |  |
| Class B | 0.24% | 0.61% | 0.61% | 97% | 100% | 0.24% | 0.51% | 1.20% | 98% | 98% |
| Class C | 0.49% | 1.12% | 1.23% | 95% | 98% | 0.55% | 1.22% | 1.35% | 94% | 93% |
| Class E |  |  |  |  |  | 0.07% | -0.06% | -0.48% | 98% | 97% |
| Overall | 0.25% | 0.63% | 0.74% | 97% | 99% | 0.30% | 0.60% | 0.83% | 97% | 96% |
| Class D | 0.80% | 1.60% | 1.60% | 94% | 91% | 0.86% | 1.73% | 1.04% | 94% | 97% |
| Class F | 0.32% | 0.59% | 0.56% | 97% | 97% | 0.45% | 0.93% | 0.71% | 96% | 99% |

# Conclusions

This contribution tests disabling bi-prediction and inter-prediction for small CUs. In Test #2 and Test #3, the worst-case memory bandwidth of VVC is reduced dramatically and the coding performance loss is relatively small. It is recommended to adopt it into VVC.

# References

- “Versatile Video Coding (Draft 4)”, B. Bross, J. Chen, S. Liu, Doc. JVET-M1001, Jan. 2019.

- “BoG report on CE2 related contributions”, Y. He, Doc. JVET-M0862, Jan. 2019.

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).