| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0254-v2 |
| --- | --- |

| Title: | CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Zhenzhong Chen, Yiming Li Wuhan University Liang Zhao, Shan Liu, Xiang Li Tencent | Tel: Email: | zzchen@whu.edu.cn shanl@tencent.com |
| Source: | Tencent, Wuhan University |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Core Experiment 13 (CE13) test 2.2 and 2.3 on neural network based filter for video coding. In test 2.2, the impact of the position of DRNLF in the filter chain is investigated. In test 2.3, the generalization capability of DRNLF when QP is different from the training QP is investigated.

Experimental results of test 2.2a (2.3a) show -1.52%, -2.12% and -2.73% BD-rates on Y, Cb, and Cr components, respectively with an All Intra configuration, -1.45%, -4.37% and -4.27% BD-rates with a Random Access configuration, and -1.54%, -6.04% and -5.86% BD-rates in a Low delay configuration.

Regarding to the first intra period test, -1.62%, -4.51% and -4.42% BD-rates are reported on Y, Cb, and Cr components, respectively with a Random Access configuration for test 2.2a (2.3a), 1.51%, 1.21% and 1.80% for test 2.2b, -0.65%, -2.18% and -1.60% for test 2.2c, -1.88%, -4.81%, -4.33% for test 2.3b, and -1.37%, -4.57% and -4.04% for test 2.3c.

Introduction

Fig. 1 shows the network structure of dense residual convolutional neural network based in-loop filter (DRNLF) [1], where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN.

Figure 1. Network structure of DRNLF.

The main body of the proposed network consists of series of DRU. The structure of DRU is shown in Fig. 2. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 2. Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

Test description

In test 2.2, the impact of the position of the DRNLF in the filter chain is investigated, where 3 kinds of filter chain are tested: (a) DBF + DRNLF + SAO + ALF; (b) DRNLF only; and (c) DRNLF +ALF.

Test 2.3 investigates the generalization capability of DRNLF when encoding QP is different from the training QP. It includes 3 kinds of QP settings: (a) CTC QP for training, CTC QP for testing; (b) CTC QP for training, CTC QP + 2 for testing; (c) CTC QP for training, CTC QP - 2 for testing.

The tests are summarized in Table 1. Note that test 2.2 uses CTC QP for training, CTC QP for testing and the filter chain of test 2.3 is DBF + DRN + SAO + ALF.

Table 1 Tests summary

| CE13-2.2a | Loop filter chain | DBF + NN filter + SAO + ALF, NN filter follows the description in M0508 |
| --- | --- | --- |
| CE13-2.2b |  | NN filter only, NN filter follows the description in M0508 |
| CE13-2.2c |  | NN filter + ALF, NN filter follows the description in M0508 |
| CE13-2.3a | Network generalization capability (QP) | CTC QP for training, CTC QP for testing (the same as CE13-2.2.a) |
| CE13-2.3b |  | CTC QP for training, CTC QP + 2 for testing |
| CE13-2.3c |  | CTC QP for training, CTC QP - 2 for testing |

Experimental results

The proposed method has been implemented on top of VTM 4.0 [2]. The simulation is conducted according to CE13 description [3] and the common test conditions (CTC) [4].

In the training phase, DIV2K dataset [5] is utilized to generate the training sets and validating set for the proposed network. There are 800 images in its training set and 100 images in its validation set. The images in DIV2K dataset are converted from RGB color space to YUV color space before compression. The images are compressed by VTM 4.0 with All Intra (AI) configuration using QP values in {22, 27, 32, 37}. L1 loss is used as the loss function for training. To balance the coding gain between different components, we set the weights of Y, Cb, Cr to 10, 1, 1, respectively. The loss is minimized by ADAM optimizer. 300 epochs of training are applied. For each epoch, 8 small patches with size of 64×64 are randomly extracted from the images to generate the training samples. The learning rate is initialized as 52×10-4 and is divided by half every 50 epochs.

In evaluation, VTM 4.0 serves as the reference software. All the sequences recommended in CTC are evaluated. The simulations are performed on Linux cluster without using GPUs. The experimental results of test 2.2a (test 2.3a) using AI, RA and LDB configuration compared to VTM-4.0 are shown in Table 2, Table 3 and Table 4, respectively. Table 5-7 shows the result of test 2.2. Table 8-10 shows the results of test 2.3.

Test 2.2

Table 2 Results for test 2.2a (2.3a) with an AI configuration (Full test)

|  | All Intra Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.64% | -2.91% | 114% | 5904% |
| Class A2 | -1.46% | -2.72% | -1.77% | 107% | 3465% |
| Class B | -0.93% | -2.19% | -3.08% | 106% | 3665% |
| Class C | -1.90% | -2.34% | -3.33% | 104% | 4520% |
| Class E | -2.57% | -1.57% | -2.13% | 108% | 7759% |
| Overall | -1.52% | -2.12% | -2.73% | 107% | 4667% |
| Class D | -2.22% | -0.92% | -3.37% | 103% | 3955% |
| Class F | -1.09% | -1.83% | -1.54% | 105% | 3173% |

Table 3 Results for test 2.2a (2.3a) with a RA configuration (Full test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.27% | -3.38% | -5.10% | 106% | 6967% |
| Class A2 | -2.21% | -5.74% | -2.88% | 106% | 6435% |
| Class B | -1.13% | -4.73% | -4.55% | 106% | 7011% |
| Class C | -1.39% | -3.63% | -4.36% | 106% | 8110% |
| Class E |  |  |  |  |  |
| Overall | -1.45% | -4.37% | -4.27% | 106% | 7156% |
| Class D | -1.39% | -1.96% | -3.08% | 105% | 4217% |
| Class F | -0.50% | -1.52% | -1.36% | 115% | 3018% |

Table 4 Results for test 2.2a (2.3a) with a LDB configuration (Full test)

|  | Low delay B Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.12% | -7.24% | -7.12% | 105% | 8387% |
| Class C | -1.40% | -6.97% | -6.45% | 104% | 9894% |
| Class E | -2.45% | -2.80% | -2.95% | 116% | 9434% |
| Overall | -1.54% | -6.04% | -5.86% | 108% | 9127% |
| Class D | -1.73% | -2.75% | -4.92% | 105% | 6881% |
| Class F | -0.74% | -3.42% | -1.89% | 117% | 5799% |

Table 5 Results for test 2.2a with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43% | -2.61% | -4.30% | 108% | 8378% |
| Class A2 | -2.28% | -5.90% | -2.92% | 106% | 6332% |
| Class B | -1.47% | -4.74% | -5.32% | 106% | 7279% |
| Class C | -1.47% | -4.62% | -4.51% | 103% | 8378% |
| Class E |  |  |  |  |  |
| Overall | -1.62% | -4.51% | -4.42% | 106% | 7559% |
| Class D | -1.47% | -2.11% | -3.21% | 105% | 4769% |
| Class F | -0.39% | -1.36% | -1.06% | 113% | 3415% |

Table 6 Results for test 2.2b with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.25% | 5.00% | 3.35% | 107% | 12176% |
| Class A2 | 1.65% | -1.64% | 1.81% | 105% | 9127% |
| Class B | 1.54% | 3.13% | 1.69% | 105% | 9583% |
| Class C | 0.80% | -1.89% | 0.79% | 103% | 9419% |
| Class E |  |  |  |  |  |
| Overall | 1.51% | 1.21% | 1.80% | 105% | 9910% |
| Class D | 1.66% | -1.01% | -1.13% | 105% | 4716% |
| Class F | 1.80% | 0.67% | 0.78% | 113% | 5026% |

Table 7 Results for test 2.2c with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.92% | 1.50% | 106% | 11955% |
| Class A2 | -1.38% | -3.55% | -1.29% | 104% | 8961% |
| Class B | -0.62% | -2.05% | -2.74% | 106% | 9736% |
| Class C | -0.74% | -3.65% | -2.73% | 104% | 9589% |
| Class E |  |  |  |  |  |
| Overall | -0.65% | -2.18% | -1.60% | 105% | 9937% |
| Class D | -1.06% | -2.31% | -2.30% | 105% | 4776% |
| Class F | 0.82% | 0.00% | -0.11% | 113% | 5303% |

Test 2.3

Table 8 Results for test 2.3a with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43% | -2.61% | -4.30% | 108% | 8378% |
| Class A2 | -2.28% | -5.90% | -2.92% | 106% | 6332% |
| Class B | -1.47% | -4.74% | -5.32% | 106% | 7279% |
| Class C | -1.47% | -4.62% | -4.51% | 103% | 8378% |
| Class E |  |  |  |  |  |
| Overall | -1.62% | -4.51% | -4.42% | 106% | 7559% |
| Class D | -1.47% | -2.11% | -3.21% | 105% | 4769% |
| Class F | -0.39% | -1.36% | -1.06% | 113% | 3415% |

Table 9 Results for test 2.3b with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.43% | -2.58% | -4.12% | 109% | 8192% |
| Class A2 | -2.36% | -5.78% | -2.87% | 108% | 6655% |
| Class B | -1.73% | -4.91% | -5.12% | 107% | 7541% |
| Class C | -2.03% | -5.62% | -4.59% | 105% | 9750% |
| Class E |  |  |  |  |  |
| Overall | -1.88% | -4.81% | -4.33% | 107% | 8008% |
| Class D | -1.85% | -1.58% | -4.04% | 106% | 5964% |
| Class F | -0.85% | -2.59% | -2.37% | 113% | 3847% |

Table 10 Results for test 2.3c with a RA configuration (Short test)

|  | Random Access Main10 Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.44% | -3.19% | -4.42% | 106% | 7894% |
| Class A2 | -1.91% | -6.42% | -3.16% | 105% | 5492% |
| Class B | -1.20% | -4.39% | -4.68% | 106% | 6556% |
| Class C | -1.12% | -4.42% | -3.63% | 104% | 7448% |
| Class E |  |  |  |  |  |
| Overall | -1.37% | -4.57% | -4.04% | 105% | 6794% |
| Class D | -0.95% | -1.34% | -2.28% | 104% | 3354% |
| Class F | -0.16% | -1.08% | -0.88% | 111% | 3095% |

References

[1] Yingbin Wang, Zhenzhong Chen, Yiming Li, Liang Zhao, Shan Liu, Xiang Li, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter,” JVET-M0508, Marrakech, MA, 9–18 Jan. 2019.

[2] VTM 4.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0.

[3] Yiming Li, Shan Liu, Kei Kawamura, “Description of Core Experiment 13 (CE13): Neural Network based Filter for Video Coding,” JVET-M1033, Marrakech, MA, 9–18 Jan. 2019.

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

[5] DIV2K, https://data.vision.ee.ethz.ch/cvl/DIV2K/.

Patent rights declaration(s)

Wuhan University and Tencent America LLC do not have any current or pending patent rights relating to the technology described in this contribution.