| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0257 |
| --- | --- |

| Title: | CE2: Alignment of affine control-point motion vector and subblock motion vector (Test 2-5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Vadim Seregin, Hongtao Wang and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, vseregin, hongtaow martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE2: Alignment of affine control-point motion vector and subblock motion vector (test 2-5.2).  There’re six tests, the overall BD-rate change in Y, U, V are shown in the following:

- Directly derive subblock MV in affine inheritance merge candidate. 0.00%， -0.06%， -0.02% in RA and 0.00%, 0.18%, -0.39% in LDB.

- a + affine inheritance using subblock MVs. 0.01%, -0.08%, -0.05% in RA and 0.01%, -0.05%, -0.10% in LDB.

- b + shifting affine CPMVs by (2,2). 0.03%, -0.04%, 0.05% in RA and 0.04%, 0.22%, -0.17% in LDB.

- c + changing neighbouring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP. 0.00%, 0.01%, 0.01%, 0.02% in RA and 0.01%, -0.04%, -0.11% in LDB.

- d + 16x16 grid affine inheritance. 0.02%, -0.01%, 0.05% in RA and 0.01%, 0.04%, -0.04% in LDB.

- 16x16 grid affine inheritance. 0.02%, -0.05%, -0.02% in RA and -0.02%, 0.05%, -0.14% in LDB.

One supplemental result is also provided, in which only the affine inheritance using subblock MVs is applied, i.e. test b – a. The overall BD-rate change in Y, U, V are *.**%, *.**%, *.**% in RA and *.**%, *.**%, *.**%  in LDB.

# Introduction

In JVET-M0110 [1], it’s proposed to use the subblock motion vectors from the neighboring affine coded block for affine motion inheritance. It’s also proposed to use the subblock motion vectors as the affine control-point motion vectors (CPMVs). In JVET-M0053 [2], it’s proposed to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than 16. In CE2, test 2-5.2 is established to study the related techniques.

# Proposed methods

## Directly derive subblock MV in affine inheritance merge candidate

In the current VTM-4.0, the affine inheritance merge candidate derives the CPMVs of the current block from the reference neighboring block, and then further derives the subblock MVs of the current block from the derived CPMVs. Therefore, two rounding processes are performed in each step. It’s proposed to directly derive the subblock MVs for affine inheritance merge candidate to reduce one unnecessary rounding process during the CPMVs derivation.

## Affine inheritance using subblock motion vectors

In the proposed method, the bottom-right, bottom-middle and right-middle subblock MVs of neighboring coded block (as shown in Fig. 1) are used to derive the predicted CPMVs of current block, such that the distance between v0 and v1 and v0 and v2 are nW/2 and nH/2, wherein nW and nH are the width and height of the neighboring block. If the neighboring block locates at the above CTU, then only v0 and v2 are used for affine motion inheritance and the 4-parameter affine motion model is assumed. The proposed method provides 3 benefits:

- The CPMV buffer is removed to reduce the storage requirement of motion information.

- The “Derivation process for luma affine control point motion vectors from a neighbouring block” in the specification is reduced by 5 lines.

- Affine inheritance from the above CTU uses the same locations as inheritance from current CTU, which provides more alignment.

Fig. 1 Proposed subblock motion vectors that used for affine motion inheritance. sbW and sbH are the width and height of the subblock, respectively. nW and nH are the width and height of the neighboring block, respectively.

The modified derivation process for luma affine control point motion vectors from a neighboring block is described in the appendix.

## Shifting of control point motion vectors

The CPMVs are shifted to the center of the subblock to better align with the affine inheritance using subblock motion vectors. The setting of CPMVs are shown in Fig. 2. The top-left CPMV v0 is the same as the MV of the top-left subblock. The top-right and bottom-left CPMVs locate at two virtual subblocks at the extended subblock column and subblock row of the current block, such that the distance between v0 and v1 and v0 and v2 remain blkW and blkH (power of 2), respectively. The motion vector for the subblock at ith row and jth column is derived as in equation (1)

| vx=(v1x-v0x)blkW*j*sbW+v2x-v0xblkH*i*sbH+v0xvy=(v1y-v0y)blkW*j*sbW+v2y-v0yblkH*i*sbH+v0y | (1) |
| --- | --- |

The checking order of motion vector predictors for CPMVs in the constructed affine motion prediction candidate is also changed accordingly.   For prediction of the top-right CPMV, the checking order of neighboring blocks is changed from B1B0 to B0B1. For prediction of the bottom-left CPMV, the checking order is changed form A1A0 to A0A1.

Fig. 2 Proposed CPMV settings. The CPMVs are shifted to the center of the subblocks.

## 16x16 grid affine inheritance

In JVET-M0053, it’s proposed to disable inherited affine motion prediction from a neighboring affine coded block if the width or height of the neighboring block is less than 16. The affine motion information can be stored in 16x16 grid, therefore, reduces the storage requirement.

# Storage requirement analysis

The storage requirement analysis in the following is based on CTU 128x128 and 64x64 block based decoder pipeline in the memory-optimized implementation [3], wherein the subblock MVs for the whole 64x64 block and the left/top-left/top neighboring MV contexts are stored in local buffer. Table 1 summarizes the local buffer usage, wherein the VTM-4.0 without affine mode is used as anchor to calculate the increasement in percentages for affine mode.

The total number of segments for the subblock MV buffer is 256 (64 / 4 * 64 / 4). The left and top neighboring MV context requires 64 (128 / 4 * 2) segments, but the top-left neighboring MV context requires only 1 segment. Therefore, the local buffer for subblock MV buffer requires 18,432 (256 * 36 * 2) bits and the left/top/top-left neighboring MV contexts requires 4,680 (65 * 36 * 2) bits. In addition, other inter prediction information, inter prediction direction (2 bits if reference index doesn’t include -1), reference indices (8 bits), and bi-predicted weighted average (BPWA) index (3 bits) need to be stored for each segment. The local buffer for other inter prediction information requires 4,173 ((256+65) * (2 + 8+ 3)) bits.

In the current VTM4.0, the local buffer for left/top/top-left neighboring CPMV contexts stores 288 MVs (128/8*3*3*2), therefore it requires 10,368 bits. Note that the CPMV is not stored for the whole 64x64 block. The top-left neighboring CPMV context requires 16 (128 / 8) segments, as in left / top neighboring CPMV context. In addition, the affine type, top-left coordinate, width and height need to be stored for affine motion inheritance, which requires 704 bits. The affine type (1 bit) and top-left coordinate (10 bits) need to be stored for each segment of all the left/top/top-left neighboring CPMV contexts, it requires 528 (128/8*3*11) bits. For the top-left neighboring CPMV context, both width and height (5 bits to store width and height combinations) need to be stored for each segment, it requires 80 (128/8*5) bits. However, only the width needs to be stored in each segment for the left neighboring CPMV context, 48 (128/8*3) bits, and only the height (3 bits) need to be stored in each segment for the top neighboring CPMV context, 48 (128/8*3) bits.

In the proposed method, the local subblock MV buffer remain the same. The local buffer for left/top-left/top neighboring CPMV contexts is removed. Instead, the local buffer needs to store 2 more MVs in each segment of the left and top neighboring subblock MV contexts, and 4 (2*2) more MVs in the top-left neighboring subblock MV context. In the 8x8 affine inheritance, it requires 2,448 ((128/8*2*2+4) * 36) bits for the added subblock MV buffer. When 16x16 grid affine inheritance is applied, the number is 1,296 (128/16*2*2+4) * 36) bits for the added subblock MV buffer, and 352 (704/2) bits for the top-left coordinate, width and height.

Table 1 Summary of local buffer usage.

CTU size 128x128, 64x64 decoder pipeline, memory-optimized implementation.

| Method | Local sub-block MV buffer (bits) | Local buffers for left/top-left/top neighboring MV contexts (bits) | Local buffers for left/top-left/top neighboring CPMV contexts (bits) | Inter direction/reference index/BPWA index (bits) | Affine type/Width/height/top-left x and y (bits) | Total local memory size (bits) | Percentage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM4.0 without affine | 18,432 | 4,680 | 0 | 4,173 | 0 | 27,285 | 100% |
| VTM4.0 | 18,432 | 4,680 | 10,368 | 4,173 | 704 | 38,357 | 141% |
| Proposed with 8x8 grid | 18,432 | 4,680 | 2,448 | 4,173 | 704 | 30,437 | 112% |
| Proposed with 16x16 grid | 18,432 | 4,680 | 1,296 | 4,173 | 352 | 28,933 | 106% |

# Test results

The 6 sub-tests conducted in CE2 test 2-5.2 are:

- Directly derive subblock MV in affine inheritance merge candidate

- a + Affine inheritance using subblock MVs

- b + shifting affine CPMVs by (2,2)

- c + changing neighboring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP

- d + 16x16 grid affine inheritance

- 16x16 grid affine inheritance

The summary result is shown in Table 2. Detailed test results are shown in Table 3 to Table 8. Another supplemental result is also provided for b-a, wherein only affine inheritance using subblock MVs is applied. The detail result is shown in Table 9. The encoding time reduction is due to skipping unnecessary chroma motion compensation in encoder search.

Table 2 Summary results

| Tests | RA BD-rate (Y, U, V) | LDB BD-rate (Y, U, V) |
| --- | --- | --- |
| (a) Directly derive subblock MV in affine inheritance merge candidate | 0.00% -0.06%, -0.02% | 0.00% 0.18% -0.39% |
| (b) a + Affine inheritance using subblock MVs | 0.01%, -0.08%, -0.05% | 0.01% -0.05% -0.10% |
| (c) b + shifting affine CPMVs by (2,2) | 0.03%, -0.04%, 0.05% | 0.04% 0.22% -0.17% |
| (d) c + changing neighboring block checking order for top-right and bottom-left CPMVs prediction in the constructed affine MVP | 0.01%, 0.01%, 0.02% | 0.01% -0.04% -0.11% |
| (e) d + 16x16 grid affine inheritance | 0.02%, -0.01%, 0.05% | 0.01% 0.04% -0.04% |
| (f) 16x16 grid affine inheritance | 0.02%, -0.05%, -0.02% | -0.02% 0.05% -0.14% |
| (g) b - a |  |  |

Table 3 Test (a) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.11% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class C | 0.01% | -0.06% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.02% | 100% | 100% |
| Class D | -0.01% | 0.05% | 0.02% | 100% | 99% |
| Class F | 0.00% | 0.07% | 0.01% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | -0.25% | 98% | 103% |
| Class C | -0.04% | 0.28% | -0.03% | 96% | 102% |
| Class E | 0.07% | 0.51% | -1.09% | 99% | 104% |
| Overall | 0.00% | 0.18% | -0.39% | 97% | 103% |
| Class D | 0.02% | 0.02% | -0.60% | 98% | 103% |
| Class F | -0.05% | 0.20% | 0.21% | 100% | 103% |

Table 4 Test (b) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.23% | -0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.03% | 100% | 100% |
| Class B | 0.00% | -0.04% | -0.09% | 100% | 100% |
| Class C | 0.01% | -0.07% | -0.07% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.05% | 100% | 100% |
| Class D | 0.03% | -0.24% | 0.07% | 100% | 100% |
| Class F | -0.03% | 0.04% | -0.05% | 100% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.30% | -0.18% | 98% | 103% |
| Class C | 0.04% | 0.10% | 0.05% | 97% | 100% |
| Class E | 0.03% | 0.16% | -0.14% | 99% | 103% |
| Overall | 0.01% | -0.05% | -0.10% | 98% | 102% |
| Class D | 0.01% | 0.22% | -0.46% | 98% | 108% |
| Class F | -0.15% | 0.68% | 0.92% | 99% | 102% |

Table 5 Test (c) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.02% | 95% | 97% |
| Class A2 | 0.03% | -0.04% | 0.08% | 92% | 98% |
| Class B | 0.03% | -0.07% | 0.14% | 97% | 100% |
| Class C | 0.05% | 0.00% | -0.05% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.04% | 0.05% | 95% | 99% |
| Class D | 0.07% | 0.11% | 0.16% | 95% | 99% |
| Class F | -0.01% | 0.00% | 0.00% | 96% | 98% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.10% | -0.16% | 95% | 99% |
| Class C | 0.07% | 0.28% | 0.10% | 93% | 91% |
| Class E | 0.05% | 0.33% | -0.57% | 95% | 96% |
| Overall | 0.04% | 0.22% | -0.17% | 94% | 95% |
| Class D | 0.11% | -0.56% | -0.63% | 96% | 97% |
| Class F | -0.09% | -0.04% | -0.73% | 97% | 96% |

Table 6 Test (d) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.10% | 0.07% | 94% | 97% |
| Class A2 | 0.00% | -0.06% | 0.08% | 88% | 92% |
| Class B | 0.02% | 0.02% | 0.02% | 97% | 100% |
| Class C | 0.02% | -0.02% | -0.05% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.02% | 94% | 97% |
| Class D | 0.07% | 0.06% | -0.08% | 95% | 99% |
| Class F | 0.02% | 0.00% | -0.01% | 96% | 97% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.09% | -0.11% | 95% | 98% |
| Class C | 0.02% | -0.01% | -0.11% | 93% | 92% |
| Class E | -0.03% | -0.01% | -0.10% | 95% | 94% |
| Overall | 0.01% | -0.04% | -0.11% | 94% | 95% |
| Class D | 0.05% | 0.01% | -0.97% | 96% | 98% |
| Class F | -0.09% | -0.13% | -0.52% | 97% | 96% |

Table 7 Test (e) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | 0.09% | 95% | 97% |
| Class A2 | 0.04% | -0.08% | 0.02% | 88% | 93% |
| Class B | 0.04% | -0.02% | 0.15% | 96% | 99% |
| Class C | 0.03% | 0.05% | -0.10% | 95% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.05% | 94% | 97% |
| Class D | 0.06% | 0.00% | 0.23% | 95% | 98% |
| Class F | 0.01% | 0.03% | -0.02% | 96% | 97% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.26% | -0.24% | 95% | 99% |
| Class C | 0.04% | 0.22% | 0.14% | 93% | 91% |
| Class E | 0.01% | 0.32% | 0.07% | 95% | 98% |
| Overall | 0.01% | 0.04% | -0.04% | 95% | 96% |
| Class D | 0.09% | 0.24% | -0.67% | 96% | 96% |
| Class F | -0.10% | -0.25% | -0.42% | 96% | 96% |

Table 8 Test (f) results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | 0.04% | 97% | 101% |
| Class A2 | 0.03% | -0.01% | 0.08% | 97% | 103% |
| Class B | 0.01% | -0.10% | -0.02% | 99% | 103% |
| Class C | 0.03% | 0.00% | -0.12% | 97% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.02% | 98% | 102% |
| Class D | 0.05% | 0.10% | 0.11% | 98% | 104% |
| Class F | 0.02% | 0.05% | -0.05% | 98% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.01% | 97% | 104% |
| Class C | -0.03% | 0.18% | 0.07% | 97% | 103% |
| Class E | -0.03% | -0.16% | -0.68% | 98% | 103% |
| Overall | -0.02% | 0.05% | -0.14% | 97% | 104% |
| Class D | 0.04% | 0.09% | -0.79% | 98% | 103% |
| Class F | -0.05% | -0.01% | 0.32% | 99% | 104% |

Table 9 Test (b-a) results

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | 0.00% | -0.06% | 0.02% | 91% | 96% |
| Class B | 0.01% | -0.14% | -0.04% | 97% | 100% |
| Class C | 0.04% | -0.02% | -0.07% | 94% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.06% | -0.01% | 0.00% | 95% | 99% |
| Class F | -0.01% | 0.04% | -0.01% | 95% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.17% | 0.25% | 92% | 88% |
| Class E | -0.09% | -0.31% | -0.42% | 96% | 95% |
| Overall |  |  |  |  |  |
| Class D | 0.14% | 0.34% | -0.33% | 95% | 96% |
| Class F | -0.12% | -0.01% | 0.55% | 96% | 96% |

# Conclusion

This contribution reports the CE test results for CE2 test 2-5.2.  It’s suggested to adopt test b or test g in the next version of VTM.

# References

- H. Huang, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz, “CE2-related: Alignment of affine control-point motion vector and subblock motion vector”, JVET-M0110, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

- H. Huang, W.-J. Chien, M. Karczewicz, “CE2: Size constrain for inherited affine motion prediction (test 2.4.7)”, JVET-M0053, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

# M. Zhou, B. Heng, “CE4-related: A clean up for affine mode”, JVET-L0047, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

#### Derivation process for luma affine control point motion vectors from a neighbouring block

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block,

- a luma location ( xNb, yNb ) specifying the top-left sample of the neighbouring luma coding block relative to the top-left luma sample of the current picture,

- two variables nNbW and nNbH specifying the width and the height of the neighbouring luma coding block,

- the number of control point motion vectors numCpMv.

Output of this process are the luma affine control point vectors cpMvLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variable isCTUboundary is derived as follows:

- If all the following conditions are true, isCTUboundary is set equal to TRUE:

- ( ( yNb + nNbH ) % CtbSizeY ) is equal to 0

- yNb + nNbH is equal to yCb

- Otherwise, isCTUboundary is set equal to FALSE.

The variables log2NbHalfW and log2NbHalfH are derived as follows:

log2NbHalfW = Log2( nNbW/2 )

log2NbHalfH = Log2( nNbH /2)

The variables mvScaleHor, mvScaleVer, are derived as follows:

- mvScaleHor = MvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 0 ] << 7

- mvScaleVer = MvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 1 ] << 7

The variables dHorX and dVerX are derived as follows:

- dHorX = ( MvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 0 ] − MvLX[ xNb + nNbW/2 − 1  ][ yNb + nNbH − 1 ][ 0 ] ) 	
	<< ( 7 − log2NbHalfW )

- dVerX = ( MvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 1 ] − MvLX[ xNb + nNbW/2 – 1 ][ yNb + nNbH − 1 ][ 1 ] ) 	
	<< ( 7 − log2NbHalfW )

The variables dHorY and dVerY are derived as follows:

- If isCTUboundary is equal to FALSE and MotionModelIdc[ xNb ][ yNb ] is equal to 2, the following applies:

dHorY = ( CpMvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 2 ][ 0 ] − CpMvLX[ xNb + nNbW − 1 ][ yNb + nNbH/2 − 1 ][ 2 ][ 0 ] )	
	<< ( 7 − log2NbHalfH )

dVerY = ( CpMvLX[ xNb + nNbW − 1 ][ yNb + nNbH − 1 ][ 2 ][ 1 ] − CpMvLX[ xNb + nNbW − 1 ][ yNb + nNbH/2 – 1 ][ 2 ][ 1 ] )	
	<< ( 7 − log2NbHalfH )

- Otherwise (isCTUboundary is equal to TRUE or MotionModelIdc[ xNb ][ yNb ] is equal to 1), the following applies,

dHorY = − dVerX

dVerY = dHorX

……