| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0258-v1 |
| --- | --- |

| Title: | CE8-related: Palette Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Li Zhang, Jizheng Xu, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +13605250535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this proposal, the palette mode is coded as a prediction mode for a coding unit. Therefore, the prediction modes for a coding unit can be MODE_INTRA, MODE_INTER, MODE_IBC and MODE_PLT. The binarization of prediction modes is changed accordingly. The simulation is conducted follow the CE 8 test condition described in M1028. The proposed method is evaluated on top of CE8.2.1. Results reportedly show that, when IBC is enabled, on class F, the proposed method achieves -0.09%, -0.08% and -0.01% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On 4:2:0 TGM, the proposed method achieves -0.40%, -0.14% and X% BD rate changes on luma component under RA, AI and LDB configurations respectively. When IBC is turned off, on class F, the proposed method achieves -0.01%, -0.02% and 0.02% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On 4:2:0 TGM, the proposed method achieves -0.31%, -0.31% and -0.19% BD rate changes on luma component under RA, AI and LDB configurations respectively.

# Introduction

CE8.2.1 propose a palette mode coding which uses a base color representation to code the screen contents. In CE8.2.1, the palette mode is coded as one additional flag at the CU-level. If the current mode is not a inter mode, the flag to indicate the usage of palette mode will be coded, which still has the redundancy due to the introduction of IBC mode. Furthermore, the palette mode can be included to one of the prediction modes to unify the syntax, as the IBC mode in the current VVC draft. Therefore, in this proposal, we include the palette mode to prediction modes for a coding unit.

# Propose method

In this proposal, the prediction modes for a coding unit can be MODE_INTRA, MODE_INTER, MODE_IBC and MODE_PLT. The binarization of prediction modes is changed accordingly.

When IBC is turned off, on I tiles, the first one bin is employed to indicate whether the current prediction mode is MODE_PLT or not. While on P/B tiles, the first bin is employed to indicate whether the current prediction mode is MODE_INTRA or not. If not, one additional bin is employed to indicate the current prediction mode is MODE_PLT or MODE_INTER.

When IBC is turned on, on I tiles, the first bin is employed to indicate whether the current prediction mode is MODE_IBC or not. If not, the second bin is employed to indicate whether the current prediction mode is MODE_PLT or MODE_INTRA. While on P/B tiles, the first bin is employed to indicate whether the current prediction mode is MODE_INTRA or not. If it’s an intra mode, the second bin is employed to indicate the current prediction mode is MODE_PLT or MODE_INTRA. If not, the second bin is employed to indicate the current prediction mode is MODE_IBC or MODE_INTER.

# Propose syntax changes (On top of JVET-M1001-v5[2])

The syntax change is described as follows, and the newly added parts are highlighted in yellow.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ¦¦ sps_plt_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_modes(x0, y0, cbWidth, cbHeight) |  |
| … |  |
| } |  |

| pred_modes ( x0, y0, cbWidth, cbHeight) { | Descriptor |
| --- | --- |
| if(tile_group_type == I ) { |  |
| if(sps_ibc_enabled_flag) |  |
| pred_mode_ibc_flag | ae(v) |
| if(CuPredMode[ x0 ][ y0 ] != MODE_IBC){ |  |
| if(sps_plt_enabled_flag && cbWidth <=64 && cbHeight <= 64) |  |
| plt_mode_flag | ae(v) |
| } |  |
| else{ |  |
| pred_mode_flag | ae(v) |
| if(CuPredMode[ x0 ][ y0 ] == MODE_INTRA ¦¦ (CuPredMode[ x0 ][ y0 ] != MODE_INTRA && ! sps_ibc_enabled_flag)){ |  |
| if(sps_plt_enabled_flag && cbWidth <=64 && cbHeight <= 64) |  |
| plt_mode_flag | ae(v) |
| } |  |
| else{ |  |
| if(sps_ibc_enabled_flag) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| } |  |
| } |  |

plt_mode_flag equal to 1 specifies that the current coding unit is coded in palette mode. intra_mode_plt_flag equal to 0 specifies that the current coding unit is coded in the palette mode.

When plt_mode_flag is not present, it is inferred to be equal to false.

When pred_mode_scc_flag is equal to 1, the variable CuPredMode[ x ][ y ] is set to be equal to MODE_PLT for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight – 1

Table 9-4 Syntax elements and associated binarization.

| Syntax structure | Syntax element |  | Binarization |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | process |  | Input parameter |  |  |  |  |
| pred_modes( ) | PLT_mode_flag |  | FL |  | cMax = 1 |  |  |  |  |
| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |  |  |  |
| Syntax element |  | binIdx |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 | 3 | 4 | >= 5 |  |
| PLT_mode_flag |  | 0 |  | na |  | na | na | na | na |  |

# Simulation Results

The simulation followed the CE8 test condition [1]. The anchor is CE8.2.1. Enabling and disabling IBC are evaluated respectively. When IBC is turned on, simulation results reportedly show that when IBC is enabled, on class F, the proposed method achieves -0.09%, -0.08% and -0.01% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On class SCC, the proposed method achieves -0.40%, -0.14% and X% BD rate changes on luma component under RA, AI and LDB configurations respectively. When IBC is turned off, on class F, the proposed method achieves -0.01%, -0.02% and 0.02% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On class SCC, the proposed method achieves -0.31%, -0.31% and -0.19% BD rate changes on luma component under RA, AI and LDB configurations respectively.

Table 1. AI Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.03% | 99% | 99% |
| Class B | -0.01% | -0.08% | -0.01% | 100% | 99% |
| Class C | 0.00% | 0.09% | 0.16% | 100% | 96% |
| Class E | 0.01% | 0.02% | -0.01% | 99% | 98% |
| Overall | 0.00% | 0.01% | 0.04% | 100% | 98% |
| Class D | -0.01% | -0.13% | -0.03% | 99% | 97% |
| Class F | -0.09% | -0.03% | -0.15% | 100% | 101% |
| TGM | -0.40% | -0.44% | -0.43% | 98% | 100% |

Table 2. RA Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.15% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.08% | 0.02% | 99% | 99% |
| Class B | 0.00% | 0.08% | 0.22% | 99% | 100% |
| Class C | 0.04% | -0.01% | -0.19% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.07% | 0.02% | 99% | 99% |
| Class D | 0.02% | -0.06% | 0.15% | 99% | 95% |
| Class F | -0.08% | -0.28% | -0.32% | 100% | 100% |
| TGM | -0.14% | -0.35% | -0.37% | 100% | 99% |

Table 3. LDB Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.01% | -0.26% | -0.12% | 98% | 94% |
| Class E | 0.06% | -0.58% | 0.00% | 100% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.08% | 0.19% | 101% | 98% |
| Class F | -0.01% | 0.67% | 0.91% | 101% | 100% |
| TGM | -0.21% | -0.30% | -0.41% | 102% | 105% |

Table 4. AI Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.03% | 0.00% | 98% | 99% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.00% | -0.07% | -0.01% | 100% | 99% |
| Class E | -0.02% | -0.03% | 0.10% | 101% | 103% |
| Overall | -0.01% | -0.02% | 0.01% | 100% | 100% |
| Class D | -0.01% | 0.31% | 0.07% | 98% | 100% |
| Class F | -0.01% | -0.10% | -0.02% | 100% | 100% |
| TGM | -0.31% | -0.13% | -0.17% | 98% | 99% |

Table 5. RA Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.17% | -0.07% | 99% | 99% |
| Class A2 | -0.01% | 0.19% | 0.09% | 99% | 100% |
| Class B | 0.01% | 0.17% | 0.13% | 99% | 99% |
| Class C | 0.02% | -0.07% | -0.08% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.00% | -0.05% | 0.39% | 98% | 102% |
| Class F | -0.02% | -0.21% | -0.39% | 100% | 99% |
| TGM | -0.31% | -0.52% | -0.60% | 101% | 100% |

Table 6. LDB Performance when IBC is turned on

|  | Over CE8.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.00% | 0.07% | -0.18% | 98% | 91% |
| Class E | 0.12% | -0.05% | 1.23% | 99% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.11% | -0.07% | 0.35% | 98% | 99% |
| Class F | 0.02% | -0.48% | -0.46% | 99% | 98% |
| TGM | -0.19% | -0.42% | -0.24% | 100% | 99% |

# Conclusion

In this proposal, we include the palette mode to prediction modes for a coding unit to unify the syntax. Compared with CE8.2.1, when IBC is enabled, on class F, the proposed method achieves -0.09%, -0.08% and -0.01% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On class SCC, the proposed method achieves -0.40%, -0.14% and X% BD rate changes on luma component under RA, AI and LDB configurations respectively. When IBC is turned off, on class F, the proposed method achieves -0.01%, -0.02% and 0.02% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On class SCC, the proposed method achieves -0.31%, -0.31% and -0.19% BD rate changes on luma component under RA, AI and LDB configurations respectively. We recommend to adopt the proposed method into the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, Description of Core Experiment 8 (CE8): Screen Content Coding Tools, JVET-M1028

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).