| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0260-v1 |
| --- | --- |

| Title: | Non-CE8: Adaptive Fractional MVD search in DMVR for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu, Hongbin Liu, Kai Zhang, Li Zhang, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +13605250535 weijia.zhu@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In DMVR mode, fractional motion vector difference (MVD) search is conditionally performed after the integer MVD refinement.  However, fractional MVD refinement may be inefficient for screen contents. To address this issue, this contribution proposes to disable fractional MVD refinement adaptively in DMVR mode.

The simulation followed the CE8 test condition. When IBC is turned on, simulation results reportedly show that the proposed method has 0.00%, 0.01% and 0.00% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.03% and -0.01% BD rate changes. When IBC is turned off, simulation results reportedly show that the proposed method has 0.00%, 0.00% and 0.01% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.01% and -0.03% BD rate changes respectively.

# Introduction

DMVR is adopted into VVC [2] to refine the motion information more accurate. The DMVR in VVC first performs integer MVD refinement as described above. This is the first step. In the first step, the DMVR will use two iterations to refine the integer MVD. After that, an MVD refinement on fractional precision is conditionally performed to further refine the motion vector. This is the second step. In the second step, a parametric error surface is fitted using integer distance positions and the costs evaluated in these positions, which is then used to determine 1/16th-pel precision sub-pixel offsets. The condition whether to perform the second step is based on whether the MVD after one iteration is a zero MV. If it’s a zero MV (vertical and horizontal components of a MV is 0.), the second step will be performed.

However, fractional MVD refinement maybe inefficient for screen contents. To address this issue, this contribution proposes to disable fractional MVD refinement adaptively in DMVR mode.

# Proposed method

To address the problems mentioned above, this contribution proposes to decide whether to enable fractional MVD refinement in DMVR or not at encoder and signal an indication flag to the decoder in the tile group header.

## Encoder decision of tile group-level flag

In intra block copy mode, hash value is generated for each 4x4 block at encoder, and we use these hash values to decide whether a slice is a screen content slice or not. If a slice is a screen content slice, fractional MVD refinement is disabled in DMVR; otherwise, fractional MVD refinement is enabled for DMVR. Specifically, for each 4x4 block, we check whether there is another 4x4 block has the same hash value with it. If yes, such block is considered as a screen content block. If the percentage of screen content blocks is larger than a threshold (10%) for a slice, such slice is considered as a screen content slice.

## Proposed syntax changes (Based on JVET-M1001-v5[3])

The syntax change is described as follows, and the newly added parts are highlighted in yellow.

7.3.2.	Raw byte sequence payloads, trailing bits and byte alignment syntax

7.3.2.1.	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_bdof_enabled_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| if(sps_dmvr_enabled_flag) |  |
| sps_ subpixel_refinement_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| sps_num_ladf_intervals_minus2 | u(2) |
| … |  |
| } |  |

sps_ subpixel_refinement_enabled_flag equal to 1 specifies that the adaptive sub pixel MVD refinement can be used in a tile. sps_fpel_mmvd_enabled_flag specifies that the adaptive sub pixel MVD refinement cannots be used in a tile.

7.3.4.	Tile group header syntax

7.3.4.1.	General tile group header syntax

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ) |  |
| pred_weight_table( ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| tile_group_fpel_mmvd_enabled_flag | u(1) |
| if(sps_ subpixel_refinement_enabled_flag) |  |
| tile_group_subpixel_refinement_enabled_flag | u(1) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| tile_group_aps_id | u(5) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( NumTilesInCurrTileGroup > 1 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumTilesInCurrTileGroup − 1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

tile_group_subpixel_refinement_enabled_flag equal to 1 specifies the sub pixel MVD refinement can be enabled in current tile group. tile_group_fpel_mmvd_enabled_flag equal to 0 specifies that sub pixel MVD refinement cannot be enabled in current tile group. When not present, the value of tile_group_fpel_mmvd_enabled_flag is inferred to be 1.

8.5.3.	Decoder side motion vector refinement process

8.5.3.1.	General

- Inputs to this process are:

–	a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top left luma sample of the current picture,

–	a variable sbWidth specifying the width of the current coding subblock in luma samples,

–	a variable sbHeight specifying the height of the current coding subblock in luma samples,

–	the luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

–	the selected luma reference picture sample arrays refPicL0L and refPicL1L.

- Outputs of this process are:

–	delta luma motion vectors dMvL0 and dMvL1.

- The variable subPelFlag is set to 0. and the variables srRange, offsetH0, offsetH1, offsetV0, and offsetV1 are all set equal to 2.

- Both components of the delta luma motion vectors dMvL0 and dMvL1 are set equal to zero and modified as follows:

- For each X being 0 or 1, the ( sbWidth + 2 * srRange ) x ( sbHeight + 2 * srRange ) array predSamplesLXL of prediction luma sample values is derived by invoking the fractional sample bilinear interpolation process specified in 8.5.3.2.1 with the luma location ( xSb, ySb ), the prediction block width set equal to ( sbWidth + 2 * srRange ), the prediction block height set equal to ( sbHeight + 2 * srRange ), the reference picture sample array refPicLXL, the motion vector mvLX and the refinement search range srRange as inputs.

- The list sadList[ i ] with i = 0..8 is derived by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- When sadList[ 4 ] is greater than or equal to sbHeight * sbWidth, the following applies:

- The variable bestIdx is derived by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1.

- Otherwise, the following applies:

dX = bestIdx % 3 − 1

dY = bestIdx / 3 − 1

dMvL0[ 0 ] += 16 * dX

dMvL0[ 1 ] += 16 * dY

offsetH0 += dX

offsetV0 += dY

offsetH1 −= dX

offsetV1 −= dY

- The list sadList[ i ] with i = 0..8 is modifed by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- The variable bestIdx is modified by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1

- Otherwise (bestIdx is not equal to 4), the following applies:

dMvL0[ 0 ] += 16 * ( bestIdx % 3 − 1 )

dMvL0[ 1 ] += 16 * ( bestIdx / 3 − 1 )

- If tile_group_subpixel_refinement_enabled_flag is equal to 1

    - When subPelFlag is equal to 1, the parametric motion vector refinement process specified in clause 8.5.3.5 is invoked with the list sadList[ i ] with i = 0..8, and the delta motion vector dMvL0 as inputs and the modified dMvL0 as output.

- The delta motion vector dMvL1 is derived as follows:

dMvL1[ 0 ] = −dMvL0[ 0 ]

dMvL1[ 1 ] = −dMvL0[ 1 ]

# Experimental results

The simulation followed the CE8 test condition [1]. Since the proposed method only modifies the DMVR process, only random-access configuration is tested. IBC turned on and turned off are evaluated respectively. When IBC is turned on, results reportedly show that the proposed method has 0.00%, 0.01% and 0.00% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.03% and -0.01% BD rate changes.  When IBC is turned off, results reportedly show that the proposed method has 0.00%, 0.00% and 0.01% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.01% and -0.03% BD rate changes respectively. Table 1 and table 2 gives the detailed results when IBC is turned on and turned off respectively. Please note that the run time is not reliable due to the homogeneous cluster.

Table 1. RA Performance when IBC is turned on

|  | Over VTM4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 102% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 102% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 102% | 101% |
| Class D | 0.01% | 0.01% | 0.01% | 102% | 95% |
| Class F | -0.03% | -0.03% | -0.03% | 102% | 101% |
| TGM | -0.01% | -0.02% | -0.03% | 101% | 101% |

Table 2. RA Performance when IBC is turned off

|  | Over VTM4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.01% | 0.02% | 102% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 101% | 102% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 110% |
| Class F | -0.01% | -0.01% | -0.01% | 102% | 103% |
| TGM | -0.03% | 0.01% | 0.01% | 101% | 102% |

# Conclusions

This contribution proposes to adaptively disable fractional MVD refinement in DMVR. When IBC is turned on, simulation results reportedly show that the proposed method has 0.00%, 0.01% and 0.00% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.03% and -0.01% BD rate changes.  When IBC is turned off, simulation results reportedly show that the proposed method has 0.00%, 0.00% and 0.01% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.01% and -0.03% BD rate changes respectively. We recommend adopting this modification into the next version of VVC draft.

# References

- X. Xu, Y.-H Chao, Y.-C Sun and J. Xu, Description of Core Experiment 8 (CE8): Screen Content Coding Tools, JVET-M0128

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Jan. 2019

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).