| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0262 |
| --- | --- |

| Title: | CE9-related: Disabling DMVR for non equal weight BPWA |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to disable decoder-side motion vector refinement if the weights used in bi-prediction weighted average is not default weights. The Y, U, V BD-rate changes are 0.00%, 0.04%, 0.02% in RA.

# Introduction

In current VTM, two decoder side motion refinement tools. One is bi-directional optical flow (BDOF), the other is decoder-side motion vector refinement (DMVR). BDOF is disabled when bi-prediction weighted average (BPWA) does not applies default weights (1/2, 1/2). However, DMVR is enabled in the same condition. It’s proposed to further disable DMVR when BPWA is not using default weight.

The text is modified as follows.

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

# Test result

The test result under random access common test condition is shown in Table 1. The overall Y, U, V, BD-rate changes are 0.00%, 0.04%, 0.02%.

Table 1

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 99% | 99% |
| Class A2 | 0.02% | 0.10% | 0.10% | 98% | 101% |
| Class B | -0.01% | 0.03% | -0.01% | 101% | 101% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.09% | -0.08% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.08% | 101% | 100% |

# Conclusion

It’s suggested to disable DMVR when BPWA is not using default weight in the next version of VTM.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).